ELCT 302

Laboratory Notebook #3

Control Design and Implementation

Group #2B (Steering Control)

October 25th, 2023

I hereby certify that I have complied

with the Spirit and the Letter

of the Carolinian Creed

in preparing this report

Mady Hines

Mady Hines

Department of Electrical Engineering

University of South Carolina

Columbia, SC 29208

Steering Control (Group B)

General Objectives

❖ Design a closed-loop DC motor controller and determine necessary coefficient values

➢Proportional-derivative control (PD)

❖ Simulate open vs. closed-loop system continuous time response using MATLAB

❖ Determine system stability by its transfer function using the Nyquist stability criterion

➢ Simulate the discretized closed-loop system model using Simulink software; compare with continuous time MATLAB results

❖ Implementation of micro-controller based closed-loop DC motor control

❖ Testing and validation of the complete control system

❖ System Analysis

➢ Time Response

➢ Bode plot, gain margin, phase margin

➢ Nyquist plot and Nyquist criteria for stability

## Main Specifications

❖ Servo voltage: 4.8V - 6V

❖ Communication frequency: 50Hz

❖ Servo Rotational range: 180⁰

❖ Feedback: LC circuit for a given inductance (at 20 kHz)

## Tasks: Steering Control Design and Simulation

Performance Characteristics Descriptions:

- Rise Time: The time taken for the system to go from 10% to 90% of the step input value.

Simulink: (3.386, 0.08454)

- Overshoot: The amount by which the system exceeds the desired step input value.

Simulink: (6.822, 0.111)

- Undershoot: If applicable, the amount by which the system initially falls below the desired step input value.

Simulink: N/A

- Steady State Error: The difference between the final output and the desired step input value.

- Settling Time: The time taken for the system to enter and remain within a certain percentage (commonly 2% or 5%) of the step input value.

Simulink: (9.818, 0.1094)

|  | Rise Time | Overshoot | Undershoot | Steady State Error | Settling Time |
| --- | --- | --- | --- | --- | --- |
| System Measurement |  |  |  |  |  |
| MATLAB (continuous) | 2.4837 | 2.0741 | 3.9187 | 0 | 6.2629 |
| Simulink (discrete) | 3.386 | 1.11e-01 | N/A | 0.3 | 9.8178 |
| Percent Error: (Mat. VS Sim) | 36.33% | 94.70% | N/A | 100% | 56.76% |
| Percent Error: (Mat. VS Sys) |  |  |  |  |  |
| Percent Error (Sim. VS Sys) |  |  |  |  |  |

*MATLAB: The continuous system analysis.

*Simulink: The discrete system simulation.

*Real System

### MATLAB plots:

### Simulink Plots:

## Questions

1) What is the fundamental difference between the simulation in Matlab and the simulation in Simulink? 	a) Hint: Time domain

- MATLAB: continuous

- Simulink: discrete

2) Is the controller designed in the discrete time domain or continuous time domain? a) Hint: Have you been taught discrete control design?

- The controller is designed in continuous time domain.

3) Is the control implementation in the continuous domain? If not, how do we determine the discrete controller is stable in our operational range?

- The controller is implemented in discrete time domain.

4) Is there some way to limit the effect from a sudden change in the reference signal of the system?

a) Hint: Research Derivative Kick

- Yes, there is a way. The low pass filter would be the easiest to implement due to our knowledge level on the subject matter.

5) How did you determine the no load bandwidth and what frequencies can be rejected?

a) Does this match the input waveform performance?

- We determined the no lad bandwidth by generating a Bode Plot in MATLAB.

  - The bandwidth of the system is generally determined by thew –3 dB point of the gain for the system.

## Requirements

❖ Check-off requirements:

➢ Implantation of controller must use software interrupt (ticker function)

➢ Comparison for Simulations and Implementation

▪ Tabularized comparison chart completed

➢ Description of derivation for control gains

➢ Description of stability

▪ Bode Plot

▪ Nyquist stability criterion

➢ Completion and demonstration of control implementation

▪ You may push the car with a pole or similar object, trying to maintain a constant set speed.

❖ Check-off report: (Group)

➢ Refer to rubric and template from Blackboard

➢ Be sure to include:

▪ Results presentation (include o-scope images, measurement records, etc.)

▪ Results analysis and discussion

▪ Hardware schematic (using PSpice, TinyCAD, EAGLE, etc.)

▪ Problem-solving (describe problems you encountered and how you solved them)

**Battery should have a short circuit fuse.

- Ask Joe:

  - 10 A fuses

- Buy Lithium polymer charger