# Full System: Autonomous Electric Vehicle

- Project Overview

  - Description

    - Goal: Design and Build an Autonomous Vehicle

      - Navigation

        - Control the vehicles speed

        - Control the vehicles direction

      - Localization

        - Determine the relative location of the car on the track

        - Display location to user

      - State Logic

        - React appropriately according to the perceived environment or input

  - Subsystems: Hardware and software (listed out)

  - Main Specifications

    - Driving:

      - Motor voltage: 7.2V

      - Switching frequency: ≥20 kHz

      - Speed range: 0-100% unidirectional (Design to match the DC motor’s rated voltage!!!)

      - External voltage (speed) reference: 0-3.3V

      - Feedback: Incremental Encoder

      - Feedback: Hall Effect Sensor

      - Feedback: Reflective Object Sensor

    - Steering:

      - Servo voltage: 4.8V - 6V

      - Communication frequency: 50Hz

      - Servo Rotational range: 180⁰

      - Feedback: LC circuit for a given inductance (at 20 kHz)

  - Hardware Software Interface Diagram

  - System Power Diagram

  - Code Function Diagram

# Subsystems:

## Driving

- The drivetrain is crucial for converting and delivering the power needed for movement (Controls Speed), while the tachometer provides valuable data on the motor's operational status. The drivetrain is a key mechanical component, and the tachometer serves as an important monitoring tool (Speed Sensor).

- Buck Converter

  - Circuit

  - Description/Purpose: To generate voltage for motor; Output DC voltage less than its input DC voltage (step-down)

- Permanent Magnet DC (PMDC) Motor

  - Circuit

  - Description/Purpose: Type of direct current motor that uses permanent magnets to create the magnetic field, rather than relying on field windings.

- Tachometer (Frequency-to-Voltage Converter)

  - Circuit

  - Description/Purpose: measures the rotational speed of the engine or motor, typically in revolutions per minute (RPM).

- Microcontroller

- PI Control Design: Simulations

  - MATLAB: (continuous)

    - Closed-loop

    - Open-loop

  - Simulink: (discrete)

## Steering

- The voltage regulator ensures stable operation of electronic systems, the servo motor enables precise control of mechanical parts (controls direction), and the magnetic distance sensor provides critical data for navigation and safety mechanisms.

- Voltage Regulator LD1085V50 (for Servo Motor)

  - Circuit

  - Description/Purpose: maintaining a stable and consistent voltage level

- Servo Motor

  - Description/Purpose: a linear actuator that allows for precise control of linear position

  - Model:

    - Open-Loop Controller

    - Servo Transfer Function

    - Vehicle Dynamics

    - Distance Sensor Characterization

    - Brushed DC Motor

- Magnetic Distance Sensors

  - Circuit

  - Description/Purpose: detect magnetic fields or changes in magnetic fields; used to measure distances or proximity to objects

- Microcontroller (Arduino)

- PD Control Design: Simulations

  - MATLAB: (continuous)

    - Closed-loop

    - Open-loop

  - Simulink: (discrete)

## Subsystems Integration

- State Inputs and Outputs

  - Circuit

  - Buttons/Input Devices

    - RUN

    - WAIT

    - STOP

  - Indicators/LED

  - Faults

    - Off-track

    - Collision

    - Low Battery Voltage (LBV)

- PID Control

- Localization

KEY:

Hardware

Software

Lab 1

Lab 2

Lab 3

# Questions:

## Lab 1

### Driving

### Steering

### PowerPoint Extras