EMCH 290-001-EE#1                  Due at 11:59 on Thursday, September 13, 2022

Points Possible: 100

- Question 1(15 Points)

The drag force, Fd, imposed by the surrounding air on a vehicle moving with velocity V is given by Fd= Cd*A*ρ*V22  where Cd is a constant called the drag coefficient, A is the projected frontal area of the vehicle, and ρ is the air density.  An automobile is moving at V = 36 kilometers per hour with Cd = 0.3, A = 2 m2, and ρ = 1.2 kg/m3.

- Determine the force, in N, required to overcome aerodynamic drag.

- Determine the power, in J, required to overcome aerodynamic drag.

Fd= Cd*A*ρ*V22=0.3*2*1.2*36kmh*1000mkm*h3600s22=36 N

W=P=F*v=36*36kmh*1000mkm*h3600s=360J

- Question 2(18 Points)

Shown in the figure, an exterior brick wall has a thermal conductivity of 1.82 W/m2·oK. and a width of L = 0.15 m. At steady state, the temperature of the wall decreases linearly from T1 = 295oK on the inner surface to T2 on the outer surface. The outside ambient air temperature is T0 = 272oK and the convective heat transfer coefficient is 29 W/m2·oK.

- Determine the temperature T2 in oK

- Determine the rate of heat transfer through the wall, in W/m2.

Fourier Law: Qx=-kAdTdx=-kAT2-T1L

Newton Law: Qconv=hAT2-T0

Qx=Qconv

-kAT2-T1L=hAT2-T0

Solving for T2,

T2=hLT0+kT1hL+k=29*0.15*272+1.82*29529*0.15+1.82=278.8°K

Qx=-kAT2-T1L→QxA=-kT2-T1L

QxA=-1.82278.8-2950.15=196.56 Wm2

- Question 3(20 Points)

A composite plane wall consists of a Li = 50 mm thick layer of insulation (ki = 0.05 W/m·K) and a Ls = 25 mm thick layer of siding (ks = 0.10 W/m·K). The inner temperature of the insulation is 300oK. The outer temperature of the siding is 265oK. Steady state conditions apply.

- Determine the temperature at the interface between the two layers, in oK.

- Determine the rate of heat transfer through the wall, in W/m2.

QiA=-kiT2-T1Li

QsA=-ksT3-T2Ls

Qi=Qs

kiT2-T1Li=ksT3-T2Ls

T2=kiLsT1+ksLiT3kiLs+ksLi=0.05*0.025*300+0.1*0.05*2650.05*0.025+0.1*0.05=272°K

QiA=-kiT2-T1Li=-0.05272-3000.05=28Wm2

- Question 4(15 Points)

A gas is compressed in a piston–cylinder assembly from p1 = 2 bar to p2 = 8 bar, V2 = 0.02 m3 in a process during which the relation between pressure and volume is pV1.7 = constant. The mass of the gas is 0.2 kg. The specific internal energy of the gas increases by 50 kJ/kg during the process. Kinetic and potential energy changes are negligible. For the gas as the system:

- Determine the work for the process, in kJ.

- Determine the heat transfer, in kJ.

p1V11.7=p2V21.7

V1=1.7p2*V21.7p1=1.78*0.021.72=0.0452m3

W=8*0.02-2*0.04521-1.7*105Pabar*kJ103J=-9.94kJ

Q=ΔU+W=mΔu+W=0.250-9.94=0.06[kJ]

- Question 5(17 Points)

Steam in a piston–cylinder assembly undergoes a polytropic process, with n = 2, from an initial state where V1 = 0.075 m3, p1 = 3450 kPa, and u1 = 3075.9 kJ/kg to a final state where u2 = 2410 kJ/kg and v2 = 0.211 m3/kg. The mass of the steam is 0.5 kg. Changes in kinetic and potential energy can be neglected.

- Determine the change in volume, in m3.

- Determine the energy transfer by work, in kJ.

- Determine the energy transfer by heat, in kJ.

V2=mv2=0.5*0.211=0.1055m3

ΔV=V2-V1=0.1055-0.075=0.0305m3

p1V12=p2V22

p2=p1V12V22=3450000*0.07520.10552=1743559Pa=1743kPa

W=1743559*0.1055-3450000*0.0751-2=74804J=74.8kJ

KE+PE+U=Q-W

Q=ΔU+W=mΔu+W=-0.5*3075.9-2410+74.804=-258.146kJ

- Question 6(15 Points)

0.4[kg] of a gas is contained within a piston. The gas undergoes a process for which the pressure–volume relationship is P*V1.5=Cconstant. The initial pressure is 3[bar], the initial volume is 0.1m3, and the final volume is 0.2m3. The change in specific internal energy of the gas in the process is u2-u1=-55kJkg. Kinetic and potential energy are negligible. Determine the work and net heat transfer for the process[kJ].

W=-V1V2P*dV C=P1V1n=P2V2n

W=-V1V2ConstantVn*dV=-CV1-n1-nV1V2=-C2V21-n-C1V11-n1-n=-P2V2-P1V11-n

Convert Units: 3[bar]=300000[Pa], 1.06[bar]=106000[Pa]

W=-P2V2-P1V11-n=-106000*0.2-300000*0.11-1.5=-17600J=-17.6kJ

Q+W=mu2-u1+mv22-v122+mgZ2-Z1→Q+W=mu2-u1

Q+W=mu2-u1→Q=0.4-55--17.6=-4.4kJ