1. [25 pts] [Understanding of Coulomb’s Law, Basic Concept of Divergence or Gauss’s Law, Electrical Flux Density, and Electric potential] As shown below Figure, a point charge 𝑞 is at the origin in free space with permittivity, 𝜖0.

(a) [5 pts] Describe succinctly Gauss’s Law and write the equations with both volume integration and surface integration forms if the charge is induced by volume charge density, 𝜌𝑣.

Gauss’s Law states: “The total outward flux of the electric field intensity, E over any closed surface in free charge space is equal to the total charge enclosed in the surface divided by ϵ0”

vol ∇⋅E(ρv) dv=1ϵ0vol ρv dv→S E dS=Qϵ0=qϵ0

(b) [5 pts] Find electric field intensity, E considering Figure above. Write all the expansions to obtain E.

Q=Ψ=0π02πD*R2*sinθdθdϕ

Q=D*4πR2

D=Q4π*R2=q4π*R2aR

In Free Space, D=ϵ0E, so E=q4π*ϵ0*R2aR

(c) [5 pts] Find electrical flux density D.

D=Q4π*R2=q4π*R2aR

(d) [5 pts] If another charge 𝑞𝑎 is at the point A, which has a distance of 𝑅𝑎 from the origin with the direction of aR (𝑅𝑎 > 𝑅), what is the force acting on a charge 𝑞, arising from charge, 𝑞𝑎?

F=q*Ea

Ea=qa4π*ϵ0*Ra2aRa

F=q*qa4π*ϵ0*Ra2aRa=q*qa4π*ϵ0*Ra2aRa

(e) [5 pts] Find the electrical potential, 𝑉, of a distance 𝑅 from a charge Q referred to that at infinity.

V=-∞REdl

V=-∞Rq4πϵ0R2dR

V=--q4πϵ0R

V=q4πϵ0R

2. [15 pts] [Applications of Gauss’s Law to find D, E, and 𝑽]. Answer below questions considering “Gauss’s Law”.

(a) [10 pts] For a line charge, ρl =10-92[Cm], on the z-axis (in the free space), find D, EA, EB, and VAB where A is (2m,π2, 0) and B is (4m, π, 5m).

D=Q4π*R2=q4π*R2aR

(b) [5 pts] Given the field, E=-16r2arVm in spherical coordinates, find DB, and the electrical potential 𝑉 of point A 2m, π,π2 with respect to (4m, 0, π).

3. [20 pts] [Understanding of Dielectrics, the relationship between D, E, Polarization P, electrical susceptibility, relative (dielectric) permittivity, and absolute permittivity]

(a) [10 pts] Find the magnitudes of D, P, and ϵr for a dielectric material in which E=0.15 MV/m and χe= 4.25

(b) [10 pts] Determine the value of E in a material for which the electrical susceptibility is 3.5 and P= 2.3 × 10-7a [C/m2]

4. [20 pts] [Understanding how to find capacitance from different types of capacitors, and understanding of Multi-Dielectric Capacitors]

(a) [10 pts] A coaxial capacitor of length 𝐿, where the inner conductor has radius 𝑎 and the outer has radius 𝑏 as below Figure. The space between the inner and outer conductors with a dielectric permittivity, 𝜖. Find:

(i) Electrical Potential, Vba

(ii) Capacitance 𝐶

(iii) The stored energy in the electric field

(b) [10 pts] Find the capacitance of a parallel-plate capacitor containing two dielectrics, ϵr1=1.5 andϵr2=3.5, each comprising one-half the volume, as shown in the Figure below. Here, A=2m2 and d=10-3m. In addition, repeat if the two dielectrics each occupy one-half of the space between the plates but the interface is parallel to the plates.

5. [20 pts] [Electrostatic Energy or Stored Energy by Electrostatic Field]

(a) [10 pts] Given the potential function V=2x+4y V in free space, find the stored energy in a 1 m3 volume centered at the origin

(b) [10 pts] The electric field between two concentric cylindrical conductors at r=0.01m and r=0.05m is given by E=105rarVm, fringing neglected. Find the energy stored in a 0.5 m length. Assume free space.