CSCE 313 – Embedded Systems design

# Module 5: Image Rotation

Jacob Christopher Vaught

Madelyn Leire Hines

03/31/2023

Department of Computer Science

University of South Carolina

Columbia, SC 29208

# Introduction

CSCE 313 module 5 focuses on Image Rotation. The project involves designing an algorithm to rotate an image and display it on an Altera DE2-115 board using C code and the Intel Quartus platform. The goal of the project is to optimize the algorithm for speed and efficiency, while also minimizing computational burden.

This report discusses the optimization techniques used in the project and the resulting performance improvements achieved. The report is organized into three sections, each detailing a different iteration of the algorithm, the modifications made, and the performance improvements achieved. In particular, the report focuses on the pre-rendering technique introduced in the third iteration of the algorithm and its impact on performance.

The report concludes by highlighting the importance of optimization techniques in embedded systems design and the benefits they can bring in terms of speed and efficiency.

# System Elements

After Compiling the system without the timer, the system specs are as follows:

| Logical Elements | Registers | Total Pins | Memory Bits |
| --- | --- | --- | --- |
| 3299/114480 | 1969 | 52/529 | 604672/3981312 |

After compiling with the timer, the system specs are as follows:

| Logical Elements | Registers | Total Pins | Memory Bits |
| --- | --- | --- | --- |
| 5943/114480 | 4067 | 134/529 | 167936/3981312 |

The results between Table 1 and 2 differ in a few ways. First the number of logical elements increases, which could be attributed to the addition of the timer component. In addition, the number of registers increases, which can also be attributed to the additional component.

# Code Optimization and FPs Calculations

After the initial  attempt at the project, a TPS of 10000 was set. The duration to draw each frame was as follows: 17 seconds for 0 degree screen, 60 seconds for 90 degrees screen, 77 seconds for 180 degrees screen, 77 seconds for 270 degrees screen. This results in an average FPS of 417+60+77+77=0.017 FPS.

## ATTEMPT 2

The second iteration of the algorithm focused on enhancing its speed by optimizing a select few items. The main bottleneck in the previous iteration was the need to calculate the cosine and sine of theta 320*240 times per screen drawing. These calculations are computationally expensive and time-consuming. To address this, we introduced two new global variables, COStheta and SINtheta, which were calculated only once and used in multiplication. This is a more efficient approach than calculating them repeatedly 75000+ times. Furthermore, we avoided division operations wherever possible and replaced them with variables like halfCOLS, resulting in a slight improvement in performance.

Additionally, we identified and removed any needless mathematical computations, either by replacing them with simpler mathematical operations or by eliminating them altogether. Wherever possible, we replaced multiplication operations with addition since the latter is faster. As a result of these modifications, the final version of the algorithm achieved significantly better performance than its predecessor.

The performance of the optimized algorithm was evaluated by measuring the time taken to render screens at different angles. The results showed that the algorithm could render a 0-degree screen in 14 seconds, a 90-degree screen in 15 seconds, a 180-degree screen in 15 seconds, and a 270-degree screen in 15 seconds. The average frames per second (FPS) achieved by the algorithm was calculated to be 414+15+15+15=0.0677 FPS, which is four times faster than the initial version.

## Attempt 3

In this attempt, the algorithm was further optimized using a pre-rendering technique. This technique involves storing output pixels in an array the first time they are drawn, which can be recalled from memory in future renderings, resulting in significant time savings. The initial start time for frame rendering was similar to that of Attempt 2, with 14-15 seconds per frame render. However, all subsequent renderings occurred much faster due to the pre-rendering technique.

This technique reduces the computational burden of the algorithm by allowing the program to recall the frames from memory, eliminating the need to redraw the frames from scratch every time they are needed. As a result, the final speed of the algorithm was significantly improved, with an average FPS of 40.1+0.1+0.1+0.11=9.75 FPS. This improvement represents a remarkable 145x improvement over the previous model and an astounding 570+x improvement over the first algorithm.

The pre-rendering technique optimizes the algorithm by reducing the number of calculations needed to render the image on the screen. When a frame is initially drawn, all of the output pixels are stored in an array for future use. Subsequent renderings of the same frame can then be recalled from memory, saving the program from having to perform the same calculations repeatedly. The pre-rendering technique effectively reduces the computational burden on the algorithm by allowing it to store and recall pixel data, which can be used to draw the image on the screen in subsequent renderings.

Overall, the pre-rendering technique significantly improves the performance of the algorithm by reducing the computational burden and time required to render the image on the screen. By reducing the number of calculations needed to render each frame, the algorithm achieves a significant improvement in speed and efficiency.