Understanding Error Correction and the Golay 24 12 8 Encoding Methodology

Fundamental to the problem is the manner in which we communicate a message or an approximation across some distance. Communication systems strive to relay information accurately despite the unavoidable presence of noise and other distortive elements. Addressing this challenge, the mid-1900s marked significant advancements with several individuals pioneering the development of error-correcting codes.

From this pioneering work, we derive the Golay 24 12 code, which encodes 12 data bits into a ‘codeword’ of 24 bits. This coding scheme is notable not only for its capacity to encode information efficiently but also for its robust error correction capabilities.

Error Correction in the Golay 24 12 Code

When encoding a message using the Golay code, we utilize a generator matrix, 𝐺G. This matrix is structured to transform 12 bits of original data into a 24-bit codeword. The structure of 𝐺G typically takes the form:

G=I12​P

where I12​ is a 12x12 identity matrix, and P is a 12x12 matrix that determines the parity bits added to the data bits for error detection and correction purposes. This formulation allows the Golay code to not only encode the data but also to integrate substantial redundancy that facilitates error detection and correction.

Encoding Process

- Data Input: A 12-bit binary vector m (the message) is prepared for encoding.

- Codeword Generation: The codeword c is generated by the equation c=mG. Here, matrix multiplication is performed over the binary field GF(2), meaning all arithmetic operations are modulo 2.

- Transmission: The resulting 24-bit vector c is sent over the communication channel.

EXAMPLE: [Keep in mind that all operations need to be done in binary when implementing computationally] Consider the message 𝑚 represented as 0111 1011 0101. When encoded using the traditional Golay 24,12 methodology, the resulting 24-bit codeword might appear as 0111 1011 0101 1110 1011 1001. Notice that the original 12-bit message is preserved in the first half of the codeword, which simplifies the process of message extraction. This encoding ensures that any errors introduced during transmission can be detected and corrected, all the while maintaining the integrity of the original data.

G = [1 0 0 0 0 0 0 0 0 0 0 0 1 0 1 0 0 0 1 1 1 0 1 1

0 1 0 0 0 0 0 0 0 0 0 0 1 1 0 1 0 0 0 1 1 1 0 1

0 0 1 0 0 0 0 0 0 0 0 0 0 1 1 0 1 0 0 0 1 1 1 1

0 0 0 1 0 0 0 0 0 0 0 0 1 0 1 1 0 1 0 0 0 1 1 1

0 0 0 0 1 0 0 0 0 0 0 0 1 1 0 1 1 0 1 0 0 0 1 1

0 0 0 0 0 1 0 0 0 0 0 0 1 1 1 0 1 1 0 1 0 0 0 1

0 0 0 0 0 0 1 0 0 0 0 0 0 1 1 1 0 1 1 0 1 0 0 1

0 0 0 0 0 0 0 1 0 0 0 0 0 0 1 1 1 0 1 1 0 1 0 1

0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 1 1 1 0 1 1 0 1 1

0 0 0 0 0 0 0 0 0 1 0 0 1 0 0 0 1 1 1 0 1 1 0 1

0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 0 0 1 1 1 0 1 1 1

0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 0];

Error Detection and Correction

Upon receipt of a potentially corrupted codeword r, the system must assess and correct any errors:

- Syndrome Calculation: The syndrome s is calculated using the parity-check matrix H, via s=rHT. If s equals zero, the codeword is presumed error-free. A non-zero syndrome indicates errors. The parity-check matrix H=PTI12.

- Syndrome Decoding: Using a precomputed lookup table based on H, the system identifies the error pattern corresponding to the observed syndrome.

- Error Correction: The error vector 𝑒, identified by the syndrome, is used to correct the codeword by calculating r'=r⊕e, where r′ is the corrected codeword.

EXAMPLE: Calculating H from the generator vector (G) above, we get:

H = [1 1 0 1 1 1 0 0 0 1 0 1 1 0 0 0 0 0 0 0 0 0 0 0

0 1 1 0 1 1 1 0 0 0 1 1 0 1 0 0 0 0 0 0 0 0 0 0

1 0 1 1 0 1 1 1 0 0 0 1 0 0 1 0 0 0 0 0 0 0 0 0

0 1 0 1 1 0 1 1 1 0 0 1 0 0 0 1 0 0 0 0 0 0 0 0

0 0 1 0 1 1 0 1 1 1 0 1 0 0 0 0 1 0 0 0 0 0 0 0

0 0 0 1 0 1 1 0 1 1 1 1 0 0 0 0 0 1 0 0 0 0 0 0

1 0 0 0 1 0 1 1 0 1 1 1 0 0 0 0 0 0 1 0 0 0 0 0

1 1 0 0 0 1 0 1 1 0 1 1 0 0 0 0 0 0 0 1 0 0 0 0

1 1 1 0 0 0 1 0 1 1 0 1 0 0 0 0 0 0 0 0 1 0 0 0

0 1 1 1 0 0 0 1 0 1 1 1 0 0 0 0 0 0 0 0 0 1 0 0

1 0 1 1 1 0 0 0 1 0 1 1 0 0 0 0 0 0 0 0 0 0 1 0

1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 1];

To demonstrate the role of 𝐻 in error detection, consider a transmitted codeword [0111 1011 0101 1110 1011 1001]. The syndrome is calculated as follows:

- When error-free, the syndrome calculated using 𝐻H is [0000 0000 0000], indicating no errors.

- If a single bit error occurs, altering the codeword to [0111 1011 0101 1110 1011 1011], the resulting syndrome is [0000 0000 0010].

By systematically flipping each bit of a known codeword and computing the resulting syndromes, we can create a lookup table. This table maps each possible syndrome to its corresponding error pattern. Using this table, any received codeword with errors can be decoded by finding the error pattern associated with its syndrome and correcting the respective bits.

Extraction of the Original Message

After correcting any errors, the original message is extracted from the corrected codeword. Given the structure of G=I12​P the first 12 bits of r′ represent the original message. This direct mapping from the corrected codeword to the original message simplifies the decoding process and ensures the reliability of communication.