Test 1

Assignment

- G signifies students that take this class for Graduate Credit; they must do all the items

- UG students are not required to do the G work, but they may do it for bonus points

- Problems solved beyond the minimum requirements will be given bonus points.

- Challenge problems are optional; they will earn bonus points.

NOTES:

- Show your work to obtain partial credit if appropriate

- Always display input data!

- Partial credit might be given based on work done and MATLAB codes if they are attached; without them, no partial credit can be given.

- Display numerical results to three significant digits (four if the first digit is 1).

- Use SI system of measurement
 https://en.wikipedia.org/wiki/International_System_of_Units

- Scale units using appropriate prefixes (e.g., nano, micro, milli, kilo, mega, giga…) 
https://en.wikipedia.org/wiki/Metric_prefix

- Problems solved beyond the minimum requirements will be given bonus points.

- Challenge problems are optional. They may be solved for extra credit

- Normalized plots are plots with the y-axis scaled between 
–1 and +1. A plotted variable is normalized by dividing it by its maximum absolute value

Notations and abbreviations:

- BVP = boundary value problem

- EOM = equation of motion

- FBD = free body diagram

- FRF = frequency response function

- IVP = initial value problem

- LHS = left hand side

- NLM = Newton’s law of motion

- ODE = ordinary differential equation

- PDE = partial differential equation

- RHS = right hand side

TABLE OF CONTENTS

A.	Pendulum	4

A.1	Analyze pendulum	4

A.2	Calculate pendulum response to initial displacement	5

A.3	Calculate pendulum response to impact	7

B.	Spring-mass-damper vibrating systems	8

B.1	Analyze spring-mass vibrating system	8

B.2	Analyze spring-mass-damper vibrating system	9

B.3	Calculate spring-mass-damper response to initial displacement	10

B.4	Calculate truck suspension response	11

B.5	Overdamped spring-mass-damper systems	13

B.6	Analyze spring-mass-damper forced vibration	14

B.7	Calculate flex-beam response to slightly unbalanced electric motor excitation	15

B.8	Graduate work: calculate horizontal flex-beam response to impact	17

C.	Extra Credit Challenge Problems	19

C.1	Analyze vertical spring-mass vibration system	19

C.2	Analyze vertical flex-beam vibration	20

C.3	Analyze horizontal flex-beam vibration	21

C.4	Calculate horizontal flex-beam response to initial displacement	22

C.5	Analyze spring-mass forced vibration	23

C.6	Calculate cantilever response to suddenly applied weight	24

C.7	Analyze flex-beam pendulum	25

C.8	Calculate flex-beam pendulum response to impact	26

C.9	Complex exponentials and trigonometric functions	28

# Pendulum

## Analyze pendulum

NOTHING HERE

## Calculate pendulum response to initial displacement

Figure 1: pendulum of length L and mass m

Take g=9.81 m/s2 as the generally accepted value of gravitational acceleration. Consider a pendulum of nominal length Lnominal=500mm and mass m=45kg as shown in Figure 1. When manufactured in series production, the pendulum length values are not exact due to manufacturing tolerance. The mm length values for ten pendulum sets are:

L  =  [502.013, 499.337, 500.472, 502.949, 502.216, 496.635, 500.416, 498.255, 498.350, 499.358] mm

Do the following:

- For the ten pendulum sets, calculate

  - rad/sec natural frequency wn

  - Hz natural frequency fn

  - period of oscillation t

  - display in table format L, wn, fn, t

- Mean values: calculate and display the mean values of wn, fn, t

- Initial value problem (IVP): assume initial displacement u0=4mm and zero initial velocity. Use the mean values of wn, fn, t to do the following:

  - Calculate and display the position of the mass u(t1) after t1=18 sec

  - Calculate and display the duration tNosc of Nosc oscillations and the corresponding position of the mass uNosc for Nosc=15

  - Plot the response of the system for Nosc oscillations and use a datatip on the plot to find the value u(t1) at t1 on the plot

- Discuss your results

## Calculate pendulum response to impact

Figure 2: impact excitation of a pendulum system

Take g=9.81 m/s2 as the generally accepted value of gravitational acceleration. Consider a pendulum of nominal length L=500 mm and mass M=45kg as shown in Figure 2. The pendulum is initially at rest hanging vertically. A small projectile of mass m=11 grams and velocity v=850 m/s hits the mass M and remains embedded into it. This impact imparts an initial velocity to the system that starts to oscillate. Do the following:

- display input data

- Calculate the initial velocity of the system  as a result of the interaction between the projectile and the mass.

- Calculate the rad/sec natural frequency wn and period of oscillation t of the pendulum with projectile embedded into it

- Calculate the response u(t1) after t1=18 sec

- Plot the response of the system for Nosc=15 oscillations and find on the plot:

  - the largest response

  - the response value calculated at item (4)

- Discuss your results

# Spring-mass-damper vibrating systems

## Analyze spring-mass vibrating system

NOTHING HERE

## Analyze spring-mass-damper vibrating system

NOTHING HERE

## Calculate spring-mass-damper response to initial displacement

Figure 3: spring-mass-damper vibrating system

Consider a spring-mass vibrating system with m=5.3 kg, k=530 N/m, c=22 N/(m/s) as shown in Figure 3. The vibrating system is given an initial displacement u0=4 mm with zero initial velocity. Do the following:

- Display input data

- Calculate the natural frequency in rad/sec wn and in Hz fn

- Calculate the critical damping ccr

- Calculate the damping ratio z

- Calculate the damped frequency in rad/sec wd and in Hz fd

- Calculate the damped period of oscillation td

- Calculate the characteristic-equation roots s1, s2 and the constants C1, C2

- Calculate the constants A, B and then the constants C, j

- Calculate the duration t1 of one oscillation and the corresponding position u1=u(t1)

- Calculate the value u0.02 representing 2% of the initial displacement

- Plot the response of the system for Nosc=12 oscillations and use datatips on the plot to find the following:

  - the largest response

  - the response value calculated at item (9)

  - the time when the response magnitude has decreased permanently below u0.02 calculated at item (10)

- Discuss your results

## Calculate truck suspension response

Consider a pickup truck wheel suspension consisting of a leaf spring and an oil damper. The truck and its suspension can be modeled, in first approximation, as a k-m-c spring-mass-damper system as shown in Figure 4.

Figure 4	Truck suspension consisting of a leaf spring and an oil damper

The leaf spring can be approximated by a uniform beam of length L=1200 mm and cross-section dimensions h=4mm, b=40mm made of steel with E=200GPa. The truck mass apportioned to the wheel can be estimated as m=520kg. Assume initial displacement u0=105mm and zero initial velocity . Do the following:

- Display input data

- Calculate the second moment of area I of the beam cross-section

- Calculate the flexural stiffness EI of the beam

- Calculate the spring constant k of the system

- Calculate the natural frequency in rad/sec wn and in Hz fn

- Calculate the critical damping ccr

- Assume 25% damping ratio, i.e., z=0.25, and calculate the physical damping c. Print z and c

- Calculate the damped frequency in rad/sec wd and in Hz fd

- Calculate the damped period of oscillation td

- Calculate the characteristic-equation roots s1, s2 and the constants C1, C2

- Calculate the constants A, B and then the constants C, j

- Calculate the duration t1 of one oscillation and the corresponding position u1=u(t1)

- Calculate the value u0.02 representing 2% of the initial displacement

- Plot the response of the system for Nosc=17 oscillations and use datatips on the plot to find the following:

  - the largest response

  - the response value calculated at item (12)

  - the time when the response magnitude has decreased permanently below u0.02 calculated at item (13)

- Discuss your results

- Graduate work: assume the damper is worn out such that z is reduced to z=10%.

  - Repeat items (6) through (15)

  - Compare and discuss the two results

## Overdamped spring-mass-damper systems

Consider a spring-mass-damper system with natural frequency fn=5Hz and various values of the damping ratio z. The system is given an initial displacement u0 = 3 mm with zero initial velocity. Do the following:

- display input data

- calculate the rad/s frequency wn

- Assume overdamped conditions z > 1 and develop the solution as follows:

  - write the expressions of the roots s1, s2 in terms of wn and z

  - write the solution u(t) in terms of s1, s2 and constants C1, C2

  - write the expression of the constants C1, C2 in terms of initial displacement u0 and initial velocity

  - take z=2.5 and calculate the numerical values of s1, s2 and C1, C2

  - plot the response u(t) for t=0,…,3 seconds

- Assume critically damped conditions z =1 and do the following:

  - write the expressions of the two partial solutions u1(t), u2(t) in terms of wn

  - write the expression of the complete solution in terms of wn and C1, C2

  - write the expressions of the constants C1, C2 in terms of initial displacement u0 and initial velocity

  - calculate the numerical values of C1, C2

  - plot the response u(t) for t=0,…,3 seconds

- Graduate Work: nothing here

- Graduate Work: some race-car manufacturers design for critical damping whereas others use overdamped design. Discuss the pros and cons of the two options

## Analyze spring-mass-damper forced vibration

NOTHING HERE

## Calculate flex-beam response to slightly unbalanced electric motor excitation

Figure 5	damped cantilever beam excited by force F(t) from slightly unbalanced electric motor

Consider a cantilever beam of length L=520 mm, elastic modulus E=200 GPa, cross-section properties h=4 mm, b=30 mm and damping ratio z=15%. (The damping is achieved through a proprietary damping treatment applied on the beam surface.) An electric motor of mass m=0.7 kg operating at 3,100 RPM is attached at the beam tip as shown in Figure 5. The electric motor is slightly unbalanced producing a slight vibratory excitation  with F=12 mN. The excitation frequency w depends on the electric motor RPM. Do the following:

- Display input data

- Calculate

  - cross-section second moment of area I, flexural stiffness EI, and spring constant k

  - natural frequency wn rad/s, critical damping ccr, physical damping c

  - quasi-static deflection uqst corresponding to wn→0

- Calculate the values of

  - resonant frequency wr and damped frequency wd

  - peak magnification Mr

  - magnification at phase resonance M90

  - physical response magnitude || at phase resonance

  - calculate the Hz frequencies fr, fn and the corresponding RPM values

- assume excitation at phase resonance:

  - calculate the excitation frequency w in rad/s and f in Hz

  - calculate period of oscillation t

  - Plot the time response for Nosc=13 oscillations. Hint: plot the real part of the complex displacement u(t)

- Graduate work:

  - plot the magnitude and phase of the frequency response function FRF vs. excitation f Hz from quasi-static to 12 times the phase resonance frequency

  - calculate the FRF magnitude at phase-resonance RPM and at double the phase resonance RPM

  - calculate the vibration magnitude when the electric motor runs at its operational RPM

  - comment on what vibration would be observed as the motor is started and accelerated to reach its operational RPM

## Graduate work: calculate
horizontal flex-beam response to impact

Figure 6: impact excitation of a horizontal flex-beam vibration system

Consider a cantilever beam of length L=525 mm with a mass M=62kg attached at its tip as shown in Figure 6. The beam material is steel with E=200 GPa. The beam cross-section dimensions are h=4 mm, b=30 mm. The beam is initially at rest.

A small projectile of mass m=13 grams and velocity v=1020 m/s hits the mass and remains embedded in it. This impact imparts an initial velocity to the system that starts to oscillate with motion u(t) contained in the horizontal plane. Do the following:

- Display input data

- Calculate the second moment of area I for flexing consistent with the direction of motion u(t)

- Calculate the cross-sectional stiffness EI of the beam

- Calculate the flex-beam spring constant k

- Calculate the initial velocity of the system  as a result of the interaction between the projectile and the mass

- Calculate the rad/sec natural frequency wn and period of oscillation t

- Calculate the position of the mass u(t1) after t1=17 sec

- Plot the response of the system for Nosc=13 oscillations and find on the plot:

  - the largest response

  - the response calculated at item (7)

- Discuss your results

# Extra Credit Challenge Problems

Challenge problems are optional.

## Analyze vertical spring-mass vibration system

NOTHING HERE

## Analyze vertical flex-beam vibration

NOTHING HERE

## Analyze horizontal flex-beam vibration

NOTHING HERE

## Calculate horizontal flex-beam response to initial displacement

Figure 7: flex-beam vibrating system

Consider a cantilever beam of length L=525 mm with a mass m=62kg attached at its tip as shown in Figure 7. The beam material is steel with E=200 GPa. The beam cross-section dimensions are h=4 mm, b=30 mm. The tip mass is given an initial displacement u0=3mm with zero initial velocity. Do the following:

- Display input data

- Calculate the second moment of area I for flexing consistent with the direction of motion u(t)

- Calculate the cross-sectional stiffness EI of the beam

- Calculate the spring constant k of the system

- Calculate the natural frequency in rad/sec wn

- Calculate the natural frequency in Hz fn

- Calculate the period of oscillation in sec t

- Calculate the position of the mass u(t1) after t1=18 sec

- Plot the response of the system for Nosc=13 oscillations and use a data tip on the plot to find the value u(t1) at t1 on the plot

- Discuss your results

## Analyze spring-mass forced vibration

NOTHING HERE

## Calculate cantilever response to suddenly applied weight

Figure 8: cantilever beam with tip mass

Take g=9.81 m/s2 as the generally accepted value of gravitational acceleration. Consider a cantilever beam of length L=525 mm and a mass m=3 kg that has to be attached to its tip as shown in Figure 8. The beam is made of steel with elastic modulus E=200 GPa and yield stress Y=710 MPa cross-section properties h=4 mm, b=30mm. Do the following:

- Display input data

- Calculate the cross-section second moment of area I, the flexural stiffness EI, the spring constant k, the natural frequency wn rad/s, fn Hz, and period of oscillation t of the system

- Calculate the weight F0 of the mass m

- Static case: assume the mass is attached onto the undeformed beam and the let go gradually without inducing any vibration and calculate the static displacement ust

- Dynamic case: assume the mass is attached onto the undeformed beam and then let go suddenly inducing a state of vibration:

  - calculate the maximum dynamic displacement udyn

  - Plot the response u(t) measured from the undeformed position for Nosc=13 oscillations

  - Put a datatip on the plot to identify the maximum displacement and compare with result of item (5)(a)

- Graduate work: calculate the maximum stress at the root of the beam and the safety factor SF=Y/s and comment on your results for the above two cases:

  - sst for the static case

  - sdyn for the dynamic case

- Discuss your results

## Analyze flex-beam pendulum

NOTHING HERE

## Calculate flex-beam pendulum response to impact

Figure 9: impact excitation of a vertical flex-beam pendulum

Take g=9.81 m/s2 as the generally accepted value of gravitational acceleration. Consider a vertical flex-beam pendulum of length L=440 mm with a mass M=62 kg attached at its end as shown in Figure 9. The beam material is steel with E=200 GPa. The beam cross-section dimensions are h=4 mm, b=30 mm. The beam is initially at rest.

A projectile of mass m=11 grams and velocity v=1020 m/s hits the mass and remains embedded into it. The azimuthal orientation of the projectile motion is such that the beam flexes about its minor cross-sectional stiffness. This impact imparts an initial velocity to the system that starts to oscillate. Do the following:

- Display input data

- Calculate the minor second moment of area I of the beam

- Calculate the minor cross-sectional stiffness EI

- Calculate the flex-beam spring constant k

- Calculate the initial velocity of the system  as resulting from the interaction between the projectile and the large mass

- Calculate the rad/s natural frequency wnFBP and period of oscillation tFBP of the flex-beam pendulum with projectile embedded into it

- Calculate the response u(t1) after t1=18 sec

- Plot the response of the system for Nosc=23 oscillations and find on the plot:

  - the largest response

  - the response value calculated at item (7)

- Discuss your results in comparison with results of Problem A.3

## Complex exponentials and trigonometric functions

NOTHING HERE