TITLE: Human-Supervised CFAR Autotuning for UAS Maritime Sensing

Authors: Nathan Kirk, J.C. Vaught, Douglas Cahl, Yi Wang

Department of Mechanical Engineering, University of South Carolina, Columbia, SC 29028

Abstract: Maritime domain awareness, search-and-rescue, and coastal monitoring requires robust on-board radar detection under tight size, weight, power, and cost constraints. Tuning Constant False Alarm Rate (CFAR) detectors for littoral scenes is notoriously difficult due to heavy-tailed sea clutter, land/sea transitions, and shoreline multipath, yet remains essential for low-latency situational awareness. We present a human-supervised methodology for tuning CFAR parameters of COTS X-band radar in coastal scenes with stationary or persistent objects. A practitioner provides bounding boxes around reference objects (e.g., navigation aids, towers, shoreline structures) on one or more frames. Then, the system (i) propagates those regions across a frame sequence, (ii) sweeps the parameter spaces of multiple CFAR variants (cell-averaging, order-statistic, censored, and others), and (iii) solves a multi-objective optimization maximizing true detections inside annotated regions while minimizing false alarms elsewhere under an explicit PFA constraint. The outcome is a scene- and sensor-tailored algorithm/parameter configuration. Performance is evaluated on synthetic heavy-tailed sea-clutter (K/Pareto) and real X-band radar sequences to characterize the detection–false-alarm trade space under explicit PFA constraints, and to illustrate how the resulting scene-tailored configuration differs from default single-algorithm tuning.