EMCH 310 Homework 1 Due September 1, 2021 by 5: 00pm

1) The position of a particle along a straight line is given by s = (1.5t^3 - 13.5t^2 + 22.5t) ft, where t is in seconds. Determine the position of the particle when t=6s and the total distance it travels during the 6-s time interval.

Position = 1.5*6^3-13.5*6^2+22.5*6=-27

At  t=0, s=0. Velocity equation =4.5t^2 - 27t+ 22.5.

When is velocity = 0?

4.5t^2 - 27t+ 22.5=0→ t=1, 5

Find Positions at 0 velocity

1.5*5^3-13.5*5^2+22.5*5 =-37.5

1.5*1^3-13.5*1^2+22.5*1 =10.5

0-(-37.5)+10.5-(-37.5)+10.5-(-27)=123 ft

2) The velocity of a particle traveling along a straight line is v = (3t^2 - 6t) ft /s , where t is in seconds. If s= 4 ft when t=0, determine the position of the particle when t= 4 s. What is the total distance traveled during the time interval t = 0 to t = 4 s? Also, what is the acceleration when t = 2 s?

3t2-6t=t3-3t2+C=s

4=t3-3t2+C, when t=0, so 4=C final position equation→s=t3-3t2+4

s=43-3*42+4=116ft

When is velocity = 0?

3t2-6t =0→ t=0, 2

Find Positions at 0 velocity (just t=2)

s=23-3*22+4=0

Total Distance = 4+116=120ft

Acceleration = derivative of 3t^2-6t =6t-6

6*2-6=6ft/s^2

3) The acceleration of a particle traveling along a straight line is a =14 s12   ms2 , where s is in meters. If v = 0, s = 1 m when t = 0, determine the particle’s velocity at s = 2 m.

A=dv/dt, v=ds/dt, ads=vdv

1sads=0vvdv→1s0.25s0.5ds=0vvdv→ s1.56-16=v22→v=1s1.5-13→v=1*21.5-13=21.5-13=0.780689251

4) A car with an initial velocity of 30 m/s accelerates in a straight line at a constant rate of 12 m/s2 . Find the time required for the car to reach a velocity of 60 m/s and the distance traveled during this time.

V=30, T=0, S=0

V=12*0+c=30

C=30

V=12t+30

S=6t^2+30t

Time(t) → 60=12t+30=30/12=2.5 seconds

Distance(s)= 6*2.5^2+30*2.5=112.5 m