EMCH 290-001-EE#3                  Due at 11:59 on Tuesday, October 4, 2022

Points Possible: 100

- Question 1(25 Points)

A rigid, well-insulated tank contains a two-phase mixture of ammonia with 0.005 m3 of saturated liquid and 1.5 m3 of saturated vapor, initially at p1 = 5 bar.  A paddle wheel stirs the mixture until only saturated vapor at higher pressure, p2, remains in the tank. Kinetic and potential energy effects are negligible.

- Determine p2, in bar.

- Determine the amount of energy transfer by work, in kJ.

- Question 2(25 Points)

A piston–cylinder assembly contains 18 kg of ammonia, initially at 100 kPa and -10°C. The ammonia is compressed to a pressure of 550 kPa. During the process, the pressure and specific volume are related by pv1.1 = constant.

- Determine the work, in kJ.

- Determine the heat transfer, in kJ.

- Question 3(25 Points)

A piston–cylinder assembly contains 0.25 kg of air initially at a pressure of 2 bar and a temperature of 200oC. The air is heated at constant pressure until its volume is doubled. Assume the ideal gas model with constant specific heat ratio, k = 1.4.

- Determine Initial Volume.

- Determine the work, in kJ

- Determine the heat transfer, in kJ

- Question 4(25 Points)

Air is compressed in a piston–cylinder assembly from p1 = 0.7 bar, T1 = 275 K, V1 = 0.25 m3 to a final volume of V2 = 0.03 m3 in a process described by pv1.25 = constant.  Assume ideal gas behavior and neglect kinetic and potential energy effects. Using constant specific heats evaluated at T1, determine the work and the heat transfer, in kJ.

- Determine the work, in kJ

- Determine the heat transfer, in kJ