Homework 01

EIGEN ANALYSIS OF FLUTTER

A.	Aeroelastic Model	3

B.	Ground Vibration Test (GVT) Analysis	10

C.	Flutter Analysis	13

D.	Damped GVT and Flutter	23

E.	Extra Credit Challenge Problems	30

List of MATLAB codes	Error! Bookmark not defined.

Consider an airfoil in wind tunnel testing with airspeed U (Figure 1). The airfoil is supported by springs at P. The center of mass is at C. The distance from P to C is the static offset xCP. The aerodynamic lift L acts at Q. The distance from P to Q is the aerodynamic offset xQP. The airfoil undergoes oscillatory motion with plunge displacement h(t) and pitch displacement q(t). The positive plunge direction is downward. The positive pitch direction is “nose up” (clockwise).

Figure 1: Airfoil in wind tunnel testing

Input data

Air density: =1.225 kg/m3

Chord: c=0.4 m

Mass, m=2.8 kg

Mass moment of inertia, I0=0.05 kg.m2

Plunge frequency, fh=1.6 Hz

Pitch frequency, f=4.8 Hz

Static offset, xCP= –10% of c

Aerodynamic offset, xQP= 30% of c

NOTES:

- Always display input data!

- Show work and relevant comments

- Partial credit might be given based on work done and MATLAB codes if they are attached; without them, no partial credit can be given.

- Display numerical results to three significant digits (four if the first digit is 1).

- Problems solved beyond the minimum requirements will be given bonus points.

- Challenge problems are optional. They may be solved for extra credit

Notations:

- EOM = equation of motion

- CL = closed loop

- FBD = free body diagram

- FB = feedback

- GVT = ground vibration test

- LHS = left hand side

- LT = Laplace transform

- MIMO = multi-input multi-output

- ODE = ordinary differential equation

- PDE = partial differential equation

- RHS = right hand side

- SISO = single-input single-output

- SS = state space

- TF = transfer function

# Aeroelastic Model

For the airfoil shown in Figure 1, do the following:

- Deduce equations of motion. Clearly state your assumptions.

- Make linearity assumption on lift L and express L(U,q)=L0(U)q, where U is the airspeed

- Express equations of motion in terms of the plunge and pitch frequencies wh, wq

- Assume ,  and cast equations of motion in matrix form as a polynomial eigenvalue problem 
 
where 
,     M=MS+MA,       K(U)=KS+KA(U)

### Solution:

#### Part a:

- Deduce equations of motion. Clearly state your assumptions

From the model shown in figure one some important vocabulary to note is listed in the below table.

| Vocab | Definition |
| --- | --- |
| C | Center of Mass |
| P | Location of Elastic Springs (Elastic center) |
| Kh | Translational Spring |
| Kθ | Rotational Spring |
| Q | Aerodynamic Center |
| L(U) | Lift, Varies with airspeed |
| LE | Leading Edge |
| TE | Trailing Edge |
| h | Plunge (Vertical Displacement of COM) |
| θ | Pitch (Rotation about the elastic axis) |

Assumptions:

- System only has 2 DOF

- Rigid Body

- Symmetric Body

The equation of motion for this figure can be deduced from Newton’s second law of motion about the center of mass.

mu

C

I0θ

Free body analysis about c (Center of Mass):

F=mu→Fx=muxFy=muyMc=I0θ

Free body analysis about P (arbitrary):

P

d

C

mu

I0θ

F=mu→Fx=muxFy=muyMc=I0θ+mud

P

d

C

MC

FR=F

Vector:

FR=mu,   Mc=I0θ

Scalar

FR=mu,    u=u

MP=MC+Fd=I0θ+mud

L

Kθθ

Forces:

C

P

Q

h

Khh

θ

Accelerations:

C

P

uy=h-xcpθ

Equation of Motion for Plunge (Vertical Force Balance)

Fy=muy=mh-xCPθ

-L-Khh=mh-mxCPθ

mh-mxCPθ+Khh+L=0

IP=I0+mxcp2

Translation of 2nd moment of inertia

Equation of Motion for Pitch (Moment Balance about P)

MP=Ioθ-muyxCP

LxQP-Kθθ=IOθ-muyxCP

=IOθ-mh-xCPθxcp

=IOθ-mxCPh+mxcp2θ

=-mxCPh+Io+mxcp2θ

=-mxCPh+Ipθ

-mxCPh+IPθ+Kθθ-LxQP=0

#### Part b:

- Make linearity assumption on lift L and express L(U,q)=L0(U)q, where U is the airspeed

Assumptions:

- Linear Dependence on angle of attack

- Symmetric airfoil (No moment at Angle of Attack equal 0)

- Ideal lift curve slope a1=2π

- Subsonic and incompressible

- Rigid airfoil (linear analysis)

CL

Fully attached airflow

- Ignore drag effects

- Small angle approximation θ≪1 rad; sinθ≈θcosθ≈1

Lift is generated by an airfoil as described by the lift equation:

L=12ρU2SCL

a1=2π

Dynamic Pressure=q=12ρU2

θ

CL=a1θ

| Vocab | Description |
| --- | --- |
| L | Lift Force |
| ρ | Air Density |
| U | Airspeed |
| S | Wing Area |
| CL | Lift Coefficient |

LU=L0uθ

L=12ρU2Sa1θ

LDU=12ρU2Sa1

L(U) can be plugged into EOM for Plunge and Pitch:

EOM Plunge:

mh-mxCPθ+Khh+L0uθ=0

EOM Pitch:

-mxCPh+IPθ+Kθθ-L0uθxQP=0

#### Part c:

- Express equations of motion in terms of the plunge and pitch frequencies wh, wq

EOM Plunge: mh-mxcpθ+Khh+L0uθ=0

EOM Pitch: -mxcph+IPθ+Kθθ-L0uθxQP=0

1 Degree of Freedom Vibration Analysis:

Equation of motion for a mass attached to a spring (k):

mx+kx=0

Assumed Solution:

x=xest

x=amplitude

Substitute into the equation:

x=xest

x=s2x

ms2+kx=0

For a nontrivial solution x≠0 so the terms inside the parentheses must equal zero:

ms2+k=0→s2=-km=-ωn2

ωn=natural freq,rads

fn=ωn2π, Hz

So, the solution is ‘s’:

s=±iωn,  ωn=km

So:

xt=c1eiωt+c2e-iωt=Ccos(ωt+φ)

Uncoupled free vibration of airfoil (U=0, xCP=0)

Plunge:

mh+Khh=0

h+ωh2h=0

ωh2=Khm

The plunge motion is governed by equation mh+Khh=0, with a natural frequency of ωh=Khm

Pitch:

I0θ+Kθθ=0

θ+ωθ2θ=0

ωθ2=KθI0

The plunge motion is governed by equation I0θ+Kθθ=0, with a natural frequency of ωθ=KθI0

EOM Plunge: mh-mxcpθ+Khh+L0uθ=0

Divide the EOM for Plunge by m:

h-xCPθ+Khmh+L0umθ=0

h-xCPθ+ωh2h+L0umθ=0

EOM Pitch: -mxcph+IPθ+Kθθ-L0uθxQP=0

Divide the EOM for Pitch by I0:

-mI0xCPh+IPI0θ+KθI0θ-L0uxQPI0θ=0

-mI0xCPh+IPI0θ+ωθ2θ-L0uxQPI0θ=0

EOM Plunge in terms of ωh:

s2h-xCPs2θ+ωh2h+L0UxQPI0θ=0

EOM Pitch in terms of ωθ:

-mI0xCPs2h+IPI0s2θ+ωθ2θ-L0UxQPI0θ=0

#### Part d:

- Assume ,  and cast equations of motion in matrix form as a polynomial eigenvalue problem

Now assume that ht=hest,  θt=θest:

EOM Plunge in terms of ωh:

s2h-xcps2θ+ωh2h+L0UxQPI0θ=0

Aerodynamic Mass Matrix is zero because of the simplified aerodynamics

Aerodynamic Stiffness Matrix

Structural Stiffness Matrix

EOM Pitch in terms of ωθ:

-mI0xCPs2h+IPI0s2θ+ωθ2θ-L0UxQPI0θ=0

Structural mass Matrix

EOM for both Plunge and Pitch can be placed in matrix form.

s21-xCP-mI0xCPIPI0hθ+ωh200ωθ2hθ+0L0Um0-L0UxQPI0hθ=00

Polynomial Eigenvalue Problem: s2MSx+KSx+KAUx=0

MS=1-xCP-mI0xCPIPI0

KS=ωh200ωθ2

KAU=0L0Um0-L0UxQPI0

MA=0000

Eigenvalue Problem: s2M+KU=0

M=MS+MA#Total system mass matrix

KU=KS+KAU#Total system stiffness matrix

The eigenvalue s provides info on the natural frequency and the damping of the system. If the real part of s is negative than the system should return to equilibrium overtime.

# Ground Vibration Test (GVT) Analysis

Consider the airfoil shown in Figure 1 but with zero airspeed U=0. This condition corresponds to ground vibration testing (GVT). Do the following:

- Display input data

- Calculate the spring stiffnesses Kh and K corresponding to the uncoupled frequencies fh and fq

- Let xCP= –10% of c and calculate the following:

  - xCP and the moment of inertia IP about elastic center P

  - structural mass matrix S and structural stiffness matrix KS

  - eigenvalues s1, s2, s3, s4 and eigenvectors V1, V2, V3, V4 (sort eigenvalues in ascending amplitude order, rearrange eigenvectors according to the eigenvalue sorting, and normalize each eigenvector wrt its largest element)

  - coupled vibration frequencies fI and fII, uncoupled frequencies fh, f and percentage difference

  - modeshapes VI, VII corresponding to the frequencies fI, fII.

- Lat xCP = –20%, –10%, –1%, 0, 1%, 10% of c, 20% and calculate in table format the following:

- xCP and the moment of inertia IP about elastic center P

- eigenvalues s1, s2, s3, s4 and eigenvectors V1, V2, V3, V4 (sort eigenvalues in ascending amplitude order, rearrange eigenvectors according to the eigenvalue sorting, and normalize each eigenvector wrt its largest element)

- coupled vibration frequencies fI and fII and percentage difference from the uncoupled frequencies fh, f.

- modeshapes VI, VII corresponding to the frequencies fI, fII.

- Comment on your results.

### Solution

#### Part a:

#### Part b:

#### Part c:

#### Part d:

#### Part e:

Discussion of the results:

The GVT analysis revealed that the calculated spring stiffness for plunge and pitch were Kh=283.0Nm and Kθ=45.48 N*mrad, respectively, corresponding to the uncoupled frequencies fh=1.6Hz and fθ=4.8 Hz. As the static offset xcp was varied from -20% to 20% of ther chord, the moment of inertia about the elastic center IP ranged from 0.05 kg*m2 to 0.0679 kg*m2. The mode shapes associated with these eigenvalues indicate a smooth transition, with the coupled vibration frequencies fI=1.59 Hz and fII=4.83 Hz showing minimal deviation from their uncoupled counterparts. This suggests that the GVT system is showing stable behavior.

# Flutter Analysis

## Eigen analysis of flutter

Recall the aeroelastic equations of motion in matrix form expressed as a polynomial eigenvalue problem:
 where ,      M=MS+MA,      K(U)=KS+KA(U) 
Give the expressions for all the matrices

### Solution:

Structural mass Matrix

EOM for both Plunge and Pitch can be placed in matrix form.

s21-xCP-mI0xCPIPI0hθ+ωh200ωθ2hθ+0L0Um0-L0UxQPI0hθ=00

Aerodynamic Mass Matrix is zero because of the simplified aerodynamics

Aerodynamic Stiffness Matrix

Structural Stiffness Matrix

Polynomial Eigenvalue Problem: s2MSx+KSx+KAUx=0

MS=1-xCP-mI0xCPIPI0

KS=ωh200ωθ2

KAU=0L0Um0-L0UxQPI0

MA=0000

M=MS+MA=1-xCP-mI0xCPIPI0

KU=KS+KAU=ωh200ωθ20L0Um0-L0UxQPI0=ωh2L0Um0ωθ2-L0UxQPI0

## Flutter diagram

Let xCP= –10% of c, xQP= 30% of c. Let U=0,…,12 m/s with 1001 steps.

  - Display input data

  - Plot frequencies and damping vs. airspeed. Use these plots to find the flutter speed UF and insert datatips appropriately.

  - State the value of UF in m/s and knots.

### Solution:

#### Part a:

#### Part b:

#### Part c:

UF≈10.884 m/s

1knot=0.51444 m/s

UF≈10.884ms*1knot0.51444ms=21.15698 knots

## Frequencies and modeshapes near flutter speed

Let U= 0, 6m/s, (1–)UF, UF, (1+)UF, with =1%

- find frequencies and modeshapes at each of these speeds

- discuss your results

#### Part a:

#### Part b:

Discussion of the results:

The mode shapes of the first three flutter speeds all consist of real numbers, whereas the last two flutter speeds U=10.884 m/s and U=10.9928 m/s both show that the airspeed has approached its critical value as can be seen by the imaginary numbers for the mode shapes for these two flutter speeds.

## Static offset effect on flutter speed

Let U=9,…,15 m/s with 1001 steps. Keep xQP= 30% of c.

Let xCP = –20%, –15%, –10%, –5%, –1%, –0.1% of c. Do the following:

- Display input data

- Plot overlapped (hold on) frequencies and damping vs. airspeed for each value of xCP. Insert datatips to locate the flutter speed UF. Present the UF results in table format.

- Plot the variation of flutter speed UF with xCP%. Make sure the plot covers the whole U range.

- Discuss how xCP influences the flutter speed.

- Repeat for xCP = 0, 0.1%, 0.5%, 10%, 15%, 20% of c.

### Solution

#### Part a:

#### Part b:

#### Part c:

#### Part d:

Discussion of the results:

The static offset effect on flutter speed analysis showed that as xcp increased from -20% to -0.1% of the chord, the flutter speed increased from 9.6 m/s to 14.47 m/s. This increase indicates that moving the center of mass closer to the aerodynamic center improves the stability of the system by increasing the flutter speed.

#### Part e:

There is no flutter for values greater than xcp≥0

## Aerodynamic offset effect on flutter speed

Let U=10,…,13 m/s with 1001 steps. Keep xCP= –10% of c.

Let xQP = 20%, 25%, 30% of c. Do the following:

- Display input data

- Plot overlapped (hold on) frequencies and damping vs. airspeed for each value of xCP. Insert datatips to locate the flutter speed UF. Present the UF results in table form.

- Plot the variation of flutter speed UF with xQP%. Make sure the plot covers the whole U range.

- Discuss how xQP influences the flutter speed.

### Solution:

#### Part a:

#### Part b:

#### Part c:

#### Part d:

Discussion of the results:

As xQP increased from 20% to 30% of the chord, the flutter speed decreased from 12.2 m/s to 10.88 m/s respectively. The graph in part c shows a linear relationship where xQP is inversely proportional to UF. So increases in xQP for this airfoil indicate that the airfoil will become more susceptible to flutter, increasing the risk of dangerous oscillations.

## Divergence speed

- Let xCP= –10% of c, xQP= 30% of c. Let U=0,…, 20 m/s with 1001 steps.

- Display input data

- Plot extended flutter diagram

- Identify with a datatip the location of UD on the extended flutter diagram

- Let xQP = 20%, 25%, 30% of c. Calculate and plot divergence speed UD vs. xQP%. Make sure the plot covers the whole range of UD.

- Discuss the comparison between the results of items (a) and (b)

### Solution:

#### Part a:

#### Part b:

#### Part c:

Discussion of the results:

The comparison between parts (a) and (b) show that as the aerodynamic offset xQP increases, the divergence speed UD decreases. Both the extended flutter diagram and the linear trend the UD vs xQP plot consistently show this relationship.

# Damped GVT and Flutter

## Analysis of damped GVT and flutter

- Recall from class the damped flutter equations of motion in terms of the plunge and pitch frequencies fh, f, damping h, , inertias m, I0, IP, and lift function L0(U). Clearly state your assumptions.

- Assume ,  and cast the equations of motion in matrix form as an polynomial eigenvalue problem 
 where ,     M=MS+MA,    C=CS+CA,     K(U)=KS+KA(U)
Give the expressions for all the matrices.

### Solution:

#### Part a:

Damped 1-DOF Vibration

Assumptions:

- Linear Dependence on angle of attack

- Symmetric airfoil (No moment at Angle of Attack equal 0)

- Ideal lift curve slope a1=2π

- Subsonic and incompressible

- Rigid airfoil (linear analysis)

- Fully attached airflow

- Ignore drag effects

- Small angle approximation θ≪1 rad; sinθ≈θcosθ≈1

mx+cx+kx=0

Where:

- m is mass

- c is the damping coefficient

- k is the stiffness

- x is the displacement

Assumed solution: x(t)=xest

x=xest

x=sxest

x=s2xest

Plugging in the solution:

ms2xest+csxest+kxest=0

ms2x+csx+kx=0

ms2+cs+kx=0

For a nontrivial solution x≠0

ms2+cs+k=0

s1,2=-c±c2-4mk2m=-c2m±ikm-c2m2

The critical damping is defined as ccr=2mk

Damping ratio: ζ=cccr=c2mk=c2m1km

c2m=ζωn

s1,2 =-ζωn±iωn2-ζ2ωn2=-ζωn±iωn1-ζ2

s1,2 =-ζωn±iωd

Solution to the Differential Equation:

xt=c1es1t+c2es2t
Substitute s1, s2 into x(t):

x(t)=e-ζωntc1eiωdt+c2e-iωdt

Written using the Euler’s Formula:

x(t)=Ce-ζωntcos⁡(ωdt+φ)

Equation of Motion for Plunge with Damping

mh-mxCPθ+chh+Khh+L=0

Equation of Motion for Pitch with Damping

-mxCPh+IPθ+cθθ+Kθθ-LxQP=0

#### Part b:

Assume ,  and cast the equations of motion in matrix form as an polynomial eigenvalue problem  where ,     M=MS+MA,    C=CS+CA,     K(U)=KS+KA(U)
Give the expressions for all the matrices.

Assumption: It has a harmonic solution

h(t)=hest

h(t)=shest

h(t)=s2hest

θt=θest

θt=sθest

θt=s2θest

Substitute the harmonic solution into the EOM’s

Equation of Motion for Plunge with Damping

ms2hest-mxCPs2θest+chshest+Khhest+L=0

L represents an aerodynamic force and is a function of airspeed and not time-dependent, so the term est doesn’t apply to the external forces like L unless problem statement states L varies with time

ms2h-mxCPs2θ+chsh+Khh+L=0

Equation of Motion for Pitch with Damping

-mxCPs2h+IPs2θ+cθsθ+Kθθ-LxQP=0

Matrix Form:

ms2+chs+Kh-mxCPs2-mxCPs2IPs2+cθs+Kθhθ=-LLxQP

Here is the matrix form of the equations of motion for plunge and pitch of the airfoil with damping and stiffness

M=m-mxCP-mxCPIP

C=ch00cθ

KS=Kh00Kθ

KAU=0L0Um0-L0UxQPIP

KU=KS+KA=KhL0Um0Kθ-L0UxQPIP

Polynomial Eigenvalue Problem

s2m-mxCP-mxCPIP+sch00cθ+KhL0Um0Kθ-L0UxQPIPhθ=00

## Setup for damped GVT and flutter Calculations

Augment input data with the following modal damping values:

- Plunge damping ratio: h=2%

- Pitch damping ratio: =3%

Display updated input data

### Solution:

## Damped GVT analysis

Let U=0 and study the effect of damping on GVT:

  - Calculate damped frequencies and compare them with the undamped frequencies

  - Calculate modal damping and compare with the uncoupled modal damping

  - Calculate damped modeshapes and compare them with the undamped modeshapes

  - Results Discussion

### Solution:

#### Part a:

#### Part b:

#### Part c:

#### Part d:

Discussion of the results:

The introduction of damping into the GVT analysis had a noticeable effect on the system’s response. The damped frequencies were ever so slightly less than that of the undamped frequencies. Something notable is that the damped mode shapes include both real and imaginary numbers whereas the undamped are only real number mode shapes.

## Damped flutter analysis

Let U=0…14 m/s with 1001 steps and study the effect of damping on flutter speed UF

- Plot frequencies and damping vs. airspeed; find the damped flutter speed UF on these plots.

- State the value of UF in m/s and knots

- Plot the undamped diagram on top of the damped diagram and identify the undamped flutter speed  and the damped flutter speed UF.

- Discuss the difference between the damped and undamped flutter speeds

### Solution:

#### Part a:

#### Part b:

UF=10.64ms=20.69 knots

UF=10.64ms*1.94384knots1ms=20.68 knots

#### Part c:

UF=10.64ms

UFundamped=10.864ms

#### Part d:

Discussion of the results:

The damped flutter analysis shows that the flutter speed UF with damping is slightly lower than that of the undamped. Specifically, it was found that the damped flutter speed was UFdamped=10.64 m/s and the undamped is UFundamped=10.864 m/s.

# Extra Credit Challenge Problems

Challenge problems are optional.

## GVT modeling

Consider the airfoil shown in Figure 1 but with zero airspeed U=0. This condition corresponds to ground vibration testing (GVT). Do the following:

- Deduce equations of motion. Clearly state your assumptions.

- Assume ,  and cast equations of motion in matrix form as a polynomial eigenvalue problem  where

### Solution:

#### Part a:

At airspeed equal zero than lift is also zero

Equation of Motion for Plunge (Vertical Force Balance)

Fy=muy=mh-xCPθ

-L-Khh=mh-mxCPθ

mh-mxCPθ+Khh=0

IP=I0+mxcp2

Translation of 2nd moment of inertia

Equation of Motion for Pitch (Moment Balance about P)

MP=Ioθ-muyxCP

LxQP-Kθθ=IOθ-muyxCP

=IOθ-mh-xCPθxcp

=IOθ-mxCPh+mxcp2θ

=-mxCPh+Io+mxcp2θ

=-mxCPh+Ipθ

-mxCPh+IPθ+Kθθ=0

#### Part b:

Assumption: It has a harmonic solution

h(t)=hest

h(t)=shest

h(t)=s2hest

θt=θest

θt=sθest

θt=s2θest

ωh2=Khm                 ωθ2=KθI0

EOM Plunge in terms of ωh:

mh-mxCPθ+Khh=0

ms2hest-mxCPs2θest+Khhest=0

ms2h-mxCPs2θ+Khh=0

s2h-xCPs2θ+Khmh=0

s2h-xCPs2θ+ωh2h=0#EOM of Plunge

EOM Pitch in terms of ωθ:

-mxCPh+IPθ+Kθθ-L0uθxQP=0

-mxCPs2hest+IPs2θest+Kθθest=0

-mxCPs2h+IPs2θ+Kθθ=0

-mIOxCPs2h+IPI0s2θ+KθI0θ=0

-mIOxCPs2h+IPI0s2θ+ωθ2θ=0#EOM of Pitch

EOM for Plunge and Pitch can be placed in matrix form:

s21-xCP-mI0xCPIPI0hθ+ωh200ωθ2hθ+0000hθ=00

Polynomial Eigenvalue Problem: s2MSx+KSx+KAUx=0

MS=1-xCP-mI0xCPIPI0 Structural Mass Matrix

KS=ωh200ωθ2 Structural Stiffness Matrix

KAU=0000 Aerodynamic Stiffness Matrix

MA=0000 Aerodynamic Mass Matrix is zero because of simplified aerodynamics

## Divergence speed modeling

Derive the expression of the divergence speed UD

#### Solution:

ωh2h+L0Umθ=ωh2fh(t)

θt=fθt1-L0UxQPI0ωθ2=fθtD(U)

DU=1-L0UxQPI0ωθ2

DUD=1-L0UDxQPI0ωθ2

L0UD=12ρUD2ca1

IOωθ2=xQPa112 ρUD2c

UD=ωθ2I0xQPa1ρc