Initial Settings:

Frequency: 20kHz

Amplitude(peak-to-peak): 10V

Data from Experiment:

Predicted Results from MATLAB:

Code For MATLAB:

clc;

clear;

format compact;

% input data

f=20e3;

R=100; C=0.1e-6; Va=10/2; VPhase=0;

om=2*pi*f;

Vs=Va*exp(j*VPhase);

%capacitor impedance

Zc=-j/(om*C);

ZT=R+Zc;

I=Vs/ZT;

Vc=I*Zc;

Vcm=abs(Vc)

Vcph=angle(Vc)

This code returns:

Voltage at Capacitor = 3.1134 V

And the phase shift = -0.8986 rad

However, when looking at the actual voltage provided to the circuit, it is seen that it is 7.809 V peak to peak, so if we put that new data into MATLAB, we get:

Voltage at Capacitor = 2.4312 V

And the phase shift = -0.8986 rad

Actual Data retrieved from the experiment:

Measured Amplitude of Input Voltage (peak to peak) is 7.809 V; thus the Amplitude is: 3.9045 V

The amplitude of the voltage at capacitor(p-to-p) is 5.155 V, thus the Amplitude is: 2.577V

The Time shift between the 2 voltages is -7.152 μs

The Phase shift is given by ϕ = Δt*2πT, where T =1f

f=20e3;

dt=-7.152e-6;

T=1/f;

dt*2*pi/T

Using the MATLAB code above, we get that the phase shift is: -0.8987 rad

The results of this experiment are reasonably close to our simulations if the initial voltage difference is accounted for. The difference in Output Voltage Amplitude has a difference of approximately 6 percent and the phase shift is 0.01 percent

I believe the large difference between initial voltage and our measured initial voltage is caused because my computer could not supply enough power.