Project 5: Westminster Chime Clock with LED Indicators

Components:

- 1 Button

- 4 LEDs

- 1 Piezo Buzzer

Functionality Overview:

This project simulates the Westminster Chime sequence with accompanying LED patterns. The sequence progresses through the quarters of the Westminster Chime every minute and increases the 'hour' count every full cycle (4 minutes).

Detailed Operation:

- Starting the Cycle:

  - The cycle initiates when the button is pressed.

- LED Illumination:

  - LEDs sequentially light up over the first minute. By the end of this period, all LEDs should be illuminated.

  - The lighting pattern of the LEDs mirrors the rhythm of the Westminster Chimes. (Refer to Section 7 for note-to-LED mapping details).

- Chime Sequence:

  - At the 1-minute mark, the piezo plays the first quarter of the Westminster Chimes.

  - At the 2-minute mark, the second quarter is played.

  - At the 3-minute mark, the third quarter is played.

  - At the 4-minute mark, the fourth and final quarter is played, followed by the 'hour' chime.

- Hour Chime:

  - Immediately after the final quarter, the 'hour chime' sounds.

  - The number of 'hour chimes' played corresponds to the number of completed 4-minute cycles, increasing by one for each cycle, up to a maximum of 12. After reaching 12, it resets to 1.

  - Concurrently, all 4 LEDs illuminate together during each 'hour chime' to visually signify the hour.

- Reset Functionality:

  - Pressing the button during any stage of the cycle resets the process.

  - Following a reset, the cycle remains inactive until the button is pressed again, which reinitiates the process from the beginning as described in Section 1.

- Appendix: Westminster Chime Notes, Frequencies, and Note Durations

Note Frequencies and LED Mapping

Each note in the Westminster Chime sequence corresponds to a specific LED, arranged from left to right. It's advisable to round the tone frequencies as needed for practical implementation.

- G♯4: 415.3 Hz - LED 1

- F♯4: 369.99 Hz - LED 2

- E4: 329.63 Hz - LED 3

- B3: 246.94 Hz - LED 4

Note Durations

- Crotchet (Quarter Note): 1 beat. At 120 beats per minute, this equals 500 milliseconds.

- Minim (Half Note): 2 beats. At 120 beats per minute, this equals 1000 milliseconds.

- Semibreve (Whole Note): 4 beats. At 120 beats per minute, this equals 2000 milliseconds.

- Pause Between Notes: Approximately 150-250 milliseconds.

Note on Sequence and Duration

In each chime sequence, the final note is a minim (half note), played for twice the duration of a crotchet (quarter note). The first three notes in each sequence are crotchets.

Chime Sequences for Each Quarter

- First Quarter (Change 1):

  - Sequence: G♯4, F♯4, E4, B3 - 0

- Half Hour (Changes 2 and 3):

  - Sequence 1: E4, G♯4, F♯4, B3 - 1

  - Sequence 2: E4, F♯4, G♯4, E4 - 2

- Third Quarter (Changes 4, 5, and 1):

  - Sequence 1: G♯4, E4, F♯4, B3 - 3

  - Sequence 2: B3, F♯4, G♯4, E4 - 4

  - Sequence 3: G♯4, F♯4, E4, B3 -0

- Full Hour (Changes 2, 3, 4, and 5):

  - Sequence 1: E4, G♯4, F♯4, B3 - 1

  - Sequence 2: E4, F♯4, G♯4, E4 - 2

  - Sequence 3: G♯4, E4, F♯4, B3 - 3

  - Sequence 4: B3, F♯4, G♯4, E4 - 4

- Hour Gong Tone

- The hour gong tone is in E3, with a frequency of approximately 164.81 Hz.

- The gong is played as a whole note, with a pause of 500 milliseconds between gongs.