Convert the following to MIPS:

Assume #A = $s0

A[3] = A[2]-A[1]

Solution:

A[1] ⇒ 			lw $t0, 4($s0)

A[2] ⇒ 			lw $t1, 8($s0)

A[2]-A[1] ⇒ 		sub $t2, $t0, $t1

A[3] = A[2]-A[1] ⇒ 	sw $t2, 12($s0)

Convert 0xaaccee12 to Little-Endian and Big-Endian forms:

Solution:

|  | Little Endian | Big Endian |
| --- | --- | --- |
| Mem [0] | 12 | aa |
| Mem [1] | ee | cc |
| Mem [2] | cc | ee |
| Mem [3] | aa | 12 |

Bitwise AND 12 and 3 together, i.e., 12 & 3

Solution:

12=1100; 3=0011

12&3=0000

Bitwise OR 15 and 2 together, i.e., 15 | 2

Solution:

15=1111; 2=0010

15|2=1111

Convert the following to HEX: 0011 1010 1100

Solution:

0011=3; 1010=A; 1100=C

Answer: 0x3AC

Convert the following to BINARY: 0XE38

Solution:

E=1110; 3=0011; 8=1000

Answer: 1110 0011 1000

What is the value of the following if it is unsigned?  0001 1010

Solution:

2^1 + 2^3 + 2^4 = 26

What is the value of the following if it is Two’s Complement?  1111 0110

Solution:

Subtract 1 first:

1111 0110

-1________

1111 0101

Flip all bits:

1111 0101

0000 1010

Convert to Decimal:

1010 = 2^1 + 2^3 = 10

ANSWER: -10

Convert the following C Code to MIPS:

while(j<10):

print(j);

i++;

Assume j=$s1; i=$s0

Solution:

li $t0, 10

loop:

bge $s1, St0, exit

li $v0, 1

move $a0, $s1

addi $s0, $s0, 1

exit:

li $v0, 10

syscall

Convert the following MIPS to Hex:

add $s2, $s1, $t1

addi $t0, $t2, 7

Solution:

000000 10001 01001 10010 00000 100000

⇒ 0x02299020

001000 01010 01000 0000000000000111

⇒ 0x21480007

Convert the following HEX to MIPS:

0x01095022

Solution:

Convert to Binary:

0000 0001 0000 1001 0101 0000 0010 0010

OPCODE ⇒ 000000, so R-type

Referring to Table:

sub $t2, $t0, $t1

An app runs in 35 seconds. A new compiler requires 0.4 as many instructions, but increases the CPI to 1.3. Can we expect a speedup?  What is the final time to run?

Solution:

Time = (35*0.4) * 1.3 / (1 Hz) = 18 seconds

2 implementations with the same ISA (instructions have same amount of cycles)

-Comp. A has a cycle time of 300ps and has a CPI of 1.8

-Comp. B has a cycle time of 650ps and has a CPI of 1.3

Which is faster and by how much?

Solution:

TimeA=1.8*300=540

TimeB=1.3*650=845

A is faster.

Mobile game runs in 200s; operation we are improving is responsible for 150s. How much improvement is needed for a 2x overall speedup?

Solution:

Time wanted: 200/2=100

100=(150/improvement) + (200-150) ⇒ improvement=3

You are refactoring some old code. You refactor a method that takes up 60% of the runtime and after you fix it, the method is 3 times faster. What is the speedup?

Solution:

Speedup=1/(1-0.6)+.6/3 = 2.7

Convert the following Decimal to Floating Point: 0.875

Solution:

Since 0.875<1, multiply by 2-> 1.75*2^-1

Exponent=-1+127=126

1.75-1=0.75-0.5=0.25-0.25=0=0=0=0=0=0=0

Final Result: 0 | 01111110 | 110 0000 0000 0000 0000 0000

5*6 via Booth’s algorithm

0101 * 0110 (5*6)

m = 0101

r = 0110

-m = 1011 (2’s complement of m)

L = 9, from (len(m) + len(r) + 1)

Convert values to be length L

m = 010100000

-m = 101100000

r = 000001100

Loop 4 times (original len(r))

Step 1: 000001100

Last 2 bits are 00, do nothing, shift right 1

Step 2: 000000110

Last 2 bits are 10, add -m

000000110 + 101100000 = 101100110

Shift right 1

Step 3: 110110011

Last 2 bits are 11, shift right by 1

Step 4: 111011001

Last 2 bits a 01, add m

111011001 + 010100000= 001111001

Shift right 1

Final value is: 000111100

Drop the rightmost bit

000011110 = 30