#1

A voltage of 0.25 V was arbitrarily chosen for a nominal value that the oscilloscope could detect without an egregious amount of distortion.

Using Vout=VinRR+Rmotor, and evaluating the motor resistance at a lower resistance: Rmotor=IV, the minimum resistance can be determined as 2.56Ω.

Calculating for the highest resistance by establishing the voltage across the resistor as 10% of the input voltage, 0.1*Vin=VinRR+Rmotor, and solving for motor resistance as previously, the maximum resistance can be calculated as 3.7 Ω.

Reviewing common resistor values, the most practical option appeared to be a 2.2 Ω resistor which results in a minimum voltage of 0.215 V.

The power rating of the aforementioned resistor would be P=VinRR+Rmotor2R which would be equal to 0.174 Watts, thus a rating of ¼ Watt would be appropriate.

#2

To design a Low-Pass filter, one must have a series of design specifications, and in this case, the deign parameter provided is the cutoff frequency, which is specified as approximately 50 Hz.

The formula for Low-Pass Filters is: f=12πRC. In this case, there are 2 unknown variables and 1 equation, thus 1 variable must be predetermined beforehand. Because a 2.2 Ω resistor has already been used, it would be beneficial to use another one in construction of the Low-Pass filter. Solving the equation, C is found to be 1.44 mF, however the closest common capacitor size is 1400 μF. This capacitor value results in a cutoff frequency of 51.67 Hz.

#3

Estimate torque coefficient:

A=0.15; m=0.1; r=0.05; g=9.81

T=k*I=m*r*g→0.1*0.05*9.81=k*0.15→k=0.327