EMCH 290-001-EE#1                  Due at 11:59 on Thursday, September 13, 2022

Points Possible: 100

- Question 1(15 Points)

The drag force, Fd, imposed by the surrounding air on a vehicle moving with velocity V is given by Fd= Cd*A*ρ*V22  where Cd is a constant called the drag coefficient, A is the projected frontal area of the vehicle, and ρ is the air density.  An automobile is moving at V = 36 kilometers per hour with Cd = 0.3, A = 2 m2, and ρ = 1.2 kg/m3.

- Determine the force, in N, required to overcome aerodynamic drag.

- Determine the power, in J, required to overcome aerodynamic drag.

- Question 2(18 Points)

Shown in the figure, an exterior brick wall has a thermal conductivity of 1.82 W/m2·oK. and a width of L = 0.15 m. At steady state, the temperature of the wall decreases linearly from T1 = 295oK on the inner surface to T2 on the outer surface. The outside ambient air temperature is T0 = 272oK and the convective heat transfer coefficient is 29 W/m2·oK.

- Determine the temperature T2 in oK

- Determine the rate of heat transfer through the wall, in W/m2.

- Question 3(20 Points)

A composite plane wall consists of a Li = 50 mm thick layer of insulation (ki = 0.05 W/m·K) and a Ls = 25 mm thick layer of siding (ks = 0.10 W/m·K). The inner temperature of the insulation is 300oK. The outer temperature of the siding is 265oK. Steady state conditions apply.

- Determine the temperature at the interface between the two layers, in oK.

- Determine the rate of heat transfer through the wall, in W/m2.

- Question 4(15 Points)

A gas is compressed in a piston–cylinder assembly from p1 = 2 bar to p2 = 8 bar, V2 = 0.02 m3 in a process during which the relation between pressure and volume is pV1.7 = constant. The mass of the gas is 0.2 kg. The specific internal energy of the gas increases by 50 kJ/kg during the process. Kinetic and potential energy changes are negligible. For the gas as the system:

- Determine the work for the process, in kJ.

- Determine the heat transfer, in kJ.

- Question 5(17 Points)

Steam in a piston–cylinder assembly undergoes a polytropic process, with n = 2, from an initial state where V1 = 0.075 m3, p1 = 3450 kPa, and u1 = 3075.9 kJ/kg to a final state where u2 = 2410 kJ/kg and v2 = 0.211 m3/kg. The mass of the steam is 0.5 kg. Changes in kinetic and potential energy can be neglected.

- Determine the change in volume, in m3.

- Determine the energy transfer by work, in kJ.

- Determine the energy transfer by heat, in kJ.

- Question 6(15 Points)

0.4[kg] of a gas is contained within a piston. The gas undergoes a process for which the pressure–volume relationship is P*V1.5=Cconstant. The initial pressure is 3[bar], the initial volume is 0.1m3, and the final volume is 0.2m3. The change in specific internal energy of the gas in the process is u2-u1=-55kJkg. Kinetic and potential energy are negligible. Determine the work and net heat transfer for the process[kJ].