EMCH 290-001-EE#4                                                               Due on Tuesday, October 31, 2023

Points Possible: 100

This assignment will add 2% to your FINAL grade if completed accurately.

- Question 1(25 Points)

20 m3/hr of air at 600 kPa, 330 K enters a well-insulated, horizontal pipe having a diameter of 1.2 cm and exits at 120 kPa. Assume steady state and use the ideal gas model for the air. Also assume constant specific heat, cp = 1.007 kJ/kg·K for air at 330 K.

- Determine the mass flow rate, in kg/s.

- Determine the exit velocity, in m/s.

- Question 2(25 Points)

As shown in the figure, Refrigerant 22 enters the compressor of an air conditioning unit operating at steady state at 5oC, 5.5 bar and is compressed to 70oC, 14 bar. The refrigerant exiting the compressor enters a condenser where energy transfer to air as a separate stream occurs, and the refrigerant exits as a liquid at 14 bar, 32oC. Air enters the condenser at 7°C, 1 bar with a volumetric flow rate of 1.7 m3/hr and exits at 47oC. Neglect stray heat transfer and kinetic and potential energy effects and assume ideal gas behavior for the air.

- Determine the mass flow rate of refrigerant, in kg/s.

- Determine the compressor power, in kW.

- Question 3(25 Points)

A tank whose volume is 15 liters is initially evacuated. A pinhole develops in the wall, and air from the surroundings at 21oC, 1 bar enters until the pressure in the tank is 1 bar. The final temperature of the air in the tank is 21oC. Determine the final mass in the tank, in g, and for the tank as the control volume, determine the heat transfer, in kJ.

- Determine the final mass in the tank, in g.

- For the tank as the control volume, determine the heat transfer, in kJ.

- Question 4(25 Points)

The rigid tank shown in the figure has a volume of 0.06 m3 and initially contains a two-phase liquid–vapor mixture of ammonia at a pressure of 2 bar and a quality of 20%. As the tank contents are heated, a pressure-regulating valve keeps the pressure constant in the tank by allowing saturated vapor to escape. The heating continues until the final quality is 50%.  Kinetic and potential energy effects can be neglected.

- Determine the final mass in the tank, in kg.

- Determine the amount of heat transfer, in kJ, for the process.