- Question 1

A reversible power cycle operating as in the figure receives energy QH by heat transfer from a hot reservoir at TH and rejects energy QC = 30 Btu by heat transfer to a cold reservoir at TC = 40oF. The work for the cycle Wcycle = 30 Btu.

- Determine the thermal efficiency.

- Determine TH, in oF.

- Question 2

A power cycle operates between a lake’s surface water at a temperature of 25 oC and water at a depth whose temperature is 4oC. At steady state the cycle develops a power output of 12 kW, while rejecting energy by heat transfer to the lower temperature water at the rate 14,400 kJ/min.

- Determine the thermal efficiency of the power cycle.

- Determine the maximum thermal efficiency for any such power cycle.

- Is the power cycle possible or impossible?

The thermal efficiency is defined as:

η=Wcycle QH

Considering the energy balance for the cycle:

Wcycle =QH-QC

The thermal efficiency is:
η=WcycleQH=WcycleWcycle+QC=10 kW10 kW+14,400kJmin1 min60 s1 kW1 kJ/s=0.04
η=4%⟵ Thermal efficiency

The thermal efficiency for a reversible cycle is:

ηmax=1-TCTH=1-10+273K25+273K=0.050ηmax=5%

The power cycle is "possible". The thermal efficiency of a power cycle is always less than or equal to the maximum theoretical efficiency.

- Question 3

For the reversible cycle shown in the figure, p1 = 2.32 bar, v1 = 0.43 m3/kg, TH = 500 K, QH = 150 kJ, and the gas is air obeying the ideal gas model.

- Determine TC, in K.

- Determine the thermal efficiency of the cycle.

- Determine the net work of the cycle, in kJ.

Since Tc=T1, the ideal gas equation of state gives:

TC=T1=p1v1R=(2.0bar)105 N/m21bar0.43 m3/kg8.31428.97kJkg⋅K103 N⋅m1 kJTC=300 K

The thermal efficiency is defined as:

η=Wcycle QH=1-QCQH

For a reversible cycle:

QCQH=TCTH

So, the thermal efficiency is:

η=1-TCTH =1-300 K500 K=0.40η =40%

From the equation for thermal efficiency:

Wcycle =ηQH =(0.40)(150 kJ)Wcycle  =60 kJ

- Question 4

At steady state, a power cycle develops a power output of 10 kW while receiving energy by heat transfer at the rate of 10 kJ per cycle of operation from a source at temperature TH. The cycle rejects energy by heat transfer to cooling water at a lower temperature of 300 K. There are 145 cycles per minute. Determine the rate of heat rejection at the lower temperature, in kW and the minimum possible value for TH, in K. Determine the thermal efficiency of the cycle assuming the cycle is reversible.

- Determine the rate of heat rejection at the lower temperature, in kW.

- Determine the minimum possible value for TH, in K, for the given operating conditions.

- Determine the thermal efficiency of the cycle assuming the cycle is reversible.

From an energy balance on the system:

QC=QH-Wcycle =10kJcycle100cycles 1 min60 s1 kW1 kJ/s-10 kW=16.67 kW-10 kWQC=6.67 kW⟵ Energy rejection  to cooling water

The minimum theoretical temperature occurs when the cycle is reversible, and:

QCQH=TCTH,

Solving for the minimum temperature of the hot reservoir, TH, min  :

TH, =QHQCTC=16.67 kW6.67 kW(300 K)

TH, =750 K

Since the minimum theoretical temperature occurs when the cycle is reversible, the thermal efficiency is:

η=1-TcTh=1-300K750K

η=60%⟵ Thermal efficiency

- Question 5

As shown in the figure, a system undergoing a power cycle develops a net power output of 1 MW while receiving energy by heat transfer from steam condensing from saturated vapor to saturated liquid at a pressure of 180 kPa. Energy is discharged from the cycle by heat transfer to a nearby lake at 19oC. These are the only significant heat transfers. Kinetic and potential energy effects can be ignored and operation is at steady state.

- Determine the temperature of the condensing steam, in oC.

- Determine the minimum theoretical steam mass flow rate, in kg/s, required by any such cycle.

- Determine the maximum cycle efficiency.

From the saturation table, the temperature of the steam is:

TH=99.63∘C

The maximum possible energy transfer from the steam to the power cycle is:

QH=mhfg

where hfg is the enthalpy of vaporization for steam at 100kPa.hfg=2258.0 kJ/kg
The cycle efficiency must be less than that of a reversible cycle. The inequality is:

η=WcycleQH=Wcyclemhfg≤ηrev=1-TCTH=1-99.63+273 K17+273 K=0.222

Solving the inequality for m :

m≥Wcycleηrev hfg =(1MW)1000 kJ/s1MW(0.222)(2258.0 kJ/kg)m ≥2.00 kg/s

The cycle efficiency must be less than that of a reversible cycle. The inequality is:

η=WcycleQH=Wcyclemhfg≤ηrev=1-TCTH=1-99.63+273K17+273K=0.222

- Question 6

A power cycle operating at steady state receives energy by heat transfer from the combustion of fuel at an average temperature of 400oC. Owing to environmental considerations, the cycle discharges energy by heat transfer to the atmosphere at 30oC at a rate no greater than 60 MW. Based on the cost of fuel, the cost to supply the input heat transfer is $4.50 per GJ. The power developed by the cycle is valued at $0.08 per kW·hr. The power plant operates for 8000 hours annually.

- Determine maximum thermal efficiency of the cycle.

- Determine the maximum value of the power generated, in $ per year.

- For the maximum power generated, determine the corresponding fuel cost, in $ per year.

The maximum power generated occurs with the maximum allowable energy rejection to the atmosphere as the cycle operates reversibly. The thermal efficiency of a reversible cycle is:

ηmax=WcycleQH=1-TCTH=1-303K673K=0.55

From an energy rate balance on the control volume at steady state:

QH=Wcycle+QC

Solving each equation for QH and setting them equal yields:

Wcycle+QC=Wcycle1-TC/TH

Solving for Wcycle:  :

Wcycle=QCTH/TC-1≤(60MW)673 K303 K-1Wcycle≤73.27MW

Therefore:

Wcycle,  =73.27MW

Considering the plant operates 8000 hours annually, with a value of $0.08/kW-h, the maximum value of power generated annually is:

$]Wcycle,  =$0.08kw⋅h8000h year 73.27MW103 kW1MW

$]Wcycle,max =$46.9 Million/year

Returning to the energy rate balance, the rate of heat addition for the maximum rate of work is:

QH,Wcycle, =Wcycle, +QC, =73.27MW+60MW=133.27MW

At a cost of $4.50/GJ, the annual cost of fuel is:

$]fuel,Wcycle,max =$4.50GJ(133.27MW)1GJ/s103MW8000h year 3600 s1 h$]fuel,wcycle,  =$17.27 Million/year ⟵ Corresponding annual  fuel cost

- Question 7

A refrigeration machine rejects QH = 500 Btu per cycle to a hot reservoir at TH = 540oR, while receiving QC = 375.0 Btu per cycle from a cold reservoir at temperature TC. The machine operates at 10 cycles per minute.

- Determine the net power input, in hp.

- Determine the minimum theoretical temperature TC, in oR

- Determine the maximum coefficient of performance of the cycle.

- Question 8

A reversible heat pump cycle operates at steady state between hot and cold reservoirs at TH = 30oC and TC = 10oC, respectively. The rate of heat transfer at the high temperature is 10 kW.

- Determine the net power input, in kW.

- Determine the heat transfer rate from the cold reservoir at TC, in kW.

- Determine the coefficient of performance of the cycle.

- Question 9

An ice maker operating at steady state makes ice from liquid water at 32oF. Assume that 144 Btu/lb of energy must be removed by heat transfer to freeze water at 32oF and that the surroundings are at 71.50oF.  The ice maker consumes 1.4 kW of power.

- Determine the maximum theoretical rate that ice can be produced, in lb/h.

- Determine the rate of heat rejection to the surroundings, in Btu/h, for the case of maximum theoretical ice production.

- Determine the maximum coefficient of performance of the cycle.

- Question 10

The refrigerator shown in the figure operates at steady state with a coefficient of performance of 6.5 within a kitchen at 23oC. The refrigerator rejects 4.8 kW by heat transfer to its surroundings from metal coils located on its exterior.

- Determine the power input, in kW.

- Determine the lowest theoretical temperature inside the refrigerator, in oC.

- Determine the coefficient of performance when the temperature inside the refrigerator is the theoretical minimum.

.

- Question 11

As shown in in the figure, a heat pump operating at steady state provides energy by heat transfer to water vaporizing from saturated liquid to saturated vapor at a pressure of 2 bar with a mass flow rate of 0.050 kg/s. The heat pump receives energy by heat transfer from a pond at 14oC. The power required by the pump is 35 kW. These are the only significant heat transfers. Kinetic and potential energy effects can be ignored.

- Determine the coefficient of performance for the heat pump.

- Determine the required power, in kW, for a reversible heat pump under the same conditions.

- Determine the coefficient of performance for a reversible heat pump under the same conditions.

- Question 12

Two pounds of carbon dioxide (CO2) as an ideal gas executes a Carnot power cycle while operating between thermal reservoirs at 540 and 100oF. The pressures at the initial and final states of the isothermal expansion are 400 and 200 lbf/in2, respectively. The specific heat ratio is k = 1.24.

- Determine the thermal efficiency.

- Determine the work during the isothermal expansion, in Btu.

- Determine the work during the adiabatic expansion, in Btu.

- Question 13

A power cycle operates between hot and cold reservoirs at 500 K and 310 K, respectively. At steady state the cycle develops a power output of 10 MW.

- Determine the minimum theoretical rate at which energy is rejected by heat transfer to the cold reservoir, in MW.

- Question 14

Data are provided for two reversible refrigeration cycles. One cycle operates between hot and cold reservoirs at 27°C and 3°C, respectively. The other cycle operates between the same hot reservoir at 27°C and a cold reservoir at -35°C. The refrigerator removes the same amount of energy by heat transfer from its cold reservoir.

- Determine the ratio of the net work input values of the two cycles, WCycle,2WCycle,1,

- Question 15

A refrigeration cycle driven by an electric motor must maintain a computer laboratory at 18°C when the outside temperature is 30°C The thermal load consists of heat transfers entering through the walls and roof of the laboratory at a rate of 75,000 kJ/h and from the computers, lighting, and occupants at a rate of 15,000 kJ/h.

- Determine the minimum theoretical power required by the electric motor, in kW, and the corresponding coefficient of performance.

- If the actual power required by the motor for this duty is 4 kW, determine the coefficient of performance.

- If the given temperature and thermal load data are observed for a total of 100 hours and electricity costs 13 cents per kW · h, determine the cost, in $, over that period for each of cases (a) and (b).

- Question 16

At steady state, a heat pump driven by an electric motor maintains the interior of a building at TH = 293 K. The rate of heat transfer, in kJ/h, from the building through its walls and roof is given by 8000(TH - TC), where TC is the outdoor temperature.

- Determine the minimum electric power, in kW, required to drive the heat pump for TC = 261 K.

- Question 17

By supplying energy at an average rate of 30,000 kJ/h, a heat pump maintains the temperature of a dwelling at 20°C.

If electricity costs 8.5 cents per kW·h, determine the minimum theoretical operating cost for each day of operation if the heat pump receives energy by heat transfer from:

- The outdoor air at -30°C.

- The ground at 2.5°C.

- Question 18

At steady state, a heat pump provides energy by heat transfer at the rate of 25,000 Btu/h to maintain a dwelling at 70°F on a day when the outside temperature is 40°F. The power input to the heat pump is 7 hp.

- Determine the coefficient of performance of the heat pump.

- Determine the coefficient of performance of a reversible heat pump operating between hot and cold reservoirs at 70°F and 40°F, respectively, and the corresponding rate at which energy would be provided by heat transfer to the dwelling for a power input of 7 hp.