EMCH 290-001-HW#2                  Due at 11:59 on Thursday, September 13, 2022

Points Possible: 100

- Question 1(7 Points)

A flywheel whose moment of inertia I = 200 kg·m2 is spinning at 200 RPM. For a flywheel, KE = Iω2/2, where ω is the angular velocity, rad/s. Let g = 9.81 m/s2.

- How high above the surface of the earth, in ft, would a 50 kg mass have to be raised to have its potential energy equal to the kinetic energy of the flywheel?

- How high above the surface of the earth, in ft, would a 140 kg mass have to be raised to have its potential energy equal to the kinetic energy of the flywheel?

- Question 2(8 Points)

An object whose mass is 90 kg is projected straight upward from the surface of the earth and reaches a height of 900 m when its velocity reaches zero. The only force acting on the object is the force of gravity. The acceleration of gravity is g = 9.81m/s2.

- Determine the initial kinetic energy of the object, in J

- Determine the initial velocity of the object, in m/s.

- Question 3(9 Points)

Beginning from rest, an object of mass 200 kg slides down a 10-m-long ramp. The ramp is inclined at an angle of 60o from the horizontal. Air resistance and friction between the object and the ramp are negligible. Let g = 9.81 m/s2. Determine the kinetic energy of the object, in kJ, and the velocity of the object, in m/s, at the bottom of the ramp.

- Determine the kinetic energy of the object, in kJ, at the bottom of the ramp.

- Determine the velocity of the object, in m/s, at the bottom of the ramp.

- Question 4(7 Points)

A system with a mass of 6 kg, initially moving horizontally with a velocity of 40 m/s, experiences a constant horizontal force of 25 N opposing the direction of motion. As a result, the system comes to rest.

- Determine the amount of energy transfer by work, in kJ, for this process.

- Determine the total distance, in m, that the system travels

- Question 5(10 Points)

A closed system consisting of 10 kg of air undergoes a polytropic process from p1 = 450 kPa, v1 = 0.25 m3/kg to a final state where p2 = 140 kPa, v2 = 0.7 m3/kg. Determine the polytropic exponent, n, and the amount of energy transfer by work, in Btu, for the process.

- Determine the polytropic exponent, n, for the process.

- Determine the amount of energy transfer by work, in kJ, for the process.

- Question 6(7 Points)

A heat pump cycle operating at steady state receives energy by heat transfer from well water at 10oC and discharges energy by heat transfer to a building at the rate of 1.2x105 kJ/h. Over a period of 14 days, an electric meter records that 1620 kW·h of electricity is provided to the heat pump. These are the only energy transfers involved.

- Determine the total amount of energy transfer by work into the heat pump over the 14-day period, in GJ.

- Determine the amount of energy that the heat pump receives by heat transfer from the well water over the 14-day period, in GJ.

- Determine the heat pump’s coefficient of performance.

- Question 7(7 Points)

An electric motor operating in steady state draws a current of 11 amp at a voltage of 120 V. The output shaft develops a torque of 8.4 N·m and a rotational speed of 1200 RPM. (W=V*A)

- Determine the electric power, in kW.

- Determine the power developed by the output shaft, in kW.

- Determine the rate of heat transfer, in kW

- Question 8(7 Points)

An object whose weight is 100 kg experiences a decrease in kinetic energy of 500 N*m and an increase in potential energy of 1500 N*m. The initial velocity and elevation of the object, each relative to the surface of the earth, are 40 m/s and 30 m, respectively. If g=9.81 m/s^2, determine:

- The final velocity, in m/s.

- The final elevation, in m.

- Question 9(7 Points)

Find the mass flow rate (in grams per second) of water for a control volume system, if the volume flow rate is 2 liter per second.  [ρwater=1gcm3]

Volume flow rate = 2 (liter/s)

Mass flow rate =? (g/s)

- Question 10(7 Points)

What is the change in the internal energy of the system if the surrounding releases 300 J of heat energy and if the system does 550 J of work on the surroundings?

- Question 11(7 Points)

The refrigerator shown in the figure steadily receives a power input of 0.15 [kW] while rejecting energy by heat transfer to the surroundings at a rate of 0.6[kW]. Determine the rate at which energy is removed by heat transfer from the refrigerated space, in [kW], and the refrigerator's coefficient of performance.

- Question 12(7 Points)

For a power cycle operating as shown in the figure, Wcycle = 600 kJ and Qout = 1800 kJ.

- What is the heat input, Qin, in kJ?

- What is the cycle thermal efficiency?

- Question 13(10 Points)

A vertical piston–cylinder assembly with a piston of mass 35 kg and having a face area of 0.006 m2 contains air. The mass of air is 2.5 g, and initially the air occupies a volume of 0.0025 m3. The atmosphere exerts a pressure of 100 kPa on the top of the piston. The volume of the air slowly decreases to 0.001 m3 as energy with a magnitude of 1 kJ is slowly removed by heat transfer. Friction between the piston and the cylinder wall can be neglected and g = 9.8 m/s2.

- Determine the work done by the air on the piston during the process, in kJ.

- Determine the change in specific internal energy of the air, in kJ/kg.