Heating Ammonia at Constant Pressure A vertical piston–cylinder assembly containing 0.1 lb of ammonia, initially a saturated vapor, is placed on a hot plate. Due to the weight of the piston and the surrounding atmospheric pressure, the pressure of the ammonia is 20 lbf/in.2 Heating occurs slowly, and the ammonia expands at constant pressure until the final temperature is 77F. Show the initial and final states on T-v and p-v diagrams, and determine

(a) the volume occupied by the ammonia at each end state, in ft.

V= m*v

Where v is specific volume with temperature and pressure already accounted for.

Because it is a saturated vapor, we can use the given value of 13.497ft3lb

V1=0.1*13.497=1.3497 ft3

We can interpolate the value of v2 because we know that the pressure is 20lbfin2 and we know that the temperature is 77F.

We have 2 data points in the table, 70F:16.436ft3lband 80F:16.765ft3lb.

To estimate the value, we shall have to interpolate it. 16.765-16.436=0.329→ 0.329*710=0.2303→ 16.436+0.230=16.666.

V2=0.1*16.666=1.6666ft3

(b) the work for the process, in Btu.

We know W=V1V2p dV ;  1 BTU =778 ft*lbf; 144in2=1 ft3

P=20lbf/in.2*144in2ft2=2880lbf/ft2

W=1.3497ft31.6666ft32880lbfft2 dV=912.672ft*lbf*1BTU778ft*lbf=1.173BTU