1. (0.4 points) (Remember that input values which do not occur are “Don’t Cares.”)

Show truth tables for each of the following: The system has four inputs, a, b, c, and d, and one output, f. The last three inputs (b, c, d) represent a binary number, n, in the range 0 to 7; however, the input 0 never occurs. The first input (a) specifies which of two computations is made.

a = 0: f is 1 iff n is a multiple of 2

a = 1: f is 1 iff n is a multiple of 3.

| a | b | c | d | f |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | X |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | X |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

2. (0.4 points) For each of the following expressions, indicate which (if any) of the following apply (more than one may apply):

i. Product term

ii. SOP expression

iii. Sum term

iv. POS expression

c. b’c’d’

(i), (ii), and (iv) applies.

e. a’+ b

(ii), (iii), and (iv) applies.

3. (0.4 points) Using properties 1 to 10, reduce the following expressions to a minimum SOP form. Show each step (number of terms and number of literals in minimum shown in parentheses).

b. x’y’z’ + x’yz + xyz (2 terms, 5 literals)

x’y’z’ + (x’+x)yz 									P8a.

x’y’z’ + yz 										P5a., P3b.

c. x’y’z’ + x’y’z + xy’z + xyz’ (3 terms, 7 literals)

x’y’(z + z’) + xy’z + xyz’ 								P8a.

x’y’ + xy’z + xyz’ 									P5a.P3b.

y’(x’ + xz) + xyz’ 									P8a.

y’(x’ + z) + xyz’ 									P10a.

y’x’ + y’z + xyz’ 									P8a.

g. x’y’z’ + x’y’z + x’yz + xy’z + xyz + xyz’ (3 terms, 5 literals)

x’y’(z+z’) + x’yz + xy’z + xy(z+z’) 						(P8a.)*2

x’y’ + x’yz + xy’z + xy 								P5a., P3b.

y(x’z + x) + x’y’ + xy’z 								P8a.

y(x + z) + y’(x’ + xz) 								P10a., P8a.

yx + yz + y’(x’ + z) 								P8a., P10a.

yx + yz + y’x’ + y’z 								P8a.

z(y’ + y) + yx + y’x’ 								P8a.

z + yx + y’x’ 									P5a., P3b.

4. (0.4 points) Show a block diagram of a system using AND, OR, and NOT gates to implement the following functions. Assume that variables are available only uncomplemented. Do not manipulate the algebra.

b. ab + c(a + b)

5. (0.4 points) For each of the following circuits,

i. find an algebraic expression.

ii. put it in sum of product form.

- a’∙c∙(b+d)+a∙(c’+b∙d)

- a’cb + a’cd + ac’ + abd

6. (0.4 points) Find the complement of the following expressions. Only single variables may be complemented in the answer.

c. h = (a + b)(b’ + c) + d’(a’b + c)

h’=a + bb’ + c'*d’a’b + c' 					P11b

h’ =a + b' + b’ + c’*(d''+a'b+c') 				P11b., P11b.

h’ = a'b'+b''*c'*d+(a'b'*c') 					P11a., P11a., P7., P11a.

h’ = a'b'+b*c'*d+a''+b'*c'				P7., P11b.

h’ = a'b'+bc'*d+c'a+b' 						P7.

7. (0.4 points) For each of the following functions:

1. f(x, y, z)=∑m(1, 3, 6)

2. g(x, y, z)=∑m(0, 2, 4, 6)

a. Show the truth table.

b. Show an algebraic expression in sum of minterms form.

c. Show a minimum SOP expression (1: 2 terms, 5 literals; 2: 1 term, 1 literal).

1a) 					2a)

| x | y | z | f |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

| x | y | z | f |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

1b) fx,y,z=x'y'z+x'yz+xyz'

2b) gx,y,z=x'y'z'+x'yz'+xy'z'+xyz'

1c) fx,y,z=x'y'z+x'yz+xyz'

fx,y,z=x'z(y'+y)+xyz'					P8a

fx,y,z=x'z+xyz'						P5a, P3a

2c) gx,y,z=x'y'z'+x'yz'+xy'z'+xyz'

gx,y,z=x'z'y'+y+xz'y+y'				P8a, P8a

gx,y,z=x'z'+xz'						P5a, P3a, P5a, P3a

gx,y,z=z'x+x'						P8a

gx,y,z=z'							P5a, P3a

8. (0.4 points) For each of the following circuits,

i. find an algebraic expression.

ii. put it in SOP form.

- f(a,b,c,d) = a(bc)’+(c+d’)’

- fa,b,c,d= ab'+c'+c'd				P11b, P11a

fa,b,c,d= ab'+ac'+c'd				P8b

9. (0.4 points) Convert each of the following expressions to sum of products form:

b. (a’ + b + c’)(b + c’ + d)(b’ + d’)

(a'd+b+c')(b’+d’)			P8b

bd'+b'(a'd+c')				P14b

b'd+b'c'+b'a'd				P8a

a’b’d+b’c’+bd’

10. (0.4 points) Convert each of the following expressions to product of sums form:

b. w’xy’ + wxy + xz

x(w’y’ + wy + z)				P8a

x(z+wy+w')(z+wy+y')			P8b

x(z+w'+y)(z+w+y')			P10a