1. For the following circuit:

a. Compute the maximum delay,

i. Assuming that all inputs are available both uncomplemented and complemented

Answer:6Δ

ii. Assuming only uncomplemented inputs are available and an additional gate must be added to complement each input

Answer:6Δ

b. Compute the maximum delay from input C to the output, assuming that all inputs are available both uncomplemented and complemented.

ANSWER: 3Δ

2. (0.5 points).

#4 We want to build an adder to simultaneously add three multidigit binary numbers. Design a single bit of that adder. It has three inputs for that digit, x, y, and z, plus two carry inputs, u and v (since you may have a carry of 0, 1, or 2). There are three outputs, a sum, s, and two carries, f and g. Show a truth table and find the minimum sum of products expressions for the three outputs

| Input bits |  |  | Carry Bits |  | Carry Bits |  | Sum |  | Input bits |  |  | Carry Bits |  | Carry Bits |  | Sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x | y | z | u | v | f | g | s |  | x | y | z | u | v | f | g | s |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |  | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |  | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |  | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |  | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |  | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |  | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |  | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |  | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |

Minterms:

fx,y,z,u,v=xyzuv+xyzuv'+xyzu'v+xyz'uv+xy'zuv+x'yzuv

→𝑦𝑧𝑢𝑣+𝑥𝑧𝑢𝑣+𝑥𝑦𝑢𝑣+𝑥𝑦𝑧𝑣+𝑥𝑦𝑧𝑢

gx,y,z,u,v=x'y'zu'v+x'y'z'uv+x'y'zuv'+x'y'zuv+x'yz'u'v+x'yz'uv'+x'yz'uv+x'yzu'v'+x'yzu'v+x'yzuv'+xy'z'u'v+xy'z'uv'+xy'z'uv+xy'zu'v'+xy'zu'v+xy'zuv'+xyz'u'v'+xyz'u'v+xyz'uv'+xyzu'v'

→𝑥𝑦′𝑧′𝑢 + 𝑥′𝑦𝑧′𝑣 + 𝑦′𝑧𝑢𝑣′ + 𝑥′𝑦𝑧𝑣′ + 𝑥′𝑦′𝑢𝑣 + 𝑥𝑦′𝑧𝑢′ + 𝑦𝑧′𝑢𝑣′ + 𝑥𝑦𝑢′𝑣′ + 𝑥𝑧′𝑢′𝑣 + 𝑥′𝑧𝑢′𝑣

sx,y,z,u,v=x'y'z'u'v+x'y'z'uv'+x'y'zu'v'+x'y'zuv+x'yz'u'v'+x'yz'uv+x'yzu'v+x'yzuv'+xy'z'u'v'+xy'z'uv+xy'zu'v+xy'zuv'+xyz'u'v+xyz'uv'+xyzu'v'+xyzuv

3. The statement, “There is no enable input required,” describes the decoder you are building. Use the ones provided as indicated in the table and diagram. (0.5 points)

#8 We wish to design a decoder, with three inputs, x, y, z, and eight active high outputs, labeled 0, 1, 2, 3, 4, 5, 6, 7. There is no enable input required. (For example, if xyz=011, then output 3 would be 1 and all other outputs would be 0.) The only building block is a two-input, four-output decoder (with an active high enable), the truth table for which is shown below.

Draw a block diagram of the system using as many of these building blocks as are needed.

4. (0.5 points). The answer is in the back of the book, but I strongly encourage you to ensure you can solve the problem before you look.

#9. We want to implement a full adder; we’ll call the inputs a, b, and c and the outputs s and cout. As always, the adder is described by the following equations:

s(a, b, c) =∑m(1, 2, 4, 7)

couta, b, c=∑m(3, 5, 6, 7)

To implement this, all we have available are two decoders (as shown below) and two OR gates. Inputs a and b are available both uncomplemented and complemented; c is available only uncomplemented. Show a block diagram for this system. Be sure to label all of the inputs to the decoders.

5. You must use two-way in multiplexers, as the problem indicates, to get credit. (0.5 points)

Implement the function fx, y, z=∑m(0, 1, 3, 4, 7) using two-way multiplexers.

#6 Complete i, ii, and iii for subproblems a and b. (0.6 points)

15. For the following sets of functions, design a system

i. Using a ROM

ii. Using a PLA with the number of product terms shown

iii. Using a PAL

a. F(A, B, C) =∑m(3, 4, 5, 7)

G(A, B, C)=∑m(1, 3, 5, 6, 7)

H(A, B, C)=∑m(1, 4, 5) (4 product terms)

b. W(A, B, C)=∑m(0, 1, 4)

X(A, B, C)=∑m(0, 3, 4, 7)

Y(A, B, C)=∑m(1, 2, 6)

Z(A, B, C)=∑m(2, 3, 6, 7) (4 product terms)