1. [20 pts] [Basic Concept of “Electromagnetics] Please succinctly answer for the following questions:

(a) [4 pts] “What is “Electromagnetics”?

The study of the effects of electric charges at rest and in motion

(b) [4 pts] What are the four fundamental SI unit in the electromagnetics?

Meter, Kilogram, Second, Ampere

(c) [6 pts] What are the four fundamental field quantities in the electromagnetics model? and write their units

E: Electric Field Intensity		V/m

D: Electric Flux Density (per unit area)	C/m^2

H: Magnetic Field Intensity		A/m

B: Magnetic Flux Density		Tesla

(d) [6 pts] What are the three-universal constants in the electromagnetic model? and find the permittivity in free space, 𝜖0, of material from the physical relationship between 𝑐 and magnetic permeability in free space, 𝜇0

C= 3*108 m/sec

μ0=BH=4π *10-7 H/m

ϵ0=136π *10-9 F/m

c=1ϵ0*μ0→ϵ0=1μ*c2→14π *10-7*3*1082 =136*109π=136π*10-9

2. [20 pts] [Understanding of Vector Presentation in terms of Orthogonal Rectangular Components] In rectangular coordinates, given points 𝑀(−1, 2, 1), 𝑁(3, −3, 0), and 𝑃(−2, −3, −4). Find:

(a) [5 pts] AMN, Magnitude of AMN, and Direction of Unit Vector, aMN

AMN=3--1, -3-2, 0-1=4, -5,-1=ax4+ay(-5)+az(-1)

AMN=42+-52+-12=42≅6.48

u=vv=aMN=4, -5,-142=442, -542,-142≅0.617,-0.772,-0.154=ax442+ay( -542)+az(-142)

(b) [5 pts] AMN + AMP

AMN=3--1, -3-2, 0-1=4, -5,-1

AMP=-2--1, -3-2, -4-1=-1, -5,-5

AMN + AMP=-1, -5,-5+4, -5,-1=3,-10,-6=ax3+ay(-10)+az(-6)

(c) [5 pts] AM, and AN

AM=-1, 2, 1

AN=3, -3, 0

AM=-12+22+12=6≅2.44948

AN=32+-32+02=18≅4.2426

(d) [5 pts] 5AP-2AN*(AMP+APN)

AP=-2, -3, -4

AN=3, -3, 0

5AP-2AN=5*-2, -3, -4-2*3, -3, 0=-10-6, -15--6, -20-0=-16,-9,-20

5AP-2AN=-162+-92+-202=737≅27.1477

AMP=-2--1, -3-2, -4-1=-1, -5,-5

APN=3--2, -3-(-3), 0-(-4)=5, 0,4

AMP+APN=-1, -5,-5+5, 0,4=4,-5,-1

5AP-2AN*AMP+APN=737*4,-5,-1≅108.59097,-135.73872,-27.14774=ax4737+ay-5737+az(-1737)

3. [30 pts] [Understanding of Vector Field and Dot and Cross Products] In Cartesian coordinates, vector A is directed from the origin to point 𝑃1 (2, 3, 3), and vector B is directed from 𝑃1 to point 𝑃2 (1, −2, 2). Find:

(a) [3 pts] vector A, its magnitude A, and unit vector a,

A =2,3,3

A =(2)2+32+32=22≅4.69

u=vv=a=2,3,322=222, 322,322≅0.4264, 0.6396, 0.6396=ax(222)+ay(322)+az(322)

(b) [4 pts] the angle that A makes with the y-axis,

θ=cos-1A⋅YA*Y

Create a vector parallel to the y-axis

Y=0,1,0

A=(2)2+32+32=22≅4.69

Y=1

A⋅Y=0*2+1*3+0*3=3

θ=cos-131*22=50.2378°

(c) [3 pts] vector B

B=1-2, -2-3,(2-3)=-1, -5,-1=ax(-1)+ay(-5)+az(-1)

(d) [4 pts] the angle between A  and B

θ=cos-1A⋅BA*B

Create a vector parallel to the y-axis

B=-1, -5,-1

A=(2)2+32+32=22≅4.69

B=(-1)2+-52+-12=27≅5.196

A⋅B=-1*2+-5*3+-1*3=-20

θ=cos-1-2027*22 =145.145898°

(e) [4 pts] the perpendicular distance from the origin to vector B

θ=cos-1V⋅BV*B=90°→V⋅BV*B=0

B=-1, -5,-1

The point where the perpendicular line will intersect vector B is modeled by this equation:

PX=P1+B*x, where x is the distance the point is from P1

PX=2-1*x,3-5*x,3-1*x

The perpendicular Vector will be from PX to the origin, so it is modeled by the following formula

V=0-2-x,0-3-5x,0-3-x=x-2, 5x-3,x-3

V=x-22+5x-32+x-32

B=-12+-52+-12=27

V⋅B=(-1)*x-2+-5*5x-3+-1*(x-3)

Solve for x:

-1*x-2+-5*5x-3+-1*x-3x-22+5x-32+x-32+27=0→

-1*x-2+-5*5x-3+-1*x-3=0→

2-x+15-25x+3-x=0→20-27x=0→x=2027

Input x back into Vector V to find the magnitude

V=x-2, 5x-3,x-3→2027-2, 5*2027-3,2027-3

V=2027-22+5*2027-32+2027-32=5829≅2.6805

Book Solution:

V=A*sin180°-θAB→22*sin180°-145.145898°≅2.68

(f) [6 pts] A ∙ B, and B ∙ A. Compare the calculation results are the same or not.

A =2,3,3

B=-1, -5,-1

A⋅B=-1*2+-5*3+-1*3=-20

B⋅A=2*-1+3*-5+3*-1=-20

They are the same.

(g) [6 pts] A × B and B × A

A =2,3,3

B=-1, -5,-1

A × B= 3*-1-3*-5,3*-1-2*(-1),2*-5-3*(-1) →12,-1,-7,=ax(12)+ay(-1)+az(-7)

B × A=-5*3—1*3,-1*2—1*3,-1*3—5*2→-12,1,7=

ax(-12)+ay(1)+az(7)

They are NOT the same

4. [10 pts] [Understanding of Basic Law of Cross Vector Products] Given A=x-y+z2=, B=y+z, and C=-x2 +z3. Find (A × B) × C and compare with A × (B × C).

A =1,-1,2

B=0, 1,1

C=-2, 0,3 -1, 0,1

A × B= -1*1-2*1,2*0-1*1,1*1-0*(-1) →-3,-1,1

(A × B) × C= -1*3-1*0,1*(-2)-(-3)*3,(-3)*0-(-1)*(-2) →-3,7,-2=ax(-3)+ay(7)+az(-2)

B × C= 1*3-1*0,1*-2-0*3,0*0-1*(-2) →3,-2,2

A × (B × C)= -1*2-2*-2,2*3-1*2,1*-2-(-1*3) →2,4,1=

ax(2)+ay(4)+az(1)

5. [20 pts] (HARD PROBLEM) [Understanding of Two Vector Positions Based on the Geometrical Properties of Dot and Cross Products] Given A=ax5-ay2+az, find the expression of a unit vector B in the case under below situations

(a) B∥A

Because B is parallel to A, we can say B = A*x

B=x*5,-2,1=5x, -2x, x

B=1;

5x2+-2x2+x2=1

x=±130

B=530,-230,130 =ax(530)+ay(-230)+az(130)

(b) B⊥A, if B  lies in the 𝑥y-plane

Because B lies in the xy-plane, we can create its vector as such: x,y,0

Because B is perpendicular to A, we know that A⋅B=0

We also know that B=1

5,-2,1⋅x,y,0=0→5x-2y=0

x2+y2=1→x2+y2=1

Solve system of equations:

5x-2y=0→5x=2y→25x2=4y2→25x2-4y2=0

To simplify the system of equations, set x2=a, y2=b

(25a-4b=0)(a+b=1)→a=1-b→251-b-4b=0→b=2529, and a=1-2529=429

x2=429,thus x=429, y2=2529,thus y=2529

Now, recall that Vector B is equal to x,y,0, and thus, substituting the previous results into the vector, we find that the result is 429,2529,0=ax(429)+ay(2529)+az(0)