ELCT 363: Introduction to Microelectronics	 Homework #1

University of South Carolina/ Spring 2022	Due by: 01/26/2022

Page1/3

Student’s Name: Jacob Vaught	Total Points = 16

PROBLEM No 1: (4+2 =6 point)

- Identify metals, semiconductors, and insulators among the materials given below (0.5 point each  total 4 points):

| Ag | Metal | Au | Metal |
| --- | --- | --- | --- |
| Ge | Semiconductor | SiO2 | Insulator |
| Si | Semiconductor | GaAs | Semiconductor |
| Ni | Metal | Al2O3 | Insulator |

- Fill in the Blanks (0.5 point each  total 2 points)

  - Conductivity of a material depends on: Free electron concentration__ and _Mobility of Electrons___________.

  - At very low temperature (~0 Kelvin) semiconductor behaves as

_an insulator______.

  - _photons and high voltage_________can break the bonds in semiconductor crystals and create free electrons.

  - Mobility and Sheet resistance in semiconductor are measured by

_vand der pauw and hall methods ____

Problem No. 2: (2 points)

A device is powered by a 75 V supply with 0.15 A of current.  If the output power of the device is 4.25 W, how much power is wasted in heat?

P=V*I

Total power consumption: 75V*0.15A=11.25W

Total power wasted: 11.25W-4.25W=7W

ELCT 363/ Fall 2021	 Homework #1 	Page 2/3

Student’s Initials: ____J. C. V.___________________________________________

Problem No. 3 (2 points)

What is the force on an electron in an electric field of 2200 N/C? (2 points)

F=q*E, where q is the charge, and E is the Electric field

F=2200 NC*1.6*10-19C=3.52*10-16N

Problem No. 4 (2points)

Calculate conductivity (due to free electrons) of an intrinsic (pure) GaAs semiconductor sample at 300K.  Assume that free electron concentration is 2.5x 106 cm-3 and electron mobility is 8200 cm2/V-s

σ=q*n*μ,n is free electron concentration, and μ is mobility of electrons

σ=1.6*10-19*2.5*106*8200=3.28*10-9seimencm=328*10-9seimen/m

Problem No. 5 (2+1+0.5+0.5 = 4 points)

A silicon slab of 1 cm length with an electron concentration of 3.5x1016 cm-3 and thickness of 0.07 cm and a cross-section area of 0.19 cm2 has an electron mobility of 1100 cm2 /V-sec. Calculate resistivity, sheet resistance, conductivity, and resistance.

Conductivity=σ=q*n*μ

σ=1.6*10-19C*3.5*1016cm-3*1100cm2V*sec=6.16seimencm=616seimenm

Resistivity=ρ=1σ

ρ=1616=0.001623 Ohm*m

Resistance=R=ρ*LA

R=0.001623 Ohm*m*0.01 m0.000019m2=0.8542 Ohm

Rsh=ρt

Rsh=0.001623 Ohm*m0.0007m=2.31857Ω☐