Problem 1: (1+1+1+1+1+1 = 6 points)

What are differences between bipolar junction transistors and field effect transistors, Describe/sketch HEFET, MOSFET, MISFET, MOSHFET, MISHFET.

BJTs require a biasing current for operation, while FETs are Voltage controlled.

BJTs have a smaller input impedance, thus they draw more power, whereas FETs have a greater input impedance, thus drawing less power.

BJTs also offer greater gain than FETs.

FETs are much smaller than BJTs, thus are used more commonly in circuits today.

BJTs are more Temperature Sensitive

See attached PowerPoint for Diagrams…

Problem 2: (3+3 = 6 points)

Define High electron mobility transistor and describe how 2DEG can make this transistor faster than other transistors. Write it in detail.

HEMT is a transistor that operates at higher frequencies, typically between 300 MHz-300GHz

HEMT utilize the 2-dimensional electron gas(2DEG) created by a junction between 2 material with different bandgaps, called a heterojunction.

Usually, to create a heterojunction, A highly doped N-Type Conductor and an undoped material are utilized.

2DEG increases electron mobility, thus increasing electron velocity, which increases the switching speed.

Problem 3: (3point each = 6 points)

What would be the ideal wavelength which GaN and GaAs Light Emitting Diodes (LEDs) can produce?

Problem 4: (3 point each = 6 points)

What are the differences between LEDs and Laser diodes?

Problem 5: (3+3 = 6 points)

In LED what are the three different efficiency components, define reach clearly. Calculate the internal quantum efficiency of a LED which has overall efficiency of 30% with injection and extraction efficiencies of 70% and 50%.