- Question 1(X Points)

The figure shows a turbine operating at a steady state that provides power to an air compressor and an electric generator. Air enters the turbine with a volumetric flow rate of 2 m3/s at 700oK, 9.0 bar and exits the turbine at 300oK, 1 bar. The turbine provides power of 1000 kW to the compressor and 2000 kW to the generator. Air can be modeled as an ideal gas and KE and PE are negligible.

- Determine specific volume at the input to the turbine.

- Determine the mass flow rate of the air, in kg/s.

- Determine work output of the turbine

- Determine Enthalpy at State 1 and 2 from Table A-22

- Determine the rate of heat transfer in the turbine, in kW.

v1=RT1p1=8.31428.97kJkg⋅K1kN⋅m1 kJ(700 K)(9bar)100kN/m21bar=0.2232 m3/kg

m=(AV)1v1=2 m3/s0.2232 m3/kg=8.96 kg/s

Wnet=Wc+WEG=1000 kW+2000 kW=3000 kW

h1=h700 K=713.27 kJ/kg, and h2=h300 K=300.19 kJ/kg

Qcv=Wcv+mh2-h1=3000 kW+8.96kgs300.19kJkg-713.27kJkg1 kW1 kJ/s=-700.197 kW

- Question 2(X Points)

Steam enters the condenser of a vapor power plant at 0.2 bar with a quality of 0.90 and condensate exits at 0.2 bar and 40°C. Cooling water enters the condenser in a separate stream as a liquid at 4°C and exits as a liquid at 30°C with no change in pressure. W, KE and PE are negligible.

- Determine the ratio of the mass flow rate of the cooling water to the mass flow rate of the condensing steam.

  - Determine h1 with quality from Table A-3

  - Determine h2, h3, h4 from Table A-2

  - Determine ratio of mass flow rate

- The energy transfer (enthalpy change) from the condensing steam to the cooling water, in kJ per kg of steam passing through the condenser.

- 0=m1h1-h2-m3h3-h4→m3m1=h1-h2h4-h3

- hf= 251.4 kJ/kg and hg = 2609.7 kJ/kg,

- h1=251.4+0.92609.7-251.4=2373.87kJkg

- h2=167.57;h3=16.78; h4=125.79

- m3m1=2373.87-167.57125.79-16.78=20.24

- Qcv+Wcv=m1h2-h1+V12-V222+gz1-z2

- Qcv=m1h2-h1

- Qcvm1=h2-h1=125.79-2373.87=-2248.08kJkg

- Question 3(X Points)

Steam flows steadily through an adiabatic turbine. The inlet conditions of the steam are 4 MPa pressure, 500 ˚C temperature, and 80 m/s velocity and the exit conditions are 50 kPa, 90% quality, and 50 m/s. The mass flow rate of the steam is 12 kg/s. Determine:

- Change in KE.

- Enthalpy at inlet & exit from Table A-4 & A-3

- Power Output

- Determine specific volume at inlet from Table A-4

- Turbine inlet area

KE=mV12-V222=12*802-5022=23400 J=23.4 kJ

- h1=3445.3 kJkg

- hf=340.49;hg=2645.9;h2=340.49+0.902645.9-340.49=2415.359kJkg

- W=mh1-h2+V12-V22211000

- W=123445.3-2645.9+23.4=9616.2 kW

- v=0.08643m3kg

v=A*Vm→A=m*vV=12*0.0864380=0.0129645m2

- Question 4(X Points)

Steam enters a converging–diverging nozzle operating at steady state with p1 = 140 [bar], T1 = 400°C, and a velocity of 10 [m/s]. The steam flows through the nozzle with negligible heat transfer and no significant change in potential energy. At the exit, p2 = 40 [bar], and the velocity is 700 [m/s]. The mass flow rate is 2 [kg/s].

- Determine h1 & h2 & v2 from Table A-4

- Determine the exit area of the nozzle, in [m2].

From Table A-4, h1=3001.9kJkg

0=h1-h2+V12-V222→

h2=3001.9kJkg+102-70022m2s2*1N1kg⋅ms2*1kJ103N⋅m=2756.95kJkg

From Table A-4, v2 =0.07341m3kg

A2=mv2V2→A2=2*0.07341700=20.9cm2