What type or types of electric motor would you specify…

a) To drive a load with large inertia

Motor with high Starting torques are more suitable for driving a large load/inertia, thus DC shunt, series, and compound wound motors and AC Single Phase Capacitor Start, and AC three phase Design C are eligible.

b) To minimize variation of speed with load variation

Motors with a horizontal torque curve is the most suitable for a consistent speed with load variation.

AC capacitor start, 3 phase design C, DC shunt wound and DC speed-controlled motors would be most suitable for the requirements above.

c) To maintain accurate constant speed regardless of load variations.

Only speed-controlled DC motors and AC synchronous motors will be suitable for constant speed regardless of load variations.

2) The torque-speed curve for a 1/8 HP permanent magnet (PM) DC motor can be modeled by N=-10T+3000. The rated speed for the fractional horsepower motor is 2500 rpm at a rated voltage of 130 V. Determine…

a) The rated torque in oz-in (ounces -inches—the U.S. industry standard for fractional hp motors)

Plugging in the rated rpm into the formula, we get that the rated torque is 50 oz-in

b) The no-load speed.

Plugging in a torque of 0 into the formula, we get that the rated speed is 3000 rpm

c) Plot the power-torque curve and find the maximum power that the motor can deliver.

Max. Power:  0.2231 hp

Max. Torque: 150 oz-in

MATLAB code:

clc;

clear;

close all;

conversionFactor=1008406;

T=0:0.01:300;

N=(-10*T.^2+3000*T)/conversionFactor;

plot(T,N)

xlabel('Torque(oz-in)')

ylabel('Power(hp)')

maxPowerTorqueValue=3000/20;

maxPower=(-10*maxPowerTorqueValue^2+3000*maxPowerTorqueValue)/conversionFactor;

disp(maxPower)