Department of Mechanical Engineering

EMCH 368

MECHATRONICS

Lab 05

Digital Alarm Project

Last Edited 06/23/24 by T. Russo

EMCH 368 Mechatronics Project 1 – Digital Alarm

So far this semester, you have learned to use your microcontroller to control a wide variety of components in diverse ways and now it is time to see what you can do on your own! It is up to you to create a digital alarm.

Objective:

Using the materials listed below to create a digital alarm that has a variable range of times. The alarm should have a visual representation using a 4-digit seven-segment display. As the time reaches zero the alarm goes off. Ensure to incorporate a buzzer that makes a sound every 3 seconds after the alarm goes off. To turn it off there should be a switch that will restart the passage of time.

- Potentiometer to adjust the time

- Uses a switch of your choosing, to start / stop the countdown, and reset it.

- Using the 4-digit 7-segment display, there should be a visible countdown of minutes and seconds.

- When the timer reaches zero a buzzer should start making sounds to alert the user the timer has ended.

Required Materials:

- 4-digit 7-Segment Display

- A tactile switch for user input (pushbutton, tilt switch, etc.)

- Potentiometer

- A buzzer to make sound

- ELEEGO microcontroller

Code Suggestions and Notes

Get as creative as you can working with this code but here are a few notes to get you on the right track. When the function delay() is in use, it freezes the current state for the length of the delay which means there can be no inputs or output when the delay() is in use. Meanwhile, the function millis(), keeps track of the time the ELEEGO has been running in milliseconds.

Additionally, recall that int is a 16-bit number meaning that it can only contain values between -32,768 and 32,767. Meanwhile, the long datatype holds a 32-bit number (-2,147,483,648 to 2,147,483,647) and an unsigned long can count to 4,294,967,295.

A valuable resource for this class is the ELEEGO Language Reference, found at https://www.ELEEGO.cc/reference/en/. This website gives an overview of the distinct functions for ELEEGO, how they work, and examples of their usage.

Electrical Suggestions and Notes

When working with electrical components it is always important, for the sake of safety and functionality, to keep track of power and ground. Make sure that whenever creating a circuit always think before you connect any wires because if you misplace any wires, you could injure yourself and destroy the circuit. Ground is also a major factor when designing a circuit, make sure that all your components have some path to ground. Reminder that even though the analog pins can give an output from 0 to 1023, you can also take a digital reading from them as well, 0 or 1. This is especially important since the 4-digit 7 segment display takes up all the usable digital pins

Design Suggestions and Notes

As engineers, designing is a major component of your career and should be considered as a skill to improve on. When designing make sure to have a clear understanding of what your requirements are, what constraints you have, and how you can improve your design. These aspects should always be kept in mind and should be kept track of. This will not only help improve your design but also help troubleshoot your design. You should always have a file of what components you used, how your circuit was designed, or even why you made a certain design-decision. Keeping track of this will also help write your lab report. You can use the information that you have collected throughout the project and directly reference it on your report without having to go redo that part of the lab.

Lab report

For this lab, a lab report is required. There is a rubric located at the bottom of this document explaining all the requirements. When writing your lab report make sure you properly incorporate all the required material listed in the rubric. It must be meticulously organized so that all the information can easily be located. Make sure that all the topics are clearly written out, so it is easy to understand the thought process you went through while you worked on this lab. A quick suggestion: try to document every component of this project as you are doing them so later you do not have any trouble referencing them when drafting the report. The Lab report should include the following:

- Introduction to this project

- A list of materials used (with pictures)

- A picture of the circuit (with appropriate caption)

- An explanation of the circuit containing procedures took to build the circuit

- A copy of the code and any serial monitor outputs if you have any

- And an explanation of the code

In-Class Circuit Creation Grading Rubric

| Criteria | Complete? |
| --- | --- |
| Seven Segment Display is operational |  |
| Includes tactile switches for user input to start/stop and reset the circuit |  |
| After the time has ended the alarm beeps |  |
| The Seven Segment Display shows minutes and seconds |  |
| The time can be adjusted between a certain range. |  |
| The alarm can be stopped and restarted. |  |

Written Report Grading Rubric

| Content | Score |
| --- | --- |
| Introduction The student provides an introductory paragraph explaining the premise of their product, the criteria it had to meet, and the potential applications of the product. | _____/5 |
| Materials Used The student lists all components used in their circuit, providing pictures and descriptions. The student explains why they might have used one component over another. | _____/5 |
| Picture of the Circuit The student provides a clear photo of their circuit. | _____/5 |
| Procedure for Building Circuit The student provides a detailed procedure for assembling the circuit. | _____/5 |
| Explanation of the Circuit The student explains how the circuit works (from an electrical aspect), starting from the 5V provided by the ELEEGO. (Do not explain the code here) | _____/5 |
| Screenshots of Sketch The student provides clear copies of all the code and relevant serial monitor screenshots (if any). | _____/5 |
| Explanation of Code The student provides a detailed discussion of their code such as describing their variables, explaining the logic of their for/while loops, and how the functions they use provide the output they want, etc. | _____/10 |
| Writing Style/Readability The student uses concise but detailed writing and proper grammar. The report lacks major and numerous typos. | _____/5 |
| Organization The student’s report is broken into distinct sections. There are headers/footers, page numbers, and a title page. There are bulleted or numbered lists only where appropriate. The report has an overall professional look. | _____/5 |
| Total | _____/50 |