Department of Mechanical Engineering

EMCH 368

MECHATRONICS

Lab 08

Servo and DC Motors

Last Edited: 6/24/2024 by C. Smith

A servo motor is a specialized type of motor equipped with a reduction gear system, enabling precise control over the angle of rotation, typically limited to a range of 0 to 180 degrees. The control mechanism for a servo motor involves Pulse Width Modulation (PWM) signals, usually at a frequency of 50 Hz, which corresponds to a 20 ms period.

The angle to which the servo motor rotates is determined by the duration of the "on-time" (pulse width) within each 20 ms cycle. Typically, this "on-time" can range from about 1 ms to 2 ms: a 1 ms pulse commands the servo to rotate to its 0-degree position, while a 2 ms pulse directs it to the 180-degree position. The actual range can vary slightly between different servo models, but it generally falls within this range. Intermediate pulse widths result in corresponding intermediate positions.

Inside the servo motor, a small microcontroller or microchip deciphers the PWM signal and translates the duration of the "on-time" into a target position angle. The actual position of the servo is then continuously monitored by a feedback mechanism, commonly a potentiometer attached to the rotor.

When a servo motor operates, it draws current from the power source. This current draw varies based on the servo's position, load, and movement speed. Initially, when the servo starts to move, it draws a higher current, which correlates with its stall torque—the maximum torque it can handle before it stops moving (or stalls). The higher the stall torque, the higher the potential stall current. The operating speed, given in seconds per degree of rotation (0.1s/60deg for ours), plays a role in current consumption. Faster movements require more immediate power, leading to spikes in the current draw.

The dead bandwidth, usually given in µs (10 µs for ours), indicates the precision of control signal interpretation. A smaller dead band width means the servo can respond to finer adjustments in the control signal.

To maintain the rotor at the desired angle, the servo often employs a Proportional-Integral-Derivative (PID) controller, which adjusts the motor's position based on a set of three parameters: the Proportional (P) component, which determines the response intensity to the current angle error, defined as the difference between the desired angle (as instructed by the PWM signal) and the actual angle (as reported by the potentiometer); the Integral (I) component, which integrates the error over time, providing a cumulative offset that aids in correcting consistent positional lag or bias; and the Derivative (D) component, which considers the rate of change of the angle error, effectively predicting future error and reducing the likelihood of overshooting the target.

A brushed DC motor efficiently converts electrical energy into mechanical energy via the interaction between electric currents and magnetic fields. This conversion process initiates when an electric current, denoted as 𝐼, passes through wire coils inside the motor, which are located in a magnetic field (𝐵) established by permanent magnets or electromagnetic coils. The principles of electromagnetic induction

combined with the Lorentz force, expressed as 𝐹 = 𝐼 × 𝐿 × 𝐵 (where 𝐿 is the length of wire in the magnetic field), dictate that the current induces a force perpendicular to both the magnetic field and the current's direction. This force propels the motor's shaft, or armature, to spin.

Central to a DC motor are its primary components: the armature (the rotor), the commutator—a rotating electrical switch—and brushes. In a brushed DC motor, the armature (rotor) contains wound coils that create a magnetic field when electric current passes through. This interacts with the stationary magnetic field from the motor's magnets, causing rotation. The commutator, attached to the armature, reverses the current's direction every half turn, ensuring continuous rotation by changing the magnetic poles of the armature. The brushes maintain electrical contact with the rotating commutator to supply the armature with current. Together, these components allow the motor to efficiently convert electrical energy into mechanical motion.

The motor's operational performance, specifically its speed (𝜔) and torque (𝜏), hinges on the

electrical current's magnitude and the magnetic field's intensity. The torque is directly influenced by the current, as per the equation ( 𝜏 = 𝐵 × 𝐼 × 𝐿 × 𝑟), where 𝑟 is the radius to the point of force

application on the armature. Conversely, the motor's speed is shaped by the applied voltage (𝑉)

and countered by the back Electromotive Force (EMF), 𝐸𝑏, following the formula 𝑉 = 𝐸𝑏 + 𝐼 × 𝑅, with R symbolizing the armature's resistance. The back EMF, which opposes the applied voltage and scales with the motor's speed, is detailed by 𝐸𝑏 = 𝑘 × 𝜔, where 𝑘 is a construction-dependent constant. This EMF serves as a natural regulator, curbing the motor's speed by diminishing the net voltage and, consequently, the current through the motor as its speed climbs.

The first image illustrates a the L293D H-Bridge Integrated Circuit (IC) in a Dual In-line Package (DIP). This format is prevalent in many electronic projects, including those involving motor control. The identification of pin 1 is critical for proper IC utilization, marked by a dimple or notch at one end of the package. From this point, the pins are numbered in a counterclockwise sequence around the IC, a standard practice ensuring that each pin connects appropriately according to its designated function.

The second image depicts a schematic of an H-bridge, a fundamental component for motor control. This setup is named after its schematic representation, which resembles the letter "H". It consists of four transistors (either MOSFET of BJT), labeled Q1, Q2, Q3, and Q4. The H-bridge enables control over a motor's direction by altering the current flow. This is achieved through the strategic activation and deactivation of these transistors. For instance, activating transistors Q1 and Q4 allows current to flow in one direction across the motor, causing it to spin in one direction. Conversely, activating Q2 and Q3 reverses the current's direction, and consequently, the motor's rotation. This method of control is essential for tasks requiring bidirectional motor operation, such as in robotics or automated machinery.

In the context of the L293D or L293DE IC, these principles are integrated into a single chip. This IC contains internal switches akin to the H-bridge's transistors, facilitating direct motor control through input signals. The IC simplifies the process by consolidating the complex switching mechanism within its architecture, thus reducing the need for external components and simplifying the circuit design.

Understanding the mechanics of the H-bridge and the IC's role within this configuration is crucial for effectively implementing motor control in your projects. By applying logic to the input pins of the IC, you can manipulate the internal switches, thereby controlling the motor's speed and direction with precision. This knowledge forms the foundation for developing more complex and dynamic systems in your electronic endeavors.

Exercise 1: Using a Dial to control a Servo Motor

Similar to how pulses were used for PWM in the color mixing lamp, servo motors expect several pulses that tell them what angle to move to. The pulses always come at the same time intervals, but the width can vary between 1000 and 2000 microseconds. While it is possible to write code to generate these pulses, the Arduino software comes with a library that allows you to easily control the motor. Because the servo only rotates 180 degrees, and your analog input goes from 0-1023, you’ll need to map these values like in your color fading lamp.

The Arduino community has built countless libraries that expand the functionality of the Arduino, and many of these libraries are easily accessible to the public. You will be importing the library that comes with a wealth of commands relating to the servo motor.

Your goal is to create a circuit with the following criteria:

- Create a circuit where the position of a servo motor is directly controlled by the position of a dial (potentiometer)

Materials:

- 1 set of long red and black jumper wires

- 1 potentiometer

- 1 servo motor

How to wire the servo motor:

- Connect the red wire of the servo motor to power and the brown wire to ground

- Wire the orange wire to a digital pin that has the ~ for PWM.

Coding the servo motor:

To use the servo library, you will first need to import it. This makes the additions from the library available to your sketch. Also, you will have to create a servo object. This is essentially naming your servo so that you can call upon it to perform commands later in your code.

In the setup, you must tell your Arduino what pin the servo is attached to using the servo’s object name and .attach(digital pin). In this instance, it is myServo.attach(9).

Finally, to control the servo motor, you will use .write() which sends the PWM pulse corresponding to the angle that is inputted.

Finishing the exercise:

Now that you know how the servo motor is wired and how to code it in the Arduino IDE, you can add a potentiometer, and code it into the system so that it controls the angle of the servo motor. Remember, the potentiometer outputs from 0 to 1023 but the servo motor can only be set from an angle of 0 to 179.

Exercise 2: Dynamic Motor Control Circuit

Motors can control speed and direction beyond simple on/off states. Changing power and ground orientation reverses rotation, but this is impractical for frequent changes. Instead, we will use an H-bridge, an integrated circuit that simplifies direction control, functioning like multiple transistors.

Your goal is to create a circuit that meets the following criteria:

- Features a motor that has a stop/start button that only requires a momentary press to turn on/off

- A second pushbutton can be pressed to change the direction the motor spins

- A potentiometer can be used to adjust the speed of the motor

For this exercise, you will be given how to wire up the H-Bridge and a brief idea of coding the motor to work with the H-Bridge. Using the 2 buttons and potentiometer is in your hands to add on.

Materials:

- 1 set of long red and black jumper wires

- 2 switches (pushbutton)

- 1 potentiometer

- 1 H bridge ***(Make sure to use L293D, there are 2 in the kits)***

- 1 DC motor

Building the H-Bridge:

- Remove USB power before tampering with the circuit

- Remove all components besides the power and ground supply.

- Place the H bridge on your breadboard so it straddles the center. Then:

  - Connect pin 1 of the H bridge to digital pin 9 of the Arduino. This is the enable pin. When it receives 5V, the motor turns on. When it receives 0V, the motor turns off. By using PWM on the H bridge, you can adjust the speed of the motor.

  - Connect pin 2 of the H bridge to digital pin 3 of the Arduino

  - Connect pin 7 of the H bridge to digital pin 2 of the Arduino

    - Arduino pins 2 and 3 will tell the motor which direction to spin. When 3 is LOW and 2 is HIGH, the motor will spin one way and vice versa.

  - The H bridge gets its power from pin 16. Connect pin 16 to 5V

  - Attach your motor to pins 3 and 6 on the H bridge. The H bridge will use this pin to supply power to the motor

  - Connect pin 8 of the H bridge to the power (+) bus.

  - Ground pins 4 and 5 on the H bridge.

H-Bridge Circuit

Understanding the programming of the motor:

Create constants for output pins of the H-Bridge.

In the void setup(), we set the control pins and onOff pin as outputs.

In the void loop(), we start by writing the speed to the onOff pin. In this scenario the speed is set to 250. The max speed of the motor is 256 and when it is set to 0, the motor is not running. After that, for demonstration purposes, the first control pin is turned on while the other is off. After a second, it switches with the first control pin off and the second on. When you plug it all in and check it, you will notice that the motor will alternate between forward and reverse.  So far, this section has shown us how to run the motor at a specific speed and direction using the H-Brigde. This leads you onto the main part of this exercise.

Finish it off:

Now that you understand how to wire the H-Bridge with the motor and code the H-Bridge as well, it is time you integrate the other components of this exercise. You must add a potentiometer which will control the speed of the motor. There must be a button that will start / stop the motor from running. Finally, there needs to be a button for switching directions of the motor. You should be able to easily integrate these components into the microcontroller as well as code them into your program. Good Luck!

Lab 8 Code Appendix (in order of appearance):

#include – used to import outside libraries. Gives the programmer access to a large group of standard C libraries and libraries made specifically for Arduino

servo.attach(pin#) – attaches the Servo variable to a pin

servo.write(angle) – Sets the angular position of the shaft, moving the shaft in that orientation

Lab Grading Rubric:

- Exercise 1 functioning (15 points)

- Exercise 2 fully functioning (25 points)

- Comprehension Questions (10 Points)