Department of Mechanical Engineering

EMCH 368

MECHATRONICS

Lab 12 – Toy Car Simulator Project

Last Edited: 07/11/2024 by Tyler Russo

EMCH 368 Mechatronics Project 2 – Toy Car Simulation Project

Objective:

The goal of this project is to simulate toy car functions. It will use a motor to simulate the car moving forward, reverse, start/stop, and speed. A servo motor and LED’s will be used for indicating turning and turn signal. Finally, you will have two options to choose from to integrate. Either an ultrasonic sensor can be integrated that will act as a preventative crash brake if it detects an object within a certain distance or a joystick can be used to control both the motor and the turning.  All these functionalities and values must be reported to the Serial monitor to verify that it is working in addition to the visual confirmation of the circuit. (Note: This will also be very useful for testing and troubleshooting your circuit).

- DC motor to indicate forward, reverse, start, stop, and speed

- Servo to indicate steering

- LED turn indicators for Left and Right turns

- Either:

  - Ultrasonic sensor to brake when too close to an object, with a piezo to alert the “Driver”

  - Joystick for controlling both the DC motor and turning.

- Serial monitor to display current state of the car (forward, reverse, turn angle, turning, start/stop, etc.)

Required Materials:

- Drive Motor (DC motor)

- Turn Signal (LEDs)

- Steering Motor (Servo)

- Preventative Crash Brake (Ultrasonic Sensor) or Remote Control (Joystick)

Code Suggestions and Notes:

In this project, the Serial monitor will be required to demonstrate some of the functionalities, but it is also a great resource for testing and troubleshooting. You can always use it to display checks for loops, if statements, and other changing variables. You will also want to keep track of your code or keep multiple versions and document what changes and additions you make, not only for the lab report, but also as you move forward with the code and troubleshoot. This way if you change or add something and a certain functionality stops working, it will be much easier to identify and resolve the issue. Leaving comments in your code and breaking apart different sections will be a great help in homing in on trouble spots, or helping you remember why and what each of these sections are intended for. This will help you when you write your lab report and when you are explaining your code and the decisions you made when being checked off in class.

Electrical Suggestions and Notes:

When working with electrical components it is always important, for the sake of safety and functionality, to keep track of power and ground. Make sure that whenever creating a circuit, you always think before you connect any wires: if you misplace any wires, you could injure yourself and destroy the circuit! Make sure you are not shorting any components such as the LEDs which will get extremely hot. Ground is also a major factor when designing a circuit, make sure that all your components have some path to ground. Also, keep in mind to use some sort of resistance to limit the current input into the LEDs or else they will become hot and possibly hurt you. Keep in mind as well the additional risks and precautions necessary when working with the two different power sources (9V and 5V). This amplifies the precautions as it is much easier to short or fry various components or your board while working with both voltages.

Design Suggestions and Notes:

The required materials list is more of a guidance and lists the items necessary to demonstrate the objectives; however, there will be other materials needed to make the fully functioning circuit. Make sure to use what you have learned through the past labs and reference those if you are unsure of what else you might need to make these items function. You will also want to keep track of your build and document what changes and additions you make, not only for the lab report, but also as you move forward with the build and troubleshoot. This way if you change or add something and a certain functionality stops working, it will be much easier to identify and resolve the issue, just like keeping multiple copies of your code as mentioned above.

Lab report:

For this lab, a lab report is required. There is a rubric located at the bottom of this document explaining all the requirements. When writing your lab report make sure to properly incorporate all the required material listed in the rubric. It must be properly organized so that all the information can easily be located. Make sure that all the topics are clearly written out, so it is easy to understand the thought process you went through while you worked on this lab. A quick suggestion: try to document every component of this project as you are doing them so later you do not have any trouble referencing them when writing the report. The Lab report should include the following:

- Introduction to this project

- A list of materials used

- A picture of the circuit (with appropriate caption)

- An explanation of the circuit containing procedures took to build the circuit

- A copy of the code and any serial monitor outputs if you have any

- And an explanation of the code

Additionally, a .txt file of your code will be required along with the lab report submission to complement the screenshots of your code in the actual report.

In-Class Circuit Creation Grading Rubric (50% of overall Project grade)

| Criteria | Complete? |
| --- | --- |
| DC motor drives forward, reverse, starts/stops and changes speed |  |
| Servo properly indicates steering |  |
| LEDs properly indicate a left or right turn signal when appropriately steering in those directions by lighting up |  |
| Joystick properly controls the car, or the ultrasonic sensor causes the car to brake when an object is within a certain distance |  |
| Serial monitor properly demonstrates all working functionalities and displays required data |  |
| Student provides complete code and can demonstrate comprehension and understanding of what they have written and why they might choose certain functionalities |  |

Written Report Grading Rubric (50% of Overall Project grade)

| Content | Score |
| --- | --- |
| Introduction The student provides an introductory paragraph explaining the premise of their product, the criteria it had to meet, and the potential applications of the product. | _____/5 |
| Materials Used The student lists all components used in their circuit, providing pictures and descriptions. The student explains why they might have used one component over another. | _____/5 |
| Picture of the Circuit The student provides at least one clear photo of their circuit. Be mindful of the wiring so that where each wire goes is visible | _____/5 |
| Procedure for Building Circuit The student provides a detailed procedure to assemble the circuit. | _____/5 |
| Explanation of the Circuit The student explains how the circuit works, starting from 5V provided by the Arduino. | _____/5 |
| Screenshots of Sketch The student provides clear copies of all the code and relevant serial monitor screenshots (if any). The student also attaches a .txt file of their Arduino code alongside the lab report submission. | _____/5 |
| Explanation of Code The student provides a detailed discussion of their code such as describing their variables, explaining the logic of them for/while loops, and how the functions they use provide the output they want, etc. | _____/10 |
| Writing Style/Readability The student uses concise but detailed writing and proper grammar. The report lacks major and numerous typos. | _____/5 |
| Organization The student’s report is broken into distinct sections. There are headers/footers, page numbers, and a title page. There are bulleted or numbered lists only where appropriate. The report has an overall professional look. | _____/5 |
| Total | _____/50 |