Department of Mechanical Engineering

EMCH 368

MECHATRONICS

Lab 12 - Car Project

Last Edited: 04/05/2024 by J. Vaught

EMCH 368 Mechatronics Project 2 – Car Project

Objective:

Your task is to build a model car that can move forward and reverse, steer with blinkers for indicating turns, and include a hard stop or brake mechanism. Here's a brief guide on how to accomplish this:

- Motion Control: To make your car move forward and reverse, use a DC motor controlled by two separate buttons - one for each direction and a potentiometer for speed. NOTE: When switching direction, the button should be pressed ONCE to change direction.

- Steering: Incorporate a servo and a potentiometer to achieve steering functionality.

- Turn Indicators: Install LEDs to serve as left and right turn signals. These will light up when the car is turning in their respective direction. It is up to you to determine how to implement this.

- Brake/Hard Stop: Equip your car with a feature that allows for a hard stop or braking.

- Feedback and Diagnostics: Use a Serial monitor to display and verify the car's current state (forward, reverse, turn angle, turning status, and brake/stop incidents).

Materials Needed:

- Drive Motor: DC motor

- Movement Control Buttons: Two buttons for controlling forward and reverse motions, potentiometer for speed

- Steering Mechanism: Servo motor

- Steering Wheel: Potentiometer or suitable alternative

- Turn Signals: LED lights

- Brake/Stop Input: A mechanism like a tilt switch, button, or similar for triggering a hard stop or brake.

Coding Recommendations and Guidelines:

- Serial Monitor: The Serial monitor is not just crucial for showcasing your project's functionalities, but it also serves as a powerful tool for testing and troubleshooting. You can use it to monitor loops, conditional statements, and variable changes in real time.

- Code Management: Ensure you maintain and track various versions of your code, documenting every change and addition you make. This practice is not only beneficial for your lab report but also vital for efficient troubleshooting. If any functionality breaks after a modification, you can easily identify and rectify the issue.

- Code Organization: Leave comments in your code and break it into distinct sections. This aids in pinpointing problem areas and helps you remember the purpose of each section. Such organization is especially helpful when writing your lab report or explaining your code during in-class presentations.

Electrical Precautions and Tips:

- Wire Connections: Always keep track of the power and ground while working with electrical components. Before connecting any wires, plan carefully to prevent any misplacement that could lead to injuries or circuit damage.

- Avoid Short Circuits: Be careful not to short any components, especially LEDs, as they can break. Ensure all your components have a path to the ground.

- Resistance: To avoid overheating LEDs, incorporate a resistor to limit the current input.

- Power Sources: Given the potential risks associated with the two different power sources (9V and 5V), you should be extra careful. Mismanagement of these voltages can easily lead to short circuits or damage to the components or the board.

Design Advice and Suggestions:

- Materials: The required materials list is a guideline, outlining the necessary items to meet the project objectives. However, to build a fully functioning circuit, you'll need additional materials. Refer to your past labs and Arduino book for guidance if you're unsure about what else you might need.

- Design Documentation: Just like code management, keep track of your design process and document any changes and additions. This is important not just for your lab report, but also for troubleshooting as you progress with your build. This way, if a modification disrupts any functionality, you can easily identify and fix the problem.

Lab Report Guidelines:

Your lab report should be comprehensive and well-structured, covering all aspects of the project.

- Introduction (5 points): Provide an opening paragraph detailing the concept of your product, the criteria it needed to meet, and its potential applications.

- Materials Used (5 points): Enumerate all the components used in your circuit, supplementing with pictures and descriptions. Also, justify your choice of certain components over others.

- Picture of the Circuit (5 points): Include a clear photograph of your completed circuit.

- Procedure for Building Circuit (5 points): Detail the step-by-step process you followed to assemble the circuit.

- Explanation of the Circuit (5 points): Explain the working mechanism of your circuit, starting from the 5V provided by the Arduino.

- Screenshots of Sketch (5 points): Provide clear copies of your code and relevant Serial monitor outputs, if any.

- Explanation of Code (10 points): Discuss your code in detail, describing variables, explaining the logic behind any loops (for/while), and how the functions used generate the desired output.

- Writing Style/Readability (5 points): Ensure your report is written in a concise yet detailed manner, using proper grammar. The report should be free of significant errors and typos.

- Organization (5 points): Your report should be divided into distinct sections with headers/footers, page numbers, and a title page. Use bulleted or numbered lists where appropriate, and maintain an overall professional look.

In-Lab Circuit Rubric (50% of Project 2 Grade)

| Criteria | Grade |
| --- | --- |
| The DC motor should successfully propel the car forward and in reverse. | _____/10 |
| The servo should accurately control the steering mechanism. | _____/8 |
| The LEDs should correctly indicate left or right turns by illuminating during the corresponding steering actions. | _____/8 |
| The car should be able to execute a hard stop or brake in case of an emergency. | _____/8 |
| The Serial monitor should correctly demonstrate all car functionalities and display all required data. | _____/8 |
| The student should provide a complete code, demonstrate understanding of the written code, and justify their choice of functionalities. | _____/8 |
| Total | _____/50 |

Report Rubric (50% of Project 2 Grade)

| Criteria | Grade |
| --- | --- |
| Introduction Introductory paragraph explaining the premise of the product, the criteria it had to meet, and the potential applications. | _____/5 |
| Materials Used Lists all components used in the circuit, providing pictures and descriptions. Explanations why each component was chosen over another. | _____/5 |
| Picture of the Circuit Clear photo of the entire working circuit. | _____/5 |
| Procedure for Building Circuit Detailed procedure to assemble the circuit. | _____/5 |
| Explanation of the Circuit Circuit explanation, starting with the 5V power provided from the Arduino. | _____/5 |
| Screenshots of Sketch Clear copies of all code and relevant serial monitor screenshots. | _____/5 |
| Explanation of Code Includes a detailed discussion of the code. Describes the variables, explains the logic of the for/while loops, and how each function provides the desired output, etc. | _____/10 |
| Writing Style/Readability Report utilizes concise, detailed writing with proper grammar. | _____/5 |
| Organization Report is broken into distinct sections with headers/footers, page numbers, and a title page. Bulleted or numbered lists are used only when necessary. The report has an overall professional look. | _____/5 |
| Total | _____/50 |