Problem 1: (1+1+1+1+1+1 = 6 points)

What are differences between bipolar junction transistors and field effect transistors, Describe/sketch HEFET, MOSFET, MISFET, MOSHFET, MISHFET.

BJTs require a biasing current for operation, while FETs are Voltage controlled.

BJTs have a smaller input impedance, thus they draw more power, whereas FETs have a greater input impedance, thus drawing less power.

BJTs also offer greater gain than FETs.

FETs are much smaller than BJTs, thus are used more commonly in circuits today.

BJTs are more Temperature Sensitive

See attached PowerPoint for Diagrams…

Problem 2: (3+3 = 6 points)

Define High electron mobility transistor and describe how 2DEG can make this transistor faster than other transistors. Write it in detail.

HEMT is a transistor that operates at higher frequencies, typically between 300 MHz-300GHz

HEMT utilize the 2-dimensional electron gas(2DEG) created by a junction between 2 materials with different bandgaps, called a heterojunction.

Usually, to create a heterojunction, A highly doped N-Type Conductor and an undoped material are utilized.

2DEG increases electron mobility, thus increasing electron velocity, which increases the switching speed.

Problem 3: (3point each = 6 points)

What would be the ideal wavelength which GaN and GaAs Light Emitting Diodes (LEDs) can produce?

GaN abdngap=3.4;wavelength=1240.83.4=364.9 nm

GaAs bandgap=1.42;Wavelength=1240.81.422=872.6 nm

Problem 4: (3 point each = 6 points)

What are the differences between LEDs and Laser diodes?

LEDs are based on the principle of electroluminescence, while LASERs are based on stimulated emission.

Lasers emit a monochromatic light beam, meaning that it is comprised of only 1 color, and the light beams are parallel, resulting in a single beam being emitted.

LEDs do not emit light in a single direction, so it is used in many consumer facing applications.

Problem 5: (3+3 = 6 points)

In LED what are the three different efficiency components, define each clearly. Calculate the internal quantum efficiency of a LED which has overall efficiency of 30% with injection and extraction efficiencies of 70% and 50%.

Injection Efficiency, Recombination/Quantum Efficiency, Extraction Efficiency

Injection Efficiency is the ratio of electrons passing through the device to the number of electrons injected into the active region.

Quantum Efficiency is the ratio of all the electron-hole recombinations to the number of electron-hole recombinations in the active region that are producing photons.

Extraction Efficiency is the ratio of photons generated in the active region to the number of photons that escape the device.

η0=ηin*ηq*ηe

η0=0.3*0.5*0.7=10.5%