2.18 Determine the values of the parameters {ω, ϕ,A} in the equations below.

(a) 9 cosωt + ϕ= Aej8t+jϕ+ Ae-j8t+j2π3

Using the inverse Euler Formula,  cosθ=ejθ+e-jθ2 the result can be determined.

Using simple algebra, we can see that Bcosθ=B2(ejθ+e-jθ), thus A=92=4.5

If we compare θ between all terms, we can see that θ=ωt+ϕ, and θ=--8t+2π3

With this information, we can see that ω=8, and ϕ=2π3

(b) 10 cos9t -π3= A cosωt -π2+ 5 cosωt + ϕ

From cosα-β=cosαcosβ+sinαsinβ:

10 cos9t -π3= 10cos9tcosπ3+sin9tsinπ3→10cos9t*0.5 +sin9t*32→

5cos9t+53sin9t

From sinθ=cosθ-π2,

5cos9t+53sin9t→5cos9t+53cos9t-π2

53cos9t-π2+5cos9t=A cosωt -π2+ 5 cosωt + ϕ

Comparing info from both sides, we get:

ω=9, ϕ=0, A=53

2.23 Complex exponentials obey a simple rule for the operation of time shifting: the complex amplitude experiences a phase change. Consider the complex signal z(t) = Zej10πt where Z = 7ej0.3π.

(a) The time-shifted version of z(t) can be represented as a new complex exponential with complex amplitude W. Thus, z(t – td)=w(t)=Wej10πt. Determine the complex amplitude W when td=0.24s.

(b) Suppose that y(t) is a different time-shifted version of z(t), and its complex amplitude is Y = 7e-j0.1π . Determine the smallest positive value for the time shift td such that y(t)=z(t–td).

zt= Zej10πt=7ej0.3πej10πt=7ej10πt+0.3π

Substituting (t-0.24) for t in the expression above: zt-td=7ej10π(t-0.24)+0.3π

Since z(t – td)=Wej10πt, we can substitute the above expression for zt-td

7ej10π(t-0.24)+0.3π=Wej10πt

Using a calculator, we can see that W=7e-j2.1π

Using the Expression for zt-td above, we can substitute t=0 to find the smallest positive value for the time shift.

zt-td=7ej10πt-td+0.3π=7ej10π0-td+0.3π

Y=zt-td→7e-j0.1π=7ej10π-td+0.3π→

-0.1π=10π(-td)+0.3π→td=0.04s

3.2 A signal composed of sinusoids is given by the equation

xt= 10 cos800πt +π4 + 7 cos1200πt -π3 - 3 cos1600πt

According to Euler’s Identity, cosπt=ejπt+e-jπt2

(a) Sketch the spectrum of this signal, indicating the complex amplitude of each frequency component. Make separate plots for real/imaginary or magnitude/phase of the complex amplitudes at each frequency.

Converting xt= 10 cos800πt +π4 + 7 cos1200πt -π3 – 3 cos1600πt into exponential form, we get:

xt=10*ej800πt +π4+e-j800πt +π42+7*ej1200πt -π3+e-j1200πt -π32 -3*ej1600πt+e-j1600πt2→

5ej400*2πt +π4+e-j400*2πt +π4+3.5ej600*2πt -π3+e-j600*2πt -π3-1.5ej800*2πt+e-j800*2πt

We can see that the frequencies are: 800π2π=400Hz,1200π2π=600Hz,1600π2π=800Hz

We can also see: 5e-jπ4,  3.5e-jπ3,  1.5

(b) Is x(t) periodic? If so, what is the period?

xt  is periodic with a frequency of 200 Hz, since 200 is the GCD of [400,600,800]

(c) Now consider a new signal defined as y(t) = x(t) + 5 cos1000πt +π2. How is the spectrum changed? Is y(t) periodic? If so, what is the period?

The spectrum has added points at 500, 2.5ejπ2,-500, 2.5e- jπ2

yt  is periodic with a frequency of 100 Hz, since 100 is the GCD of [400,500,600,800]

3.6 Shown in the Figure is a spectrum plot for the periodic signal x(t). The frequency axis has units of rad/s.

(a) Determine the fundamental frequency ω0 of this signal.

The fundamental frequency is 1.2π which is the GCD of 3.6π, 8.4π

(b) Determine the (fundamental) period T0 of x(t), which is the shortest possible period.

T=2πf=2π1.2π=1.666s

(c) Determine the DC value of this signal.

7ejπ=7cosπ+jsinπ=-7

(d) A periodic signal of this type can be represented as a Fourier series of the form

xt= k=-∞akejω0kt

If the Fourier series coefficients of x(t) are denoted by ak, k = 0, ±1, ±2, ±3,... , determine which coefficients are nonzero. List these nonzero Fourier series coefficients and their values in a table.

Coefficients for K=0,  3.6π1.2π=3,  8.4π1.2π=7

| k | -7 | -3 | 0 | 3 | 7 |
| --- | --- | --- | --- | --- | --- |
| ak | 3e- jπ4 | 4ejπ3 | 7ejπ | 4e- jπ3 | 3e jπ4 |

P-3.13 The signal x(t) in the figure consists of a DC component plus a cosine signal.

(a) Determine the frequency of the DC component, as well as the frequency of the cosine component.

The frequency of the DC component is 0.

The period is 25 ms, so10.025=40Hz

(b) Write an equation for the signal x(t). You should be able to determine numerical values for all the amplitudes, frequencies, and phases in your equation by inspection of the above graph.

The amplitude is 15, the DC component is 5V, and the cosine is delayed by 5 ms, so

xt=5+15cos2π*40t-0.005=5+15cos80πt-0.4π

(c) Expand the equation obtained in the previous part into a sum of complex exponential signals with positive and negative frequency components.

xt=5+7.5ej80πt-0.4π+7.5e-j80πt-0.4π

(d) Plot the two-sided spectrum of the signal x(t). Label the complex amplitudes for each positive and negative frequency component in the spectrum.

Last MATLAB Problem.

MATLAB CODE

clc;

clear;

t=0.00005;

n=10000;

A=3;

fc=16;

f1=2;

x=zeros(1,n);

y=zeros(1,n);

for k=1:n

x(k) = k*t;

y(k) = A*cos(2*pi*fc*x(k))*sin(2*pi*f1*x(k));

end

plot(x, y)

P = rms(x)^2 %=0.0833

MATLAB Code for plots in other problems

clc; clear; close all;

fs = 100;%range of frequency, 1000=[-500, 500]

f = 400; %Hz

t = 0:1/fs:1-1/fs;

%2nd problem

signal = 5+15*cos(80*pi*t-0.4*pi);

%1st problem

%signal=10*cos(800*pi*t+pi/4)+7*cos(1200*pi*t-pi/3)-3*cos(1600*pi*t);

fftSignal = fftshift(fft(signal));

f = fs/2*linspace(-1,1,fs);

figure;

plot(f, abs(fftSignal));

title('magnitude FFT');

xlabel('Frequency (Hz)');

ylabel('magnitude');