EMCH 290-001-Quiz #3                                                               Thursday, September 29th, 2022 Chapter #3

Points Possible: 100

If unit conversion is unclear, ask TA/professor.  Box in ALL answers!

Question 1. (15 points)

The quality of a two-phase liquid–vapor mixture of H2O at 40°C with a specific volume of 10 m3/kg is? Use Table A-2, Eq(s). 3.7

- 0

- 0.481

- 0.519

- 1

v=vf+xvg-vf → x=v-vfvg-vf

From Table A-2, vf=0.0010078, vg=19.258

x=v-vfvg-vf=10-0.001007819.258-0.0010078=0.5192

Question 2. (15 points)

Given the following table, interpolate the value of y if x = 5.5. Use Eq(s). 3.6

| Y | X |
| --- | --- |
| 1 | 2 |
| 10 | 8 |

y=y2-y1x-x1x2-x1+y1→10-15.5-28-2+1=6.25

Question 3. (10 points)

An automobile’s radiator cap is labeled “Never open when hot.” Why not?

Because of Pressure buildup due to high temperature, and when opened causes hot water to splash onto your body.

Question 4. (20 points)

Water contained in a closed, rigid tank, initially saturated vapor at 45°C, is cooled to 20°C.

Use Table A-2, Eq(s). 3.7

- Determine the initial pressure, in bar.

- Determine the initial specific volume

- Determine the final specific volume

- Determine the final quality.

From Table A-2(Saturated Water: Temperature Table) @ 45°C,  p=0.09593 bar

From Table A-2, vfinal=vinitial=vgas, 45°C=15.258m3kg

From Table A-2, vfluid,20°C=1.0018*10-3m3kg, vgas,20°C=57.791m3kg

Quality:  xfinal=v-vfvg-vf=vfinal-vf,100°Cvg, 100°C-vf, 100°C=15.258-1.0018*10-357.791-1.0018*10-3=0.264

Question 5. (20 points)

A closed, rigid tank contains 2.50 kg of water, initially a two-phase liquid–vapor mixture at T1 = 10°C. Heat transfer occurs until the tank contains only saturated vapor at T2 = 45oC.

Use Table A-2, Eq(s). 3.7, 3.8

- Determine the initial pressure, p1, in kPa.

- Determine the initial quality

- Determine the initial specific internal energy in kJ/kg

- Determine the initial internal Energy in kJ

Referring to Table A-2, p1=0.01228bar=1.228 kPa

Referring to Table A-2, v1=v2=vg,45=15.258m3kg, u2=2436.8kJkg

Referring to Table A-2, v1g=106.379,v1f=1.0004*10-3, u1f=42, u1g=2389.2

x=15.258-1.0004*10-3106.379-1.0004*10-3=0.1434225

v=vf+xvg-vf=42+0.14342252389.2-42=378.64kJkg

U1=m*u1=378.64*2.5=946.6 kJ

Question 6. (20 points)

Determine the specific volume (ʋ), in ft3/lbm, of Sulfur dioxide (SO2) at 51.7625 atm and 813.42oR, using the attached Generalized Compressibility Chart.

(Please mark your work on attached chart to identify Z).  Eq(s). 3.1, 3.2, 3.3, 3.4, 3.5

Determine TR and pR first:

TR=813.42775=1.05;pR=51.762577.7=0.666

Z=pvRT→0.78=761lbfin2*144in2ft2*v813.42*154564.06*ft. ⋅lbflbmol.°R*lbmollb*°R

→v=0.78*813.42*154564.06*ft. ⋅lbflbmol.°R*lbmollb*°R761lbfin2*144in2ft2=0.139ft3lb

Equations:

- TR=TTc

- pR=ppc

- Z=pvRT

- R=RM

- R=1545ft. ⋅lbflb⋅mol.°R

- y=y2-y1x-x1x2-x1+y1

- v=vf+xvg-vf → x=v-vfvg-vf or u=uf+xug-uf → x=u-ufug-uf

- U=m*u

- 14.7 psi=1 atm