- Question 1-(8 points)

A power cycle operates between a lake’s surface water at a temperature of 25 oC and water at a depth whose temperature is 4oC. At steady state the cycle develops a power output of 12 kW, while rejecting energy by heat transfer to the lower temperature water at the rate 14,400 kJ/min.

- Determine the thermal efficiency of the power cycle.

- Determine the maximum thermal efficiency for any such power cycle.

- Is the power cycle possible or impossible?

The thermal efficiency is defined as:

η=Wcycle QH

Considering the energy balance for the cycle:

Wcycle =QH-QC

The thermal efficiency is:
η=WcycleQH=WcycleWcycle+QC=10 kW10 kW+14,400kJmin1 min60 s1 kW1 kJ/s=0.04
η=4%⟵ Thermal efficiency

The thermal efficiency for a reversible cycle is:

ηmax=1-TCTH=1-10+273K25+273K=0.050ηmax=5%

The power cycle is "possible". The thermal efficiency of a power cycle is always less than or equal to the maximum theoretical efficiency.

- Question 2-(8 points)

For the reversible cycle shown in the figure, p1 = 2.32 bar, v1 = 0.43 m3/kg, TH = 500 K, QH = 150 kJ, and the gas is air obeying the ideal gas model.

- Determine TC, in K.

- Determine the thermal efficiency of the cycle.

- Determine the net work of the cycle, in kJ.

Since Tc=T1, the ideal gas equation of state gives:

TC=T1=p1v1R=(2.0bar)105 N/m21bar0.43 m3/kg8.31428.97kJkg⋅K103 N⋅m1 kJTC=300 K

The thermal efficiency is defined as:

η=Wcycle QH=1-QCQH

For a reversible cycle:

QCQH=TCTH

So, the thermal efficiency is:

η=1-TCTH =1-300 K500 K=0.40η =40%

From the equation for thermal efficiency:

Wcycle =ηQH =(0.40)(150 kJ)Wcycle  =60 kJ

- Question 3-(8 points)

At steady state, a power cycle develops a power output of 10 kW while receiving energy by heat transfer at the rate of 10 kJ per cycle of operation from a source at temperature TH. The cycle rejects energy by heat transfer to cooling water at a lower temperature of 300 K. There are 145 cycles per minute. Determine the rate of heat rejection at the lower temperature, in kW and the minimum possible value for TH, in K. Determine the thermal efficiency of the cycle assuming the cycle is reversible.

- Determine the rate of heat rejection at the lower temperature, in kW.

- Determine the minimum possible value for TH, in K, for the given operating conditions.

- Determine the thermal efficiency of the cycle assuming the cycle is reversible.

From an energy balance on the system:

QC=QH-Wcycle =10kJcycle100cycles 1 min60 s1 kW1 kJ/s-10 kW=16.67 kW-10 kWQC=6.67 kW⟵ Energy rejection  to cooling water

The minimum theoretical temperature occurs when the cycle is reversible, and:

QCQH=TCTH,

Solving for the minimum temperature of the hot reservoir, TH, min  :

TH, =QHQCTC=16.67 kW6.67 kW(300 K)

TH, =750 K

Since the minimum theoretical temperature occurs when the cycle is reversible, the thermal efficiency is:

η=1-TcTh=1-300K750K

η=60%⟵ Thermal efficiency

- Question 4-(8 points)

A power cycle operating at steady state receives energy by heat transfer from the combustion of fuel at an average temperature of 400oC. Owing to environmental considerations, the cycle discharges energy by heat transfer to the atmosphere at 30oC at a rate no greater than 60 MW. Based on the cost of fuel, the cost to supply the input heat transfer is $4.50 per GJ. The power developed by the cycle is valued at $0.08 per kW·hr. The power plant operates for 8000 hours annually.

- Determine maximum thermal efficiency of the cycle.

- Determine the maximum value of the power generated, in $ per year.

- For the maximum power generated, determine the corresponding fuel cost, in $ per year.

The maximum power generated occurs with the maximum allowable energy rejection to the atmosphere as the cycle operates reversibly. The thermal efficiency of a reversible cycle is:

ηmax=WcycleQH=1-TCTH=1-303K673K=0.55

From an energy rate balance on the control volume at steady state:

QH=Wcycle+QC

Solving each equation for QH and setting them equal yields:

Wcycle+QC=Wcycle1-TC/TH

Solving for Wcycle:  :

Wcycle=QCTH/TC-1≤(60MW)673 K303 K-1Wcycle≤73.27MW

Therefore:

Wcycle,  =73.27MW

Considering the plant operates 8000 hours annually, with a value of $0.08/kW-h, the maximum value of power generated annually is:

$]Wcycle,  =$0.08kw⋅h8000h year 73.27MW103 kW1MW

$]Wcycle,max =$46.9 Million/year

Returning to the energy rate balance, the rate of heat addition for the maximum rate of work is:

QH,Wcycle, =Wcycle, +QC, =73.27MW+60MW=133.27MW

At a cost of $4.50/GJ, the annual cost of fuel is:

$]fuel,Wcycle,max =$4.50GJ(133.27MW)1GJ/s103MW8000h year 3600 s1 h$]fuel,wcycle,  =$17.27 Million/year ⟵ Corresponding annual  fuel cost

- Question 5-(8 points)

A reversible heat pump cycle operates at steady state between hot and cold reservoirs at TH = 30oC and TC = 10oC, respectively. The rate of heat transfer at the high temperature is 10 kW.

- Determine the net power input, in kW.

- Determine the heat transfer rate from the cold reservoir at TC, in kW.

- Determine the coefficient of performance of the cycle.

For an internally reversible heat pump cycle:

γ=γmax=QHWcycle=THTH-TC

Solving for the power input to the cycle:

Wcycle=QHTH-TCTH =(10 kW)30-10 K30+273 KWcycle  =0.66 kW

From the energy rate balance on the system at steady state:

QC=QH-Wcycle=10 kW-0.66 kWQC=9.34 kW

The coefficient of performance of the cycle is:

γmax=THTH-TC=(30+273K)30-10K=15.15

- Question 6-(8 points)

The refrigerator shown in the figure operates at steady state with a coefficient of performance of 6.5 within a kitchen at 23oC. The refrigerator rejects 4.8 kW by heat transfer to its surroundings from metal coils located on its exterior.

- Determine the power input, in kW.

- Determine the lowest theoretical temperature inside the refrigerator, in oC.

- Determine the coefficient of performance when the temperature inside the refrigerator is the theoretical minimum.
.

β=QcWcycle=QcQH-Qc

Qc=QHβ1+β=(4.8 kW)5.01+5.0=4 kW

Wcycle =Qcβ=4kW5.0

Wcycle =0.8 kW

β≤βmax=TCTH-TC

TC≥THβ1+β=(23+273 K)5.01+5.0=246.7 KTC, =-26.33∘C⟵cold

βmax=TC, TH-TC, =(-26.33+273)K(23+26.33)K=5.0

- Question 7-(8 points)

As shown in in the figure, a heat pump operating at steady state provides energy by heat transfer to water vaporizing from saturated liquid to saturated vapor at a pressure of 2 bar with a mass flow rate of 0.050 kg/s. The heat pump receives energy by heat transfer from a pond at 14oC. The power required by the pump is 35 kW. These are the only significant heat transfers. Kinetic and potential energy effects can be ignored.

- Determine the coefficient of performance for the heat pump.

- Determine the required power, in kW, for a reversible heat pump under the same conditions.

- Determine the coefficient of performance for a reversible heat pump under the same conditions.

γ=QHWcycle

QH=mhg,2bar-hf,2bar=0.05kgs(2706.7-504.70)kJkg1 kW1 kJ/s=110.1 kW

γ=QHWcycle=110.1 kW35 kW=3.15

γrev=QHWrev=THTH-TC

Wrev =QHTH-TCTH=(110.1 kW)120.2-14 K120.2+273 K=29.7 kW

γrev =THTH-TC=(120.2+273)K(120.2-14)K=3.70

- Question 8-(8 points)

A power cycle operates between hot and cold reservoirs at 500 K and 310 K, respectively. At steady state the cycle develops a power output of 0.1 MW.

- Determine the minimum theoretical rate at which energy is rejected by heat transfer to the cold reservoir, in MW.

η=wcycle QH

η≤ηmax=1-TCTH 
 Wcycle QH≤1-TCTH.

Wcycle Wcycle +QC≤1-TCTH

Wcycle Tc/TH1-TCTH≤QC
0.1MW[310/500]1-310500≤QC

0.163MW≤Qc→Qcmin =0.163MW

- Question 9-(8 points)

Data are provided for two reversible refrigeration cycles. One cycle operates between hot and cold reservoirs at 27°C and 3°C, respectively. The other cycle operates between the same hot reservoir at 27°C and a cold reservoir at -35°C. The refrigerator removes the same amount of energy by heat transfer from its cold reservoir.

- Determine the ratio of the net work input values of the two cycles, WCycle,2WCycle,1,

Refrigeration Cycle @1 → QWcycle,1 =TCTH-TC=3+272.1527-3=11.464583

Refrigeration Cycle @2 → QWcycle,1 =TC'TH-TC=-35+272.1527--35=3.825

Wcycle,2Wcycle,1=11.4643.825=2.997

- Question 10-(8 points)

A refrigeration cycle driven by an electric motor must maintain a computer laboratory at 18°C when the outside temperature is 30°C The thermal load consists of heat transfers entering through the walls and roof of the laboratory at a rate of 75,000 kJ/h and from the computers, lighting, and occupants at a rate of 15,000 kJ/h.

- Determine the minimum theoretical power required by the electric motor, in kW, and the corresponding coefficient of performance.

- If the actual power required by the motor for this duty is 8.3 kW, determine the coefficient of performance.

- If the given temperature and thermal load data are observed for a total of 100 hours and electricity costs 13 cents per kW · h, determine the cost, in $, over that period for each of cases (a) and (b).

QC=(75000+15000)kJh=90,000 kJ/h.

βmax =TCTM-TC=18+272.1530-18=24.26

Wcycle min=QCβmax=90,000 kJ/h24.251 h3600s1 kW1 kJ/s=1.03KW

β=Qc Wale 2=(40,000 kJ/h)(8.3 kW)1 h3600s1 kW1 kJ/s=3.01

For case (a): cost =(1.03 kW)(100 h)0.13$kW⋅h=$13.40
For case (b):  cost=(8.3 kW)(100 h)0.13$KW.h=$107.90

- Question 11-(10 points)

At steady state, a heat pump driven by an electric motor maintains the interior of a building at TH = 293 K. The rate of heat transfer, in kJ/h, from the building through its walls and roof is given by 8000(TH - TC), where TC is the outdoor temperature.

- Determine the minimum electric power, in kW, required to drive the heat pump for TC = 261 K.

γ=QHWcycle≤THTH-TC→ Wcycle ≥TH-TCTHQH=(8000)TH-TCTH2

Wcycle max =8000kJ/hk293k-26122931h3600s*1kW1kJ/s

=7.58×10-3kWK2293-2612K2=7.76192

- Question 12-(10 points)

By supplying energy at an average rate of 30,000 kJ/h, a heat pump maintains the temperature of a dwelling at 20°C.

If electricity costs 8.5 cents per kW·h, determine the minimum theoretical operating cost for each day of operation if the heat pump receives energy by heat transfer from:

- The outdoor air at -7°C.

- The ground at 5°C.

γmax≥γTHTH-Tc≥QHWcycleWcycle≥QHTHTH-Tc=QHTH-TcTH

=24,000kJh293 K-(-7+273.15)293 K1 h3600 s1 kW1kJs

=0.614 kW

Daily Operating Cost: $≥Wcycle 24 h1 day $0.085 kW⋅h

$≥(0.614 kW)24 h1 day $0.085 kW⋅h=$1.25/ day

24,000kJh293 K-(5+273.15)293 K1 h3600 s1 kW1kJs

=0.341 kW

Daily Operating Cost: $≥Wcycle 24 h1 day $0.085 kW⋅h

$≥0.341 kW24 h1 day $0.085 kW⋅h=$0.70/ day