#1 Problem 1.b, Remember to both draw the state diagram and complete the timing sequence as x changes. You only need to trace one step further after the x sequence stops.

| x | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| q | A | B | B | C | D | C | D | C | D | C | A | B | C | D | B |  |
| z | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |

#2 Problem 1.d, Remember to both draw the state diagram and complete the timing sequence as x changes. You only need to trace one step further after the x sequence stops.

| x | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| q | A | A | B | C | A | A | B | D | D | D | C | B |
| z | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |

#3 Problem 4.a, The problem asks you to complete the timing diagram; show the states of q in a timing diagram. Remember to take the CLR’ signal into consideration. Number the clock edges (starting from 1 and then 2, 3, . . . ) and state the JK’s action (HOLD, RESET, SET, or TOGGLE) at each clock edge.

For the following JK flip flops, complete each of the timing diagrams. First, assume that CLR’ and PRE’ are inactive (1). Then, use the values shown.

Q1:

at 1, J=1, K=0, Q1 is SET to 1; at 2, J=0, K=0, Q1 HOLD; at 3, J=1, K=1, Q1 TOGGLE;

at 4, J=1, K=1, Q1 TOGGLE; at 5, J=1, K=0, Q1 HOLD; at 6, J=0, K=0, Q1 HOLD; at 7, J=1, K=1, Q1 TOGGLE;

Q2:

at 2, J=0, K=0, Q1 RESET; at 3, J=1, K=1, Q1 TOGGLE; at 4, J=1, K=1, Q1 TOGGLE;

at 5, J=1, K=0, Q1 HOLD; at 6, J=0, K=0, Q1 HOLD; at 7, J=1, K=1, Q1 TOGGLE;

4. Problem 5.a,

IN=x ⨁ Q

5. Problem 8.b, Do both i and ii. In addition, you are also required to derive the next state equations for A* and B*

8. For each of the following circuits and input strings i. Construct a state table (calling the states 00, 01, 10, 11). ii. Show a timing trace for the values of the flip flops and the output for as far as possible. Assume that the initial value of each flip flop is 0.

Formulas: J1=x'B; K1=x; J2=x'; K2=xA'; z=x'A'+xB

Present State: AB=00

If x=0; J1=0, K1=0, J2=1, K2=0, z=1, thus next state will be A*B*=01

If x=1; J1=0, K1=1, J2=0, K2=1, z=0, thus next state will be A*B*=00

Present State: AB=01

If x=0; J1=1, K1=0, J2=1, K2=0, z=1, thus next state will be A*B*=11

If x=1; J1=0, K1=1, J2=0, K2=1, z=0, thus next state will be A*B*=00

Present State: AB=10

If x=0; J1=0, K1=0, J2=1, K2=0, z=0, thus next state will be A*B*=11

If x=1; J1=0, K1=1, J2=0, K2=0, z=0, thus next state will be A*B*=00

Present State: AB=11

If x=0; J1=1, K1=0, J2=1, K2=0, z=0, thus next state will be A*B*=11

If x=1; J1=0, K1=1, J2=0, K2=0, z=1, thus next state will be A*B*=01

STATE TABLE

| AB | A*B* |  | z |  |
| --- | --- | --- | --- | --- |
|  | x=0 | x=1 | x=0 | x=1 |
| 00 | 01 | 00 | 1 | 0 |
| 01 | 11 | 00 | 1 | 1 |
| 10 | 11 | 00 | 0 | 0 |
| 11 | 11 | 01 | 0 | 1 |

Timing Trace

| x | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| B | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| z | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |