ELCT 363: Introduction to Microelectronics University of South Carolina/ Spring 2022

Homework # 3  Due by:2/28/2022

Total Points 40

# PROBLEM No 1: (1+1+1 =3 points)

Determine the probability that an energy level is empty of an electron if the state is below the Fermi level by (a) kT , (b) 5 kT , and (c) 10 kT .

# Solution:

fEOccupied=1 +eE-EFkT-1→fEEmpty=1 +eEF-EkT-1

- fEF-kT=1 +eEF+kT-EFkT-1=1 +ekTkT-1=1 +e1-1=0.26894142137

- fEF-5kT=1 +eEF+5kT-EFkT-1=1 +e5kTkT-1=1 +e5-1=0.0066928509243

- fEF-10kT=1 +eEF+10kT-EFkT-1=1 +e10kTkT-1=1 +e10-1=4.5397868702434*10-5

PROBLEM No. 2 (2 points)

The probability that a state at Ec + kT is occupied by an electron is equal to the probability that a state at Ev − kT is empty. Determine the position of the Fermi energy level as a function of Ec and Ev.

Solution:

fEOccupied=1 +eE-EFkT-1→fEEmpty=1 +eEF-EkT-1

fEc+kT=1 +eE-EFkT-1=1 +eEc+kT-EFkT-1

1-fEv-kT=1 +eEF-EkT-1=1 +eEF-Ev+kTkT-1

1 +eEc+kT-EFkT-1=1 +eEF-Ev+kTkT -1

eEc+kT-EFkT=eEF-Ev+kTkT

Ec+kT-EFkT=EF-Ev+kTkT

Ec+kT-EF=EF-Ev+kT

Ec-EF=EF-Ev

2EF=(Ec+Ev)

EF=Ec+Ev2

# PROBLEM No 3: (2.5 + 2.5 + 2 = 7) points

- Calculate the thermal equilibrium electron and hole concentration in silicon at T= 350K for the case when Fermi Energy is 0.22 eV below the conduction band. Given that Eg = 1.12 eV; at 350K Nc = 2.8 x 1019  cm-3, and Nv = 1.05 x 1019 cm-3.

- Draw a representative energy band diagram (using parallel lines) showing conduction band, valence band, Fermi energy level and intrinsic energy level. Mark band-gap and other energy levels.

Solution:

Fermi energy is (1.12-0.22=0.9eV) above the Valence Band.

n0=Nc*exp-Ec-EF kT→n0=2.8*1019*e-0.228.617*10-5*350=1.902*1016cm-3

p0=Nv*exp-EF-Ev kT=1.05*1019*e-0.98.617*10-5*350=1.15*106cm-3

# PROBLEM No 4: (2 + 2 +2 + 2 = 8) points

Given the effective density of states of holes, electrons, and the bandgaps in the in table provided at the end of this HW, calculate the position of Fermi levels with respect to center of the bandgap at 300K for

- Si

- Ge

- GaAs

- Plot all of the above (separately for each material) calculations using straight lines indicating conduction/valance bands mid-gap and Fermi levels.

Solution:

- Nc=2.8*1019; Nv=1.04*1019; Ev=1.12; mp*=0.56; mn*=1.08;   EFi-Emidgap=34kT*lnmp*mn*

EFi-Emidgap=34*8.617*10-5*300*ln0.561.08=-0.01273eV

- mp*=0.37; mn*=0.55;  EFi-Emidgap=34kT*lnmp*mn*

EFi-Emidgap=34*8.617*10-5*300*ln0.370.55=-0.00768579eV

- mp*=0.48; mn*=0.067;  EFi-Emidgap=34kT*lnmp*mn*

EFi-Emidgap=34*8.617*10-5*300*ln0.480.067=0.038177eV

- Bandgap data: Si = 1.12eV; GaAs=1.42eV; Ge=0.66eV

Part A

Part B

Part C

Problem 5: (3+3 = 6 points)

(a) Determine the values of n0 and p0 in GaAs at T = 300 K if EF − Ev = 0.25 eV.

(b) Assuming the value of p0 in part (a) remains constant, determine the values of EF − Ev and n0 at T = 400 K. Use Nc and Nv and Ec=Eg from the Room Temperature provided at the end of the questions.

a) GaAs Eg=1.42 eV

Ec-EF=1.42-0.25=1.17eV

n0=Nc*exp-Ec-EF kT→n0=4.7*1017*e-1.178.617*10-5*300=1.038*10-2cm-3

p0=Nv*exp-EF-Ev kT=7.0*1018*e-0.258.617*10-5*300=4.417*1014cm-3

b) p0=4.417*1014cm-3

EF-Ev=kT*lnNvp0=8.617*10-5*400*ln7.0*10184.417*1014=0.33333eV

n0=4.7*1017*e-1.42-0.33338.617*10-5*400=9544.4508cm-3

Problem 6:( 2+4 =6 points)

Assume that silicon(Si), germanium(Ge), and gallium arsenide(GaAs) each have dopant concentrations of Nd = 1 x 1013 cm−3 and Na = 2.5 x 1013 cm−3 at T = 300 K. For each of the three materials:

(a) Is this material n type or p type?

(b) Calculate n0 and p0.

Use value of ni the from the table at the end of the questions.

ni: Si=1.5*1010; Ge=2.4*1013; GaAs=1.8*106

- Because Na>Nd , all 3 materials are P-types.

- Na-Nd=1.5*1013

Silicon

n0=ni2p0=1.5*101021.5*1013=1.5*107cm-3

p0=ni2n0=1.5*101021.5*107=1.5*1013cm-3

Germanium (because ni is on the same magnitude as Na-Nd

n0=Na-Nd2+Na-Nd22+ni2=1.5*10132+1.5*101322+2.4*10132≅3.265*1013cm-3

p0=ni2n0=2.4*101323.265*1013≅1.764*1013cm-3

Gallium Arsenide

n0=ni2p0=1.8*10621.5*1013=0.216cm-3

p0=ni2n0=1.8*10620.216=1.5*1013cm-3

Problem 7:( 2+2+2+2 =8points)

A hypothetical semiconductor has an intrinsic carrier concentration(ni) of 1.0 × 1010/cm3 at
300K, it has conduction and valence band effective densities of states, Nc and Nv, both
equal to 1019/cm3.
(a) What is the bandgap, Eg?

(b) If the semiconductor is doped with Nd = 1 x 1016 donors/cm3, what are the equilibrium
electron and hole concentrations at 300K?

(c) If the same piece of semiconductor, already having Nd = 1 x 1016 donors/cm3, is also
doped with Na = 2 x 1016 acceptors/cm3, what are the new equilibrium electron and
hole concentrations at 300K?

(d) Consistent with your answer to Part (c), what is the Fermi level position with respect
to the intrinsic Fermi level, EF –Ei?

ni2=Nc*Nv*e-EgkT→Eg=-kT*lnni2NcNv→Eg=-8.617*10-5*300*ln1010210192=1.0714eV

(Material is N-TYPE) n=1016cm-3;  p=101021016=104cm-3

(Material is P-TYPE) p=2*1016-1*1016=1016cm-3;  n=101021016=104cm-3

Ei-EF=kT*lnNA-Ndni→8.617*10-5*300*ln10161010=0.357eV