ELCT 302

Laboratory Notebook #2

Model Identification

Group #2B (Steering Control)

December 8th, 2023

I hereby certify that I have complied

with the Spirit and the Letter

of the Carolinian Creed

in preparing this report

Jacob Vaught

Jacob Vaught

Department of Electrical Engineering

University of South Carolina

Columbia, SC 29208

Background	2

Task 1: Servo Motor Model Identification	2

Task 2: Dynamic Steering Frame	3

Steering Control (Group B)

General Objectives

❖ Design circuity necessary to perform a step response of the servo motor

❖ Record data for use in transfer function estimation.

❖ Use software and data to estimate transfer function

❖ Run the simulation and compare with actual results.

Main Specifications

❖ Servo voltage: 4.8V - 6V

❖ Communication frequency: 50Hz

❖ Servo Rotational range: 180⁰

❖ Feedback: LC circuit for a given inductance (at 20 kHz)

Plan of Action (Procedure)

### Background

We now assume to have a fully operational closed-loop control system capable of setting the vehicle’s speed, measuring that speed, and controlling the speed based on a set point. Since our vehicle is assumed to have forward movement, it is necessary to steer in order to follow a specified path. We must now begin to develop the transfer function in order to design the appropriate control system. Modeling the system will need to be done in two parts. First, the motor used to control the steering angle, and secondly to dynamics of the steering frame. The layout of system is seen in Figure 4.

Here, a reference “𝑅(𝑠)” distance, in meters. The difference between the reference signal and the feedback signal “𝐹(𝑠)” gives the error signal “𝐸(𝑠)” in meters. With the error signal, a control system will determine the control output “𝑈(𝑠)” to drive the error signal to zero. This signal, in the linear system diagram will stay as in radians. However, in the implementation, this angle will need to be converted to a duty cycle so a signal can be sent to the servo motor. The servo-motor angle will then be linked to an imaginary center wheel angle “Δ(𝑠)” representing the wheel in the bicycle cycle model discussed later. The vehicle dynamics then relate the wheel angle to the distance “𝐷(𝑠)” from the center line of the track. The linearized model of the sensor then provides a relationship between the distance and the sensor's output voltage, “𝑌(𝑠)” as represented in Figure 4. For an accurate simulation, the collected data would be used as a lookup table to represent the sensor. Next, the approximated linear relationship is then used to calculate the measured distance “𝐹(𝑠)” or feedback signal from the given sensor voltage. The motor used to control the RC car is a standard DC servo motor. The servo motor uses a pulse width modulated (PWM) signal to provide a reference for the steering angle. These servo motors have a required input current to achieve the necessary torque to keep the shaft from rotating. We assume in this laboratory that the required torque to reach the turning angle will always be achievable. Thus, it is not necessary to model the torque on the motor as a function of rotational and forward speed. This separation allows the focus to be on just the servo motor for part one. The motor, controller, and potentiometer are all inside a sealed package. This prevents us from using standard measurement techniques unless we open the box. So instead, we will learn how to estimate a function by gathering data from the control signal and corresponding action for a step input. However, with this type of technique we cannot guarantee the accuracy of the model. Therefore, we will also estimate this system in two other ways, the first as a second order lag, and second, as a delay. With all three models complete, we can compare with the original test for the best matching approximation. You should be familiar with a second order lag system, such as a mass spring dampener, or other examples used in a linear controls course.

### Task 1: Servo Motor Model Identification

❖ Design regulation circuit to power servo motor.

❖ Design a voltage divider circuit with an output displaying the process variable (servo motor position). ❖ Design a circuit to allow for recording the control variable.

➢ Analog Output

❖ Perform a unit step response of the servo motor.

➢ Design, upload and verify code to perform a step response with a microcontroller from 0 radians to 1 radian.

❖ Record Data using the oscilloscope.

➢ Log Control Variable Data

➢ Log Process Variable Data

❖ Adjust the data and import to Matlab

➢ Control Variable should be scaled as a function of radians

▪ 𝑢 = 5𝑥 + 5 is non-linear so we need to convert duty cycle to a commanded radian

➢ Process variable should be scaled as a function of radians

❖ Use Matlab to estimate the transfer function with ‘tfest’

➢ Three input arguments:

▪ The data as an ‘iddata’ structure

▪ Number of poles

▪ Number of zeros

❖ Approximate using one of the other two given approximation methods:

➢ Approximate as a Second-Order Lag Transfer Function

➢ Approximate as a Pade Approximation

❖ Use ‘step’ function in Matlab or Simulink with the estimated transfer functions to the response graph and compare with recorded data.

➢ Make sure the initial and final values are set to the control signal used in the original test.

➢ Determine the most accurate estimated transfer function

The PID is the most accurate according to the MATLAB simulation.

### Task 2: Dynamic Steering Frame

❖ Research “Ackerman Steering” and the bicycle model commonly used for modeling

➢ A great resource to use is [2]

❖ Estimate the mass of the vehicle.

➢ 𝐹 = 𝑚𝑎 ▪ Force or weight of the object ‘F’

▪ Mass of the object ‘m’

m=0.588 kg

▪ Acceleration of the object ‘a’ (gravitational constant)

❖ Find the Center Of Mass of the vehicle and associated constants.

➢ Calculate ‘𝑙𝑓’, the distance from the COM to the front axel.

Lf=0.095 m

➢ Calculate ‘𝑙𝑟 ’, the distance from the COM to the rear axel.

Lr=0.11 m

❖ Calculate an estimate of the moment of inertia.

➢ Calculate ‘𝐼𝑧 ’, the moment of inertia around the z-axis of the vehicle

Iz=(1/12)*m*(w^2 + l^2)

Iz=(1/12)*(0.588 kg)*[(0.185m^2) + (0.29m^2)]

Iz=0.0058 kg*m^2

➢ Approximate using standard shapes, i.e. motors as cylinders.

❖ Validate the model estimation parameters

➢ Use Matlab to estimate a radius of a path given a turn angle ‘𝛿’

▪ Cornering stiffness for both tires should be approximated 20 kN/rad to 25kN/rad

Caf= 22.5e3

Car=22.5e3

▪ Remember the velocity should be low as you will push the car for validation.

Vx=0.2 m/s

➢ Measure the radius of the path from a commanded turn angle and push the car.

Inquiries and Responses

- Assumptions in the Dynamic Model:

- a. Linear Dynamics: The model uses a simplified linear representation of the vehicle's dynamics focused on a specific operational point, implying minor deviations that can be linearly approximated.

- b. Steady Longitudinal Speed: It is assumed that the vehicle maintains a constant forward speed, not considering any acceleration or deceleration.

- c. Bicycle Model Simplification: The vehicle is modeled as having two wheels (bicycle model), which disregards the distinct behaviors of each of the four wheels, reducing it to a single mass point.

- d. Uniform Cornering Stiffness: The model presumes a constant cornering stiffness for both front and rear axles, which might not hold in scenarios with complex tire-road interactions.

- e. Rigid Body Assumption: The vehicle is considered a rigid entity with a constant moment of inertia.

- f. Exclusion of External Forces: External influences such as varying friction or road incline are not factored in, except for cornering stiffness.

- g. Level Terrain Operation: The model operates under the assumption of flat terrain.

- h. Deterministic Parameters: All parameters, including cornering stiffness, vehicle mass, and moment of inertia, are considered precisely known and constant.

- i. Initial State Zero: The model starts with all variables at zero, assuming the vehicle begins stationary.

- Determining the Servo's Control System: a. We employ simulations of various controller types and then compare them visually to actual data to identify the most suitable one. b. The Pade approximation method proved to be the most precise.

- Impact of Steady-State Data on Transfer Function Estimation:

- a. Noise Influence: Steady-state data, which often contains less dynamic information, can introduce noise affecting the estimation. [Resolved by standardizing steady state data to a constant value]

- b. Potential for Bias: A large proportion of steady-state data compared to dynamic data might bias the estimation towards models that fit the steady state well but are less accurate for dynamics. [This is a possible but unlikely effect]

- c. Data Accuracy: Typically, transient data offers more insight into system dynamics. Excluding steady-state data could lead to overlooking valuable information, potentially enhancing the accuracy of the transfer function model. [This potential benefit is negated by setting the data to a constant value]

- d. Computational Load: Incorporating extensive steady-state data might increase the computational demands for estimating parameters without notably enhancing the model's quality. [Not a concern for this lab]

Top of Form

Requirements

❖ Check-off requirements:

➢ Demonstrate simulation of the estimated transfer function.

➢ Answer questions pertaining to the laboratory

➢ Answer specification questions of the components used

❖ Check-off report: (Group)

➢ Refer to rubric and template from Blackboard

➢ Be sure to include:

▪ Results presentation (include o-scope images, measurement records, etc.)

▪ Results analysis and discussion

▪ Hardware schematic (using PSpice, TinyCAD, EAGLE, etc.)

▪ Problem-solving (describe problems you encountered and how you solved them)

Here is all the information for the lab checkoff.

Task 1 code generates outputs for each of the 4 types of controllers vs the actual output from the servo(P, PI, PD, PID), and the Pade_approx code generates the Pade Approximation.

Pade Approximation is most accurate for Task 1.

Motor Model State Equation Variables:

Part I:

• i a (t): armature current

• Ra : armature resistant

• La : armature inductance

• eb (t): back emf

• ea (t): applied voltage

Part II:
• Tm(t): motor torque

• TL (t): load torque

• 𝜃𝑚(𝑡): angular displacement

• 𝜔𝑚(t): angular velocity

• Jm: motor inertia

• Bm: viscous friction

Task 2 code generates state space equations, shows state graphs, and compares the predicted position to the one measured in lab by Mady.

Bicycle Model

– Constant Parameters must be determined

– State variables

• 𝑦 is the distance from COM to the instantaneous center of curvature (ICC)

• 𝜑 is the vehicle heading angle

– Control variable

• 𝛿 is the vehicle turning angle

What the Task2 State Space Graphs represent:

- Lateral Position (y) vs Time: This graph shows how the lateral (side-to-side) position of the vehicle changes over time. If the vehicle is perfectly going straight, this would be a constant value. If the vehicle is moving laterally (perhaps due to a turn), you would see this value change.

- Lateral Velocity (y˙ ) vs Time: This shows the rate of change of the lateral position. It gives you an idea of how quickly the vehicle is moving from side to side. A high magnitude indicates rapid lateral movement, which might happen during sharp turns.

- Yaw Angle (ψ) vs Time: Yaw angle represents the orientation of the vehicle in the plane. It shows how much the vehicle has turned at any given time. This will help you understand how the orientation of the car is changing throughout the time span of the simulation.

- Yaw Rate (ψ˙ ) vs Time: This is the rate of change of the yaw angle and represents how quickly the vehicle is turning. A higher yaw rate would indicate faster turns.

# Time Sheet

| Jacob Vaught |  |
| --- | --- |
| Date/Amount of Time | Work Done |
| 10/2: 2 hours | Wrote code for servo motor to collect data. |
| 10/9: 2 hours | Took measurements of car to finish MATLAB code for task 1. |
| 10/16: 3.5 hours | Updated both MATLAB codes. |

# Problems

| Problems: | Solutions: |
| --- | --- |
| Difficulty in properly measuring the radius from pushing the car. | Performed multiple trials. |
| Data not smoothed | Added smoothing function |
| Converting PWM to step function |  |
| Wrong Values in MATLAB | Corrected it. |
| Did not have Pade Approximation for first check off | Added it to MATLAB code |
| Did not have Global Positioning Graph for first checkoff | Made it in MATLAB |