ELCT 302

Laboratory Notebook #1

Open-Loop Control & Obtaining Feedback

Group #2B (Steering Control)

September 24th, 2023

I hereby certify that I have complied

with the Spirit and the Letter

of the Carolinian Creed

in preparing this report

Mady Hines

Mady Hines

Department of Electrical Engineering

University of South Carolina

Columbia, SC 29208

Steering Control (Group B)	2

General Objectives	2

Main Specifications	2

Plan of Action (Procedure)	2

Background	3

Calculations	3

Task 1: Open-Loop Control	5

Task 2: Distance Measurement	7

Questions	13

Requirements	14

# Steering Control (Group B)

## General Objectives

❖ Design and implement a steering system using concepts of resonance, amplification and rectification

❖ Design and implement servo-position control for steering

## Main Specifications

❖ Servo voltage: 4.8V - 6V

❖ Communication frequency: 50Hz

❖ Servo Rotational range: 180⁰

❖ Feedback: LC circuit for a given inductance (at 20 kHz)

## Plan of Action (Procedure)

### Background

We now assume to have a fully operational closed-loop control system capable of setting the vehicle’s speed, measuring that speed, and controlling the speed based on a set point. Since our vehicle is assumed to have forward movement, it is necessary to steer it in order to follow a specified path. The purpose of this lab is to now develop this steering system using a combination of sensor circuitry and software. The track has an embedded wire propagating a 1 amp, 20 kHz square wave signal. We will develop hardware to sense this current and then determine its location in relation to the center of the vehicle so the processor can make appropriate corrections to the steering mechanism.

Figure 1. Steering Diagram

### Calculations

### Task 1: Open-Loop Control

❖ Design regulation circuit to power servo motor.

Figure 2. Regulation Circuit

❖ Design a voltage divider circuit with an output ranging from 0-3.3V.  (Arduino: 0->5V)

❖ Write code that reads in the voltage divider output as an ADC number, and writes the ratio to the desired maximum as the PWM duty cycle.

Code:

#include <Servo.h>

Servo myservo;  // Create servo object

int potPin = A1; // Analog pin A1 for voltage divider

int val;         // Value read from voltage divider

void setup() {

myservo.attach(9);  // Attaches the servo on pin 9

pinMode(potPin, INPUT);  // Sets potPin as INPUT

Serial.begin(9600); // Initialize serial monitor

}

void loop() {

val = analogRead(potPin);  // Read voltage from potentiometer

val = map(val, 0, 1023, 0, 180);  // Map value to angle (0 - 180)

myservo.write(val);  // Set servo angle

Serial.print("Potentiometer Value: ");

Serial.print(val);

Serial.println(" degrees");

delay(15);  // Small delay for stability

}

❖ Upload and verify with microcontroller.

❖ Connect servo-motor to microcontroller.

❖ Servo Control

➢ Use a 50 Hz control signal to the servo

➢ Verify for full range of motion. (0⁰ - 180⁰)

• Servo motor receives power from voltage regulator

• Servo motor receives PWM signal from microcontroller

• Servo duty cycle vs position

– Center the servo -- approximately 7.5% duty cycle

– Fully left - approximately 5% duty cycle

– Fully right – approximately 10% duty cycle

– Design this in mbed code.

Figure 3. Servo-Motor Control Signal

### Task 2: Distance Measurement

❖ Research data sheets for components and understand why they are used (their ratings)

❖ Build this circuit and understand how it works.

➢ VCC should be from a laboratory power supply with a 1 Amp current limit.

➢ Gate Signal should be a square wave from a function generator.

▪ High Side greater than threshold voltage

▪ Low Side at common voltage

▪ Frequency of 20 kHz

➢ Why build a track circuit on the desk?

❖ Build this circuit and understand how it works.

➢ Keep the common-mode input voltage range in mind when choosing a LM324 V+

➢ Take screenshots of the signals at N1, N2, and N3

Figure 4. Signal at Node 3 (N3).

Figure 5. Signal at Node 2 (N2).

Figure 6. Signal at Node 1 (N1).

➢ Why is the signal seen at the non-inverting input of the LM324 a sine wave when the track signal is a square wave?

Figure 7. Steering Hardware Schematic

❖ Tabularize the output from both sensors.

➢ Signal distance from center of car (m)

➢ Vout (V), measurement in volts from sensor

➢ VoutADC, measurement voltage digital conversion

| Distance: (m from center, left=negative) | Left Vout (V) | Left Vout ADC (V) | Right Vout (V) | Right Vout ADC (V) | Difference of Left and Right Vout ADC (V) {fig 10} |
| --- | --- | --- | --- | --- | --- |
| -0.0635 | 0.6 | 3.26 | 0.65 | 3.14 | -0.11 |
| -0.0508 | 0.6 | 2.89 | 0.4 | 3.04 | 0.15 |
| -0.0381 | 3 | 4.5 | 0.5 | 2.5 | -2 |
| -0.0254 | 3 | 2.18 | 0.7 | 3.8 | 1.62 |
| -0.0127 | 2.8 | 3.69 | 0.25 | 2.38 | -1.31 |
| 0 | 0.5 | 3.67 | 0.5 | 3.7 | 0.03 |
| 0.0127 | 0.5 | 2.41 | 2 | 3.96 | 1.55 |
| 0.0254 | 0.8 | 2.12 | 2.8 | 3.98 | -1.86 |
| 0.0381 | 2.5 | 3.9 | 3 | 4.23 | 0.33 |
| 0.0508 | 0.4 | 3.27 | 0.6 | 2.8 | -0.47 |
| 0.0635 | 1 | 2.92 | 0.7 | 3.2 | 0.28 |

Code:

// Initialize variables for storing analog readings

int analogValueA0 = 0;

int analogValueA1 = 0;

// Initialize variables for storing voltage values

float voltageA0 = 0.0;

float voltageA1 = 0.0;

float voltageDifference = 0.0;

void setup() {

// Start Serial communication

Serial.begin(9600);

}

void loop() {

// Read analog values from A0 and A1

analogValueA0 = analogRead(A0);

analogValueA1 = analogRead(A1);

// Convert analog readings to voltage

voltageA0 = (analogValueA0 * 5.0) / 1023.0;

voltageA1 = (analogValueA1 * 5.0) / 1023.0;

// Calculate the difference in voltage

voltageDifference = voltageA0 - voltageA1;

// Print voltages and their difference to Serial Monitor

Serial.print("Voltage on A0: ");

Serial.print(voltageA0, 2);  // Two decimal places

Serial.print(" V\t");  // Tab for alignment

Serial.print("Voltage on A1: ");

Serial.print(voltageA1, 2);  // Two decimal places

Serial.print(" V\t");  // Tab for alignment

Serial.print("Voltage Difference: ");

Serial.print(voltageDifference, 2);  // Two decimal places

Serial.println(" V");

// Delay for a bit before the next reading

delay(1000);

}

❖ Graph the distance of the car to center of track vs output voltage of each sensor

Figure 8. Graph of the Distance of Car to Center of Track vs Output Voltage of each Sensor.

❖ Graph the distance of the car to center of track vs output difference of the sensors

➢ A check of linearity by using interpolation will complete this lab. Superimpose the average slope on the plot and include the slope’s equation.

Figure 9. Graph of the Distance of Car to Center of Track vs Output Difference of each Sensor.

## Questions

- Why create a linear representation of the final graph?

  - To better understand the relationship between the output voltage ADC and the distance (m).

  - It shows the accuracy of the sensors.

- What is the operational range of the sensor?

  - Arduino: 0V-5V

- Is there a delay in the sensor and is this delay negligible?

  - Yes, there is a delay in the sensor, but it is negligible.

- What is the purpose of the two diodes in the circuit? Could one of these diodes be removed?

  - Diodes ae used for conducting the current in one direction (a positive direction).

  - The Zener diode is not necessary and we did not include it in our circuit design.

- What are the four stages of the sensing circuit above?

  - LC Resonate Circuit

    - Inductor: L1

    - Capacitor: C1

  - Amplification Circuit

    - Op-Amp (Operational Amplifier): LM324

    - 2 Resistors: R1 & R2

  - Rectification Circuit

    - Diode

  - Peak Detection Low Pass Filter

    - Capacitor: Cp

    - Resistor: Rp

Problems

| Problems: | Solutions: |
| --- | --- |
| Finding correct resistor values; Started out using a potentiometer. | Used 1 kohm resistor for R1 and R2 |
| Soldered diode in the wrong direction | Put diode in the correct direction |
| MBED microcontroller stopped working properly | Had to use Arduino and change code to be compatible with Arduino |
| Arduino has operational range of 0-5V and MBED only had operational range of 0-3V; Zener diode of 3.3V does not work | Zener diode is not necessary, so we took it out. |

## Requirements

❖ Check-off requirements:

➢ Demonstrate servo control over entire range using ADC →PWM

➢ Answer questions listed above, and any questions from the instructor and/or TA.

➢ Lab notebook (When? Who? What happened? Schematics and explanations)

➢ Demonstrate servo control and feedback covering the entire range (including 0)

➢ Table with at least 10 data points shown and measurements/calculations explained

➢ Trend line of interpolation must be within range of expected values

❖ Check-off report: (Group)

➢ Refer to rubric and template from Blackboard

➢ Be sure to include:

▪ Encoder/sensor/converter design procedure and explanation

▪ Results presentation (include o-scope images, measurement records, etc.)

▪ Results analysis and discussion

▪ Hardware schematic (using PSpice, TinyCAD, EAGLE, etc.)

▪ Problem solving (describe problems you encountered and how you solved them)