EMCH 290-001-Quiz #4 Solutions                                                   Thursday, October 20th, 2022 Chapter #4

Points Possible: 100

If unit conversion is unclear, ask TA/professor.  Box in ALL final answers!

- Question 1(30 Points)

A pump steadily draws water from a pond at a volumetric flow rate of 0.015[m3s] through a pipe having a 0.25[m] diameter inlet. The water is delivered through a hose terminated by a converging nozzle. The nozzle exit has a diameter of 0.025[m] and is located 10[m] above the pipe inlet. Water enters at 20°C, 1 [bar] and exits with no change in temperature or pressure. The rate of heat transfer is 5% of the power input.

Use Eq 2, 3

- Determine specific volume from Table A-2

- Determine mass flow rate

- Determine the velocity of the water at the inlet and exit, each in m/s,

- Determine change in enthalpy [Note: No change in pressure/temperature]

- Determine the power required by the pump, in kW

- From Table A-2, v=0.0010018

- m=A1V1v1=0.0150.0010018=14.97kgs

- V1=mvA1=14.97*0.0010018π*0.2524=0.3055ms

- V2=mvA2=14.97*0.0010018π*0.02524=30.55ms

- h1-h2=0

- Q=0.05W→Wcv= m0.95h1-h2+V12-V222+gΔZ1000

Wcv= 14.970.950+0.30552-30.5522+9.8-101000=-8.89 kW

- Question 2(30 Points)

Steam flows steadily through an adiabatic turbine. The inlet conditions of the steam are 4 MPa pressure, 500 ˚C temperature, and 80 m/s velocity and the exit conditions are 50 kPa, 90% quality, and 50 m/s. The mass flow rate of the steam is 12 kg/s.

Use Eq 1, 2, 4, 5

Determine:

- Change in Kinetic Energy

- Enthalpy at inlet & exit from Table A-4 & A-3

- Power Output

- Determine specific volume at inlet from Table A-4

- Turbine inlet area

KE=mV12-V222=12*802-5022=23400 J=23.4 kJ

- h1=3445.3 kJkg

- hf=340.49;hg=2645.9;h2=340.49+0.902645.9-340.49=2415.359kJkg

- W=mh1-h2+V12-V22211000

- W=123445.3-2415.359+23.4=12382.7 kW

- v=0.08643m3kg

v=A*Vm→A=m*vV=12*0.0864380=0.0129645m2

- Question 4(20 Points)

Steam enters a converging–diverging nozzle operating at steady state with p1 = 140 [bar], T1 = 400°C, and a velocity of 10 [m/s]. The steam flows through the nozzle with no change in PE, Q, W. At the exit, p2 = 40 [bar], and the velocity is 700 [m/s]. The mass flow rate is 2 [kg/s].

Use Eq 1, 2

- Determine h1 from Table A-4

- Determine h2

- Determine v2 from Table A-4

- Determine the exit area of the nozzle, in [m2].

From Table A-4, h1=3001.9kJkg

0=h1-h2+V12-V222→

h2=3001.9kJkg+102-70022m2s2*1N1kg⋅ms2*1kJ103N⋅m=2756.95kJkg

From Table A-4, v2 =0.07341m3kg

A2=mv2V2→A2=2*0.07341700=20.9cm2

- Qcv+Wcv=mh1-h2+V12-V222+gz1-z211000[1000 is needed in metric.]

- m=AVv→V=mvA→A=mvV

- Q+W=∑mihi+Vi22+gzi-∑mehe+Ve22+gze

- KE=mV12-V222

- h=hf+xhg-hf