For problems 1-4, find ALL minimum sum of products expressions. (If there is more than one solution, the number of solutions is given in parentheses.)      Write ALL solutions.

1. Problem 2.a, 0.5 Points.

f(a, b, c)=∑m(1, 2, 3, 6, 7)

fa,b,c,d=b+a'c

2. Problem 2.d, 0.5 Points.

f(a, b, c, d)=∑m(1, 2, 3, 5, 6, 7, 8, 11, 13, 15)

fa,b,c,d= a’c+bd+cd+a’d+ab’c’d’

3. Problem 2.f, 0.5 Points.

h(a, b, c, d)=∑m(2, 4, 5, 6, 7, 8, 10, 12, 13, 15) (2 solutions)

f1=bd+bc'+ab'd'+a'cd'

f2=bd+a'b+ac'd'+b'cd'

4. Problem 2.k, 0.5 Points.

f(w, x, y, z)=∑m(0, 1, 2, 4, 5, 6, 9, 10, 11, 13, 14, 15) (2 solutions)

f1=w'y'+wz+yz'

f2=w'z'+y'z+wy

5. Problem 3.a, 0.5 Points.

For the following function,

i. List all prime implicants, indicating which are essential.

ii. Show the minimum sum of products expression(s).

f(a, b, c, d)=∑m(0, 3, 4, 5, 8, 11, 12, 13, 14, 15)

c'd',bc',ab, b'cd  are prime implicants

c'd',bc',ab, b'cd  are essential prime implicants

fa,b,c,d=c'd'+bc'+ab+b'cd

6. Problem 5.a, 0.5 Points.

For the following, find ALL minimum sum of products expressions. (The number of solutions is given in parentheses.) Label the solutions f1, f2, ...

f(w, x, y, z)=∑m(1, 3, 6, 8, 11, 14) + ∑d(2, 4, 5, 13, 15) (3 solutions)

f1=xyz'+wx'y'z'+x'yz+w'x'z

f2=xyz'+wx'y'z'+wyz+w'x'z

f3=xyz'+wx'y'z'+x'yz+w'y'z

For problems 7 and 8, find ALL the minimum sum of products expressions and ALL the minimum product of sums expressions:

7. Problem 7.b, 0.5 Points.

f(w,x,y,z)=∑m(2, 4, 5, 6, 7, 10, 11, 15)

SOP

fw,x,y,z=w'x+wyz+x'yz'

POS

f'w,x,y,z=wy'+x'y'+w'x'z+wxz'

f'w,x,y,z'=w'+yx+yw+x+z'w'+x'+z

8. Problem 7.j, 0.5 Points.

f(a, b, c, d)=∑m(0, 1, 6, 15) +∑d(3, 5, 7, 11, 14) (1 SOP and 2 POS solutions)

SOP

Made mistake. I did not need to circle the purple square for SOP. I have removed it from the algebraic expression below.

fa,b,c,d=a'b'c'+bc

POS

f1'a,b,c,d=bc'+b'c+ab'

f1a,b,c,d=b'+cb+c'a'+b

f2'a,b,c,d=bc'+ac'+ab'

f2a,b,c,d=b'+ca'+ca'+b