Department of Mechanical Engineering

EMCH 368

MECHATRONICS

Lab 11

Stepper Motor

Last Edited: 11/5/2024 by Dylan Homiak

In this lab you will use the ELEEGO environment to control the speed of a stepper motor by stepping a low-power digital signal from the ELEEGO digital pins to a high-power driving signal that will enable the stepper motor to rotate. This will be accomplished using a micro-stepper driver and an external power supply.

Materials:

- ELEEGO/ breadboard/ button/ resistor/ potentiometer/ jumper wires

- Stepper motor

- ULN2003 Stepper Motor Driver Module

- IR Receiver Module

- Remote Control

Exercise 1: Constant speed

In this exercise you will connect the stepper motor to the driver and power supply then wire the direction and speed control ports into the ELEEGO digital pins. A control script is then written in ELEEGO IDE with no user input beyond the initial set speed and direction.

Building the Circuit:

- Connect In1, In2, In3, and In4 pins on the Stepper Motor Driver Module to digital pins 8, 9, 10 and 11 respectfully.

- Connect – pin on the module to ground and the + pin to 5v

- Connect the Stepper Motor to the Stepper Motor Driver Module

Exercise 1 Circuit

Writing the Program:

Exercise 2: Speed and direction control

- In this exercise you are expected to add to the previous exercise to allow for user input.

- Use a button circuit to switch the direction of the motor rotation

- Use a potentiometer to change the speed

Materials:

- 220-ohm resistor

- Button

- Potentiometer

- Stepper motor

- Micro step driver

Writing the Code:

You are expected to modify the code given to you in the first exercise to include the speed and direction controls.

Exercise 3: IR Receiver Module and Remote Control

For this exercise, you will be introduced to the IR Receiver Module and the use of the Remote Control in your kit. First we have to start with downloading a library. To download the library first follow this link GitHub - Arduino-IRremote/Arduino-IRremote: Infrared remote library for Arduino: send and receive infrared signals with multiple protocols, then click the green code button and download as a zip. Then on Arduino Ide go into the sketch tab, then Include Library and add the zip file for the library.

Once the library is installed, we can start making the circuit. For this simple introduction to the IR Receiver module, you only need a few wires, the module itself and your UNO board.

Materials:

- IR Receiver Module

- Remote Control

- ELEEGO UNO R3

- 3 Male to Female Jumper Wires

Circuit:

On your IR Receiver you will see GRY on the pins, G goes to ground, R to power, and Y to a digitalPin.

Code:

To learn more about the IR Receiver, this website IR Remote and Receiver with Arduino Tutorial (4 Examples) (makerguides.com) gives really good information about how it works and is also where the code you're using above comes from.

Challenge: Remote Controlled Stepper Motor

Using what you have learned before about the stepper motor and the IR Receiver Module, combine the two to make a controller that Stops, Increase/Decreases Speed, and Changes Direction.

Expectations:

- 0-9 can set speed

- Vol+, Vol- ,  increase and decrease speed by increments.

- Pause button pauses and plays the motor

- Next and Back button control Direction

Hints:

- Speed should never equal 0. The equation used for the stepper motor is stepsPerRevolution/speed, and it’s not possible to divide by 0, so this will cause the program to no longer work as it’s trying to do the impossible.

- Use Serial print to help monitor your program

Helpful Link for Reference:

- https://www.youtube.com/watch?v=09Mpkjcr0bo

- https://www.youtube.com/watch?v=FggSRQqB_a4

Lab 11 Grading Rubric:

- Exercise #1 Works properly (Motor spins with Constant speed, 10 points)

- Exercise #2 Works properly (Speed and direction control, 10 points)

- Exercise #3 Works properly (10 points)

- Comprehension Questions (10 points)

- Challenge Question (20 points)

- Grade distribution template (total 50 points)