#1 What fundamental role does a capacitor play in an electrical circuit?

A) To convert AC to DC

B) To store and release electrical energy

C) To increase the voltage

D) To produce light

#1d Which of the following is a capacitor NOT typically known for?

A) Amplifying signals

B) Storing electrical energy

C) Smoothing out voltage fluctuations

D) Blocking direct current while allowing alternating current to pass

#2e What aspect of servo motor movement is controlled by the PWM 'on-time' or pulse-width?

A) The speed of the servo motor.

B) The direction of the servo motor.

C) The angle of rotation of the servo motor.

D) The stall torque of the servo motor.

#2h Within what range can a standard servo motor typically rotate?

A) 0 to 90 degrees.

B) 0 to 180 degrees.

C) 0 to 270 degrees.

D) 0 to 360 degrees.

#2i How long is the typical 'on-time' pulse duration for controlling a servo motor?

A) 500 µs to 2500 µs.

B) 1000 µs to 2000 µs.

C) 1500 µs to 3000 µs.

D) 2000 µs to 4000 µs.

#2j If aiming for a 45-degree angle, what should the 'on-time' pulse duration be for a servo motor?

A) 1000 µs.

B) 1250 µs.

C) 1500 µs.

D) 1750 µs.

E) 2000 µs.

#6a Which of the following equations is NOT directly associated with the operation of a brushed DC motor?

A) F=I×L×B

B) V=Eb+I×R

C) τ=B×I×L×r

D) P=V×I

E) Eb=k×ω

#6g How does the magnetic field (B) impact the functionality of a brushed DC motor? (Think about the Lorentz Force)

A) It influences the motor's torque and speed.

B) It cools the motor during operation.

C) It reduces the electrical resistance of the armature.

D) It increases the motor's weight.

#6i What is the main purpose of the brushes found in a brushed DC motor?

A) To clean the commutator.

B) To maintain electrical contact with the rotating commutator.

C) To cool the armature.

D) To lubricate the motor's moving parts.

E) To increase the motor's rotational speed.

#6l Describe the function of the armature (also known as the rotor) in a brushed DC motor.

A) To provide structural support to the motor.

B) To rotate within the magnetic field, creating motion.

C) To store electrical energy for the motor.

D) To convert the motor's energy into light.

E) To insulate the motor's internal components.

#7a What does MOSFET stand for?

A) Metal-Organic Semiconductor Field-Effect Transistor

B) Metal-Oxide-Semiconductor Field-Emission Transistor

C) Metal-Oxide-Semiconductor Field-Effect Transistor

D) Metallic-Oxide Structure Field-Energy Transistor

#7c What are the three terminals of a MOSFET called?

A) Emitter, Base, Collector

B) Source, Drain, Gate

C) Anode, Cathode, Electrode

D) Input, Output, Control

#8a In the H-Bridge schematic from the lab document, what would turning on Q3 and Q4 simultaneously cause? (Think about it as if each transistor is a switch)

A) The motor would spin in the forward direction.

B) The motor would spin in the reverse direction.

C) The motor would experience rapid braking and come to a stop, as activating Q3 and Q4 induces a dynamic braking effect by shorting the motor's terminals, leveraging the motor's back EMF.

D) The motor would increase in speed.

#8c With the dimple aligned to the top and looking down at a 16 PIN Integrated Circuit (IC), where would pin 9 be located?

A) Top left pin.

B) Top right pin.

C) Bottom left pin.

D) Bottom right pin.

E) Left pin, 4 rows down.

#8f How many Transistors are needed to build an H-Bridge?

A) Two

B) Four

C) Six

D) Eight

#9b What does the term "forward bias voltage" mean in the context of diode operation?

A) The voltage at which the diode begins to conduct current in the reverse direction.

B) The voltage required across the diode to prevent any current flow.

C) The minimum voltage required across the diode to initiate current flow in the forward direction.

D) The maximum voltage the diode can withstand without conducting in either direction.

#9d What is the typical forward bias voltage required for a silicon diode to conduct current?

A) 0.2 to 0.3 volts

B) 0.6 to 0.7 volts

C) 1.0 to 2.0 volts

D) 3.0 to 4.0 volts

#9e How does the physical structure of a diode indicate its correct orientation in a circuit?

A) The cathode is typically marked with a colored band.

B) The anode is typically marked with a colored band.

C) Both terminals are marked with different colors.

D) Orientation is indicated by the size of the terminals.

#9g Which condition describes a diode operating in reverse bias?

A) The anode is connected to a higher voltage than the cathode.

B) The cathode is connected to a higher voltage than the anode.

C) The diode is connected in parallel with the power source.

D) The diode is connected in series with the load.

#9h In what application is the knowledge of forward and reverse bias most critical?

A) When implementing diodes for AC-to-DC rectification.

B) When using diodes to increase circuit temperature.

C) When diodes are used as resistive elements.

D) When diodes are used to increase signal frequency.

#11b Which of the following is the correct way to create a new instance of a class named Motor368?

A) int Motor368 = new Motor368(pin1, pin2, pin3);

B) Motor368 motor = Motor368(pin1, pin2, pin3);

C) Motor368 motor(pin1, pin2, pin3);

D) new Motor368 motor(pin1, pin2, pin3);

#11d Why might private member variables of a class start with an underscore (_)?

A) To indicate that they are constants.

B) To indicate that they are global variables.

C) To signify that they are private and should not be accessed directly outside the class.

D) To cause syntax errors for educational purposes.

#11h What is the standard file extension for an Arduino library header file?

A) .txt

B) .cpp

C) .h

D) .ino

#11i Which Arduino statement is used to include a library at the beginning of your sketch?

A) #include

B) #library

C) import

D) useLibrary

#11k In object-oriented programming, what is a class?

A) A specific set of instructions for performing a task.

B) A blueprint from which individual objects are created.

C) A variable that stores numeric data.

D) A function that performs calculations.