EMCH 290

Mechanical Engineering Thermodynamics

HW # 2

Points Possible: 100 [2% of FINAL Class Grade]

Name: ………………………………

Date: Due September 19th, 2023

- Question 1(7 Points)

A flywheel whose moment of inertia I = 200 kg·m2 is spinning at the speed shown in the figure. The flywheel is connected to a mass via a pulley.  For a flywheel, KE = Iω2/2, where ω is the angular velocity in rad/s. Let g = 9.81 m/s2. Assume air resistance and friction is negligible

- If the mass connected to the flywheel is 50 kg, what will be the change in height of the mass in meters? (Hint: KE = PE)

- Now, assuming the mass connected to the flywheel is 140 kg, what will be the change in height?

- Question 2(8 Points)

A foreign country is aiming a missile at a target destination, however the missile is accidentally fired straight upward and reaches a height of 900 meters when its velocity reaches zero (apogee). The only force acting on the object is the force of gravity(assume air resistance is negligible). g = 9.81m/s2.

- How much KE was needed to fire the missile, in kJ

- Determine the firing velocity of the missile, in km/s.

- Question 3(9 Points)

A vehicle is stopped at the red light on Main St. and Blossom St. While waiting, the vehicle’s brakes fail, causing the vehicle to roll down the hill towards Swearingen. Assuming the street is inclined at 60o from the horizontal, what will the vehicle’s Kinetic Energy and velocity be at the 100 meter mark? Air resistance and friction between the vehicle and the street are negligible.

Let g = 9.81 m/s2.

- Find the kinetic energy of the vehicle, in kJ, at the bottom of the street.

- What is the velocity of the vehicle, in m/s, at the bottom of the street.

- What is the velocity in mph?

- Question 4(7 Points)

You're enjoying a day at the beach when you spot a sailboat, weighing 500 kg, with a 100 kg sailor aboard. The boat is making good speed when, out of nowhere, a storm brews. The storm's force, a formidable 1000 N, causes the sailboat to come to a rest, before it capsizes the sailboat.

Assume the force the wind applies is parallel to the direction of the boat initially.

- Determine the amount of energy transfer by work, in kJ, for this process. (Include both sailor and boat mass)

- Determine the distance, in m, the sailboat managed to travel before the storm overpowered it.

- Question 5(10 Points)

A Formula One car tire after a race has 10 kg of air in it with a pressure of p1 = 450 kPa, and a specific volume of v1 = 0.25 m3/kg. After the tire sits for a while, the tire’s pressure and specific volume change to p2 = 140 kPa, v2 = 0.7 m3/kg.

- Determine the polytropic exponent, n, for the process.

- Determine the amount of energy transfer by work, in kJ, for the process.

- Question 6(7 Points)

A heat pump cycle operating at steady state receives energy by heat transfer from well water at 10oC and discharges energy by heat transfer to a building at the rate of 1.2x105 kJ/h. Over a period of 14 days, an electric meter records that 1620 kW·h of electricity is provided to the heat pump. These are the only energy transfers involved.

- Determine the total amount of energy transfer by work into the heat pump over the 14-day period, in GJ.

- Determine the amount of energy that the heat pump receives by heat transfer from the well water over the 14-day period, in GJ.

- Determine the heat pump’s coefficient of performance.

- Question 7(7 Points)

Imagine you're working as an engineer in a high-tech aerospace company, and you're currently testing a new model of an electric motor for a future drone design. The electric motor, while operating under steady-state conditions, draws a current of 11 A at a voltage of 120 V. During operation, the output shaft of the motor develops a torque of 8.4 N·m and rotates at a speed of 1200 RPM.

(W=P=V*I)

- Determine the electric power, in kW.

- Determine the power developed by the output shaft, in kW.

- Determine the rate of heat transfer, in kW

- Question 8(7 Points)

An object whose mass is 100 kg experiences a decrease in kinetic energy of 500 N*m and an increase in potential energy of 1500 N*m. The initial velocity and elevation of the object, each relative to the surface of the earth, are 40 m/s and 30 m, respectively. If g=9.81 m/s^2, determine:

- The final velocity, in m/s.

- The final elevation, in m.

- Question 9(7 Points)

Find the mass flow rate (in grams per second) of water for a control volume system, if the volume flow rate is 2 liter per second.  [ρwater=1gcm3]

Volume flow rate = 2 (liter/s)

Mass flow rate =? (g/s)

- Question 10(7 Points)

What is the change in the internal energy of the system if the surrounding releases 300 J of heat energy and if the system does 550 J of work on the surroundings?

- Question 11(7 Points)

The refrigerator shown in the figure steadily receives a power input of 0.15 [kW] while rejecting energy by heat transfer to the surroundings at a rate of 0.6[kW]. Determine the rate at which energy is removed by heat transfer from the refrigerated space, in [kW], and the refrigerator's coefficient of performance.

- Question 12(7 Points)

For a power cycle operating as shown in the figure, Wcycle = 600 kJ and Qout = 1800 kJ.

- What is the heat input, Qin, in kJ?

- What is the cycle thermal efficiency?

- Question 13(10 Points)

A vertical piston–cylinder assembly with a piston of mass 35 kg and having a face area of 0.006 m2 contains air. The mass of air is 0.0025 kg, and initially the air occupies a volume of 0.0025 m3. The atmosphere exerts a pressure of 100 kPa on the top of the piston. The volume of the air slowly decreases to 0.001 m3 as energy with a magnitude of 1 kJ is slowly removed by heat transfer. Friction between the piston and the cylinder wall can be neglected and g = 9.8 m/s2.

- Determine the work done by the air on the piston during the process, in kJ.

- Determine the change in specific internal energy of the air, in kJ/kg.