TITLE: Surface-Object Detection and Tracking from Intensity-Only Commercial X-Band Radar Using Adaptive ST-DBSCAN

Authors: Samuel Cancilla, J.C. Vaught, Douglas Cahl, Yi Wang

Department of Mechanical Engineering, University of South Carolina, Columbia, SC 29028

Abstract: Maritime missions such as over-water navigation, ship-based operations, and airborne/space-enabled maritime surveillance require reliable surface-object detection and tracking in the presence of strong, nonstationary sea clutter and intermittent object returns. We present an end-to-end, real-time pipeline for vessel detection and multi-sweep tracking using commercial X-band radar providing only intensity measurements (range, azimuth, and intensity) without Doppler phase history. Raw polar returns from a Furuno NXT solid-state unit are transformed into Cartesian point clouds and clustered using ST-DBSCAN to link detections across sweeps. Neighborhood radii adapt automatically via k-nearest-neighbor distance statistics to handle varying clutter and point densities. A lightweight gap-recovery mechanism reconnects clusters across brief temporal dropouts. We evaluate the approach on (i) a synthetic dataset with ground-truth trajectories and (ii) coastal radar data from a shore-mounted installation, characterizing detection-to-track association behavior relative to baseline DBSCAN and standard ST-DBSCAN under nonstationary clutter and intermittent returns. A Rust implementation runs at 2 sweeps/s on an NVIDIA Jetson Orin, exceeding real-time requirements for 48 RPM radar.