# Antenna Research Project ELCT 499

[Dr. Chandra]

Written by Jacob Vaught

- Introduction or Summary:

  - This project notebook compiles essential information for the design and simulation of an antenna meeting NRF24L01 specs for Dr. Downey’s SHM UAV Deployable sensor package.

- Technical Specifications and Data:

  - Relative Permittivity of Polyimide (Kapton): 3.2-3.5

    - Source: Engineering Toolbox

  - NRF24L01 Datasheet: Components101

    - Key Point: NRF24L01 works with GFSK (Gaussian Frequency Shift Keying)

- Product Links:

  - Kapton Purchase Page: Voltera Store

  - Kapton Datasheet: Gitbook

  - Voltera Ink Datasheet (resistivity: 1.27× 10−4 Ω.m): Gitbook

  - US Radio Allocations: january_2016_spectrum_wall_chart_0.pdf (ntia.gov)

- Tutorials and Guides:

  - Simulation of Antennas in ADS:

    - Basic Instructions: Stanford University

    - Detailed Guide: Stanford University

    - Alternative Guide: Pipiads

- Design Requirements:

  - Antenna to fit inside a 2-inch diameter cylinder.

  - Operational Frequency: 2.4-2.525 GHz (RF24 specs).

  - Recommended Load: 15Ω+j88Ω for maximum output power (0dBm).

  - Goal: Performance comparable or superior to integrated antenna design.

- Notes and Observations:

  - Most likely will start with design in research paper 4.d

    - Simple, single sided, easy to implement with current NRF24L01 modules.

    - Need to measure impedance once printed

    - Did not function correctly. Paper seems to have small errors.

    - Results show 2.9 GHz instead of 2.45 GHz.

  - Would like to try using some designs referenced in book “Practical microstrip and Printed Antenna Design” By Anil Pandey

  - Will try design in research paper 4.e

# Research Papers:

- Polyimide or Kapton Antenna Design:

  - Brain tumor detection by Kapton Polyimide based on-body patch antenna in K band (2023) [Microstrip monopole antenna, double sided]

  - Design and analysis of a Foldable Miniaturized Single Band Origami Antenna for Space Applications (2023) [Origami, Probably monopole, foldable]

  - Antenna array design on flexible substrate for wireless power transfer (2021) [Directional, monopole, microstrip, double sided]

- Best Antenna Types for Flexible Antennas:

  - Flexible Antennas: A Review (2020)[Good for overview]

  - Effect of Conductive Materials and Substrates for Flexible Patch Antennas: A Comprehensive Review (2021) [Good review of possible materials for future work, NOT applicable to my work today]

  - Flexible Antennas for a Sub-6 GHz 5G Band: A Comprehensive Review (2022)[Good overview of antenna types that can be printed]

- Basics of Antenna Design (e.g., Dipole, Monopole):

  - Design an Extended Circular Planar Microstrip Patch Antenna for UWB Application (2016)[monopole, double sided, High frequency]

- Polyimide (Kapton) Dipole Antenna Design:

  - Design of Kapton based passive circuits at microwave frequencies (2015)[Good analysis, Material properties, multiple antennas tested]

  - Design and development of a compact wearable dipole GPS antenna (2016)[Di-pole, single-sided, Low frequency, kapton, still useful]

  - A Compact Kapton-Based Inkjet-Printed Multiband Antenna for Flexible Wireless Devices (2015)[triangular Monopole, single sided, Kapton polyimide, multi band]

  - Design, Fabrication, and Testing of Flexible Antennas [monopole, single side, Kapton]

  - Ultra-thin flexible fractal antenna for 2.45 GHz application with wideband harmonic rejection - ScienceDirect [monopole, single side, fractal]