With 5G new radio, a new high data rate communication link is introduced at f_c=28GHz compared to the 4G LTE. Suppose that we want to analyze its performance through a single carrier waveform. Let eh corrsepodning complex baseband signal be the expression given by:

xt=m=-∞∞dmgtxt-mTspacingϵC

Where T_spacing is the symbol spacing, g_tx(t) is the pulse shape, and dmϵC is a complex-valued data symbol.

- The receiver noise figure is 10 dB

- Tranmsit power is 1 watt

- TX antenna gain and RX antenna gain are G_tx=G_rx=10 dBi

- The pulse shape g_tx(t) is a root-raised cosine filter with a roll-off factor of 0.2

- The symbol rate is 400 Msymbols/sec

- The power delay profile for this communication channel is given as follows

  - Path 1: Delay: 0ns, relative power: 0 dB

  - Path 2: Delay: 100ns, relative power: -10dB

Questions:

- What is the transmit power in dBm?

Ptx=10log101000 mW=30 dBm

- Assume that path loss is based on the “simplified path loss model” where the measured path loss is 80 db at 20 meters and the path loss exponent is 3. What is the maximum link distance if the receiver sensitivity is -70 dBm?

Pr=Pt+Gt+Gr-L0+10n*log10dd0

-70=30+10+10-80+30*log10d20

-120=-80-30*log10d20

4030=log10d20

d=20*1043≈430.887

- What should T_spacing be to achieve the desired symbol rate?

Tspacing=1rate=1400e6=2.5e9 or 2.5 ns

- What is the signal bandwidth (around the carrier frequency) (you can calculate null-to-null bandwidth with the consideration of expansion due to the non-zero roll-off factor)?

BW=rate*1+α=400e6*1+0.2=480MHz

- What is the total noise power within the signal bandwidth in dBm? (kT[dBm/Hz] =-174)(if you couldn’t find the bandwidth in previous problem, assume null-to-null signal bandwidth is 500MHz) (Hint don’t forget to consider noise figure)

PnoisedBm=kT+10log10B+NF

PnoisedBm=-174+10log10480MHz+10≈-77.18759dBm

- What is the signal to noise ratio at the maximum link distance in dB (hint, less than 10 dB)

SNRdB=Pr-N0=-70--77.18759=7.18759dB

- Since SNR(computed in prior problem) is equal to E_s/N_0. What are the bit-error rates for “on-off” keying and “binary antipodal signaling” (if you could not find SNR in previous problems, assume it is 7 dB)

BERBPSK=Q2N0 =1.455%

BEROOK=Q1N0=6.142%

N0=0.19109144186567883

- Why is the bit error rate larger with OOK than the binary antipodal signaling? Please consider the corresponding normalized constellations for the explanation, not the formula.

In OOK, the signal constellation consists of two points along a single dimension (usually the amplitude). One point represents a signal presence (usually the '1' bit), and the other point represents no signal (the '0' bit). This means that the '0' bit is represented by the absence of energy, and noise can easily affect the detection of the '1' bit since the decision boundary between '0' and '1' is closer to the '1' bit than in antipodal signaling.

binary antipodal signaling places the two symbols (bits) at equal distances from the origin but on opposite sides along the same dimension (usually the phase or the amplitude). This configuration creates a larger "gap" between the two possible signal states, making it easier to distinguish between them even in the presence of noise.

- What are the minimum sampling rates at the I and Q branches if a 20 MHz guard band is provided between the aliases?

Total BW=BW+2*GB=480+2*20=520MHz

- What is the maximum excess delay of the channel?

Maximum Delay=100-0=100ns

- What should be the signal bandwidth to observe a frequency flat fading channel (assume that the minimum correlation of the channel frequency response is 0.5.)

Taverage=0×100/10+100×10-10/10100/10+10-10/10

Taverage=9.09ns

Trms=τi-Taverage2×10Adbi/1010Adbi/10

Trms=28.75 ns

Bc=15*Trms=6.96MHz

- Assume that the coherence bandwidth is 5 MHz, and we want to ensure that the signal does not experience frequency-selective fading. What should be the new symbol spacing and the signal bandwidth?

Assuming Rs≤5 MHz then Tspacing=1Rs≥15e6=0.2μS

BW=5 MHz