- Assume that the path loss measured at d0=20 m is 80 dB

  - Shadowing variance is 5 dB

  - Path loss exponent is 4

Express the simplified path loss model including shadowing

Pld=K+10log10dd0γ+ψ [dB]

ψ~N(0,σ2=5)

=80+40log10d-40log1020 +ψ [dB]

- Pl​d is the path loss at a distance d.

- K is the path loss measured at the reference distance d0​, which in this case is 80 dB.

- γ is the path loss exponent, which is 4 in this scenario.

- d0​ is the reference distance, set to 20 m.

- ψ represents the shadowing effect, modeled as a random variable with a normal (Gaussian) distribution N(0,σ2) where σ2=5 dB (variance).

Assume that the transmit power is 1 Watts and the receiver sensitivity is -80 dBm. What is the outage probability at d=100 meters?

EPrx=Ptx-Pld=10log1000-80+40log10d-40log1020

poutaged=PrPrxd<Pmin=1-QPmin-EPrxσ=0.1807 σ2=5

Propose three methods that can improve the coverage rate?

Increase the receiver and the transmitter heights (i.e., lower the exponent)

Decrease the carrier frequency

Decrease the receiver sensitivity with a better modulation

- Consider a communication link between two drones at a high altitude. The complex baseband signal is given by

xt=m=-∞∞dmgtx(t-mTspacing)∈C

  - Tspacing is the symbol spacing, gtx(t) is a RRC pulse, and dm∈C is a complex signal

  - The maximum distance between drones: 20 km, the receiver noise figure: 5 dB

  - The transmit power: 0.5 Watts, the antenna gains at the TX and RX: Gtx=Grx=5 dBi

What is the transmit power in dBm?

Ptx=10log10500 mW=27 dBm

What is the received signal power in dBm based on the free-space path loss model?

PrxdBm=PtxdBm-PLdB=30[dBm]-PL[dB]

PL=20log10d+20log10fc-147.55-Gtx-Grx=116[dB]

PrxdBm=27dBm-116dB=-89 dBm

What is the noise power in dBm? (kT = -174 dBm/Hz)

B=20 MHz⇒PnoisedBm=kT+10log10B+NF=-96 dBm

What is the SNR in dB?

SNRdB=PrxdB-PnoisedBm=-89 dBm+96 dBm=7 dB

What is the channel capacity?

R≤C=Blog21+SNRlin=20e6log21+10710=51.2 Mbit/s

If the pulse shape gtxt is root-raised cosine with the roll-off factor of 0.2, what should be Tspacing?

B=1+ρTspacing⇒Tspacing=1+ρB=1+0.220 MHz=60 ns

If the pulse shape gtxt is root-raised cosine with the roll-off factor of 0.2, what is the symbol rate?

Rsymbol=1Tspacing=160 ns=16.67e6 symbols/sec

If we want to reach 50 Mbps for the bit rate, how many bits need to be carried per data symbol?

# of bitssymbol≥RbitsRsymbol=50e61.67e6=3 bit/sym

Calculate the energy received per data symbol and the noise density

Prx [lin] =EsTspacing⇒EsdBm=PrxdBm+10log10Tspacing=-161 dBmJoule

Pnoise [lin] =N0Tspacing⇒N0dBm=PnoisedBm+10log10Tspacing=-168dBmJoule

What are the bit error rate & data rate if we use on-off keying? Give an example bit mapping for OOK

EsN0dB=-161--168=7 dB 	Bit 0⇒0, Bit 1⇒2

Perr=QEsN0=Q107/10=0.013	Rbit=1Tspacing=16.67 Mbit/s

Would the shadowing variance be large or small for this communication channel? Why?

Most likely low because there are less obstacles between the drones

Is it better to model the small-scale fading with Rayleigh or Rician? Why?

Rician because it is likely that there is line-of-sight path

Assume that drones are approaching each other. The speed of each drone is 200 meters/sec. What is the carrier frequency that the receiver observes?

If the total velocity is 2v=400 m/s, the Doppler shift is fd=400×fcc=3.2 kHz

frx=2.4 GHz+fd

Assume that the drones flying in an area where the PDP is characterized as follow:

What is maximum excess delay? What is the coherence bandwidth?

| Path | Delay | Relative Power |
| --- | --- | --- |
| 1 | 0 ns | 0 dB |
| 2 | 200 ns | -5 dB |

Tmax=200 ns

Taverage=∑τi10Adb10∑10Adb10=0×10010+200ns×10-51010010+10-510=48 ns

Trms=∑τi-Taverage210Adb10∑10Adb10=0-Taverage2×10010+0-Taverage2×10-51010010+10-510+…=85.4 ns

Bc≈15Trms=2.34 MHz for r=0.5⇒ Channel is frequency-selective

- Assume that Wi-Fi router transmit a signal where its bandwidth is 18 MHz signal around carrier for 100 μs duration

  - What are the minimum sample rates at I and Q branches with 2 MHz guard band

ut=At×cos2πfct+θt

=mI(t)×cos2πfct-mQ(t)sin2πfct

fs>2×B2+2 MHz=20 MHz